3GPP TSG-RAN WG2 Meeting #129	R2-2501023

Athens, Greece, Feb. 17th – 21st, 2025

# Agenda item:	8.6.2

# Source:	CMCC

# Title:	Discussion on conditional LTM

# Document for:	Discussion, Decision

- Introduction

In RAN2#128 meeting, RAN2 reached the following agreements on conditional LTM [1]:

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this paper, we further discuss the open issues of intra-CU conditional LTM.

- Discussion

  - Scenario

For  Rel-18 intra-CU LTM, the following scenarios are supported in [2]:

LTM supports both intra-gNB-DU and intra-gNB-CU inter-gNB-DU mobility. LTM supports both intra-frequency and inter-frequency mobility, including mobility to inter-frequency cell that is not a current serving cell. LTM is supported only for licensed spectrum. The following scenarios are supported: - PCell change in non-CA scenario and non-DC scenario; - PCell and SCell(s) change in CA scenario; - Dual connectivity scenario, PCell and MCG SCell(s) change and intra-SN PSCell and SCG SCell(s) change without MN involvement. LTM for simultaneous PCell and PSCell change is not supported.

Observation 1: intra-CU LTM can be used in DC and non-DC scenario.

The current discussion on conditional LTM mainly focuses on the SA scenario, in which conditional LTM is used to modify UE’s PCell.  And it’s not clear whether it can be supported in SCell or PScell change, in CA/DC cases. In our opinion, since RAN2 has made progress in supporting early DL/UL synchronization, while the main difference between legacy intra-CU LTM and conditional LTM is the entity for LTM decision. It seems that there’re no technique issues to support the above scenarios in CLTM. However, based on the revised WID [3], whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases will be check at RAN#107.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

Therefore, at least conditional LTM can be supported in non-CA and CA cases for PCell change, while for DC cases, it is decided by RAN plenary.

Proposal 1: Conditional LTM can be supported in non-CA and CA cases for PCell change, while for DC cases, it is decided by RAN plenary.

  - Preparation of CLTM

Based on the understanding of conditional mobility, the configuration of CLTM should at least include the candidate cell configuration and the related execution condition. The first issue needs to be clarified is whether CLTM candidate set is the same with legacy LTM candidate set or not. In our view, there’s no need to restrict the configuration of candidate set of the two types LTM to be the same. Though if both sets are the same, it can reduce signalling overhead of Uu and Xn, it brings the limitation to the flexibility of network configuration. Therefore, we think it can be network implementation whether the candidate set of CLTM is the same with legacy LTM candidate set. Besides, this issue also has relationship with the IE design for CLTM, if similar design to CHO, CLTM candidate set can be totally different from legacy LTM candidate set, while if legacy LTM-config IE is reused, CLTM candidate set is a subset of legacy LTM candidate set.

Proposal 2: It can be network implementation whether the candidate set of CLTM is the same with legacy LTM candidate set.

Additionally, another related issue to CLTM candidate is whether to enlarge the maximum number of LTM candidate cells. There’re similar discussion when CPAC was introduced in Rel-17 and when CHO with candidate SCG(s) was introduced in Rel-8, while the consequence is to kept the maximum number of  conditional  cells as eight. Therefore, the maximum number of candidate LTM cells can be kept as eight.

Proposal 3: There’s no need to introduce bigger number of LTM candidate cells. The maximum number of candidate LTM cells can be kept as eight.

The next issue should be discussed for CLTM preparation is about the execution condition configuration. In the first discussion of CLTM, RAN2 agreed to use Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions [4]. And in last meeting, RAN2 agreed that the L1 execution condition of a candidate cell is associated to only one triggering event. Besides, RAN2 also made progress on CHO condition reusing, that the triggering condition of conditional LTM can be based on L3 measurement. CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

Currently, it is unclear if it is possible to configure both L1 and L3 events for a specific candidate cell simultaneously. The motivation to introduce L3 measurement based LTM is due to the decouple of UE capability of supporting LTM and supporting L1 measurement, in other words, L3 measurement based LTM is focus on the UE without L1 measurement capability, for this type of  UE, only L3 based events, i.e., CHO events can be configured. As the UE with L1 measurement capability, if both L1 and L3 events are configured to a candidate cell, since L3 events are evaluated by RRC, while L1 events are evaluated by MAC, it may cause collision on conditional LTM decision or enlarged CLTM evaluation time. Therefore, we think it’s unnecessary to configure both types of events simultaneously to a candidate cell.

Proposal 4: For a specific candidate cell, if the UE not supporting L1 measurement, only CHO events is configured as execution condition; else, only L1 events (Event LTM3-like or Event LTM5-like) is configured as execution condition.

If Proposal 4 is adopted, for the UE with L1 measurement capability, only L1 based Event can be configured as an execution condition for a specific cell. Besides, in last RAN2 meeting, it was agreed to configure only one triggering event for L1 execution condition. And it seems that most companies prefer to follow the discussion in event triggered reporting agreements for the evaluation of Event LTM3-like/ Event LTM5-like, in other words, only one beam of candidate cell is evaluated.

Considering that CLTM decision is made by the UE itself, but not the network, if only one beam is evaluated, it  may cause frequent mobility among CLTM candidate cells. Therefore, cell level measurement should be considered in CLTM with only L1 event configured. To achieve this, multiple beams should be considered for CLTM execution, and the number of evaluated beams to trigger CLMT can be configured by the network. In this way, the network can achieve the balance of mobility robustness and mobility latency with different value of the number to trigger CLTM. The UE will not initiate CLM procedure to a specific candidate cell, unless the number of beams fulfilling  L1 event measurement threshold is not less than the number of beams to trigger CLTM.

Observation 2: If only one beam is evaluated for CLTM execution, the robustness of mobility cannot be assured. It may cause frequent ping-pong among candidate cells.

Proposal 5: For conditional LTM execution, more than one beam should be considered for CLTM execution.

Proposal 6: The network can configure the number of evaluated beams for CLTM execution, and the maximum number can be further discussed.

Proposal 7: The UE will not initiates CLTM procedure to a candidate cell, unless the number of beams fulfilling the L1 event measurement threshold is not less than the number of beams to trigger CLTM execution.

  - Execution condition evaluation and execution

For legacy CHO, the UE starts to evaluate CHO execution condition once it receives the related CHO configuration. However, in Rel-18, for CHO for NES, the UE evaluate the additional NES-specific CHO events only when it is notified via DCI to enable CHO conditions(s) configured with NES event indication [2]. When it comes to CLTM, though it may increase the LTM gain (e.g., low mobility latency) and UE power saving gain, if UE has complete early synchronization procedure when execution condition is satisfied, it should be noticed that both RACH-based and RACH-less CLTM is supported. Therefore, it’s not necessary to limit the sequence of early synchronization and execution condition evaluation. In other words, the UE can start execution condition evaluation once it receives CLTM configuration, but not after the completion of early synchronization.

Proposal 8：The UE starts execution condition evaluation once it receives CLTM configuration.

During the evaluation of execution conditions, there could be one case that multiple candidate cells fulfil the execution condition simultaneously, in this case, it’s UE implementation to select one candidate cell as its mobility target cell.

Proposal 9: It’s UE implementation to select which candidate cell as its mobility target cell if multiple cells fulfil the execution condition simultaneously.

As to RACH-based CLTM, RAN2 should clarify how to provide RACH configurations. In legacy LTM, it can be provided in LTM preparation phase by RRC signalling or indicated in LTM cell switch command, for CLTM, RACH resources can be preconfigured by RRC signalling, and there’s no need to be included in the new MAC CE for TA indication.

Proposal 10: RACH resources for RACH-based CLTM is preconfigured by RRC signalling in CLTM preparation phase.

Proposal 11: The new MAC CE for TA indication in CLTM is not used for RACH resources indication.

In last meeting, RAN2 agreed that candidate cell TA is maintained by a new timer. However, PDCCH order based early TA acquisition is triggered by the network based on UE’s measurement results, therefore, it cannot be assured that the TA is valid when the execution condition is satisfied. If the TA is invalid or in case there’s no TA, when the related execution condition is satisfied, if the UE has RACH resource, RACH-based CLTM is performed by the UE. If the TA timer expires before the related execution condition is satisfied, it is preferred that the UE indicates to the network for TA updating.

Proposal 12: If the TA is invalid or there’s no TA, when the related execution condition is satisfied, if the UE has RACH resource, RACH-based CLTM is performed by the UE.

Proposal 13: If the TA timer expires before the related execution condition is satisfied, the UE indicates to the network for TA updating.

What’s more, fast recovery mechanism is supported for intra-CU LTM, it can also be supported for intra-CU conditional LTM.

Proposal 14：Fast recovery mechanism is supported for  intra-CU CLTM.

  - Collision with other mobility mechanism(s)

Based on [5], if the gNB-DU is not able to accept the UE CONTEXT MODIFICATION REQUEST message for mobility because an LTM command has been triggered to the UE, it shall reply with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value. In other words, if there’s HO and LTM collision, DU may refuse CU’s HO command, in other words, LTM has higher priority than L3 HO.

Observation 3: If L3 HO and LTM almost triggered simultaneously, DU may refuse CU’s UE CONTEXT MODIFICATION REQUEST message for mobility, which means the priority of LTM is higher than L3 HO.

Considering the introduction of conditional LTM, two collision cases should be discussed:

- If L3 HO command/LTM cell switch command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied. In this case, UE follows L3 HO command or LTM cell switch command.

- If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time. In this case, we think LTM may have higher priority like priority of legacy LTM if higher than L3 HO, UE performs LTM.

Proposal 15: If L3 HO command/LTM cell switch command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied, UE follows L3 HO command or LTM cell switch command.

Proposal 16: If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time, LTM may have higher priority than CHO, and UE performs LTM.

- Conclusion

In this contribution, we discuss inter-CU LTM related issue. Following are the proposals and observations made in this contribution:

Observations:

Observation 1: intra-CU LTM can be used in DC and non-DC scenario.

Observation 2: If only one beam is evaluated for CLTM execution, the robustness of mobility cannot be assured. It may cause frequent ping-pong among candidate cells.

Observation 3: If L3 HO and LTM almost triggered simultaneously, DU may refuse CU’s UE CONTEXT MODIFICATION REQUEST message for mobility, which means the priority of LTM is higher than L3 HO.

Proposals:

Proposal 1: Conditional LTM can be supported in non-CA and CA cases for PCell change, while for DC cases, it is decided by RAN plenary.

Proposal 2: It can be network implementation whether the candidate set of CLTM is the same with legacy LTM candidate set.

Proposal 3: The maximum number of candidate LTM cells can be kept as eight.

Proposal 4: For a specific candidate cell, if the UE not supporting L1 measurement, only CHO events is configured as execution condition; else, only L1 events (Event LTM3-like or Event LTM5-like) is configured as execution condition.

Proposal 5: For conditional LTM execution, more than one beam should be considered for CLTM execution.

Proposal 6: The network can configure the number of beams for evaluation, and the maximum number can be further discussed.

Proposal 7: The UE will not initiates CLTM procedure to a candidate cell, unless the number of beams fulfilling the L1 event measurement threshold is not less than the number of beams to trigger CLTM execution.

Proposal 8：The UE starts execution condition evaluation once it receives CLTM configuration.

Proposal 9: It’s UE implementation to select which candidate cell as its mobility target cell if multiple cells fulfil the execution condition simultaneously.

Proposal 10: RACH resources for RACH-based CLTM is preconfigured by RRC signalling in CLTM preparation phase.

Proposal 11: The new MAC CE for TA indication in CLTM is not used for RACH resources indication.

Proposal 12: If the TA is invalid or there’s no TA, when the related execution condition is satisfied, if the UE has RACH resource, RACH-based CLTM is performed by the UE.

Proposal 13: If the TA timer expires before the related execution condition is satisfied, the UE indicates to the network for TA updating.

Proposal 14：Fast recovery mechanism is supported for  intra-CU CLTM.

Proposal 15: If L3 HO command/LTM cell switch command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied, UE follows L3 HO command or LTM cell switch command.

Proposal 16: If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time, LTM may have higher priority than CHO, and UE performs LTM.

- References

- Report of 3GPP TSG RAN WG2 meeting #128;

- TS 38.300; NR and NG-RAN Overall Description; Stage 2 (Release 18);

- RP-242356	Revised Work Item: NR mobility enhancements Phase 4; Apple Inc, China Telecom;

- Report of 3GPP TSG RAN WG2 meeting #127bis;

- TS 38.473 F1 application protocol (F1AP) (Release 18).