3GPP TSG RAN WG2 Meeting #129	R2-2501028

Athens, Greece, Feb 17h – 21st, 2025

Agenda Item:	8.2.2

Source:	CMCC

Title:	Discussion on A-IoT paging

Document for:	Discussion

Introduction

In RAN#106 meeting, a new WID are made for solutions for Ambient IoT in NR. The general scope of R19 A-IoT is shown as [1]:

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only.

Apart from this, the following RAN2-led objectives have been identified within the general scope:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

Then in this contribution, we would like to provide our views on A-IoT paging with Topology 1.

Discussion

## 2.1 Message content

### 2.1.1 Identifier of target device(s)

In SI, there are three cases studied for A-IoT paging, that is containing an identifier of a single A-IoT device, containing a group ID that maps to multiple A-IoT devices and containing no identifier which indicating all devices are targeted. Note that with the fact that the identifier of target device(s) is always indicated by and transmitted from CN, the format of the three types of device IDs, that is, full device ID, group ID and ID targeting to all devices, should be discussed and designed by SA2, not in RAN2. To this end, what RAN2 needs to discuss firstly is that whether the non-AS layer device ID is visible to A-IoT enabled gNB. From our point of view, since temporary identifier and AS-layer encryption/decryption are not supported, upper-layer encryption/decryption is necessary which result in upper-layer device/group ID invisible to A-IoT enabled gNB. Thus, the upper-layer device/group ID is just considered as the upper-layer data by A-IoT enabled gNB. To this end, another issues to be discussed is how to carry the invisible upper-layer data (non-AS layer device/group ID) in A-IoT paging message. Further taking the statement in WID that “RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes” into consideration, then legacy multiplexing scheme in NR can be referred, that is adding a field/header, in front of the upper-layer data (e.g. non-AS layer device/group ID). In terms of the design of this field/header, one indication representing the type of the upper-layer data (e.g. identifier of target device(s), upper-layer data, etc.) and one indication indicating the length of upper-layer data seem to be necessary. RAN2 can further discuss and determine other necessary information, if needed.

Observation 1: The format of the three types of device IDs, that is, full device ID, group ID and ID targeting to all devices, should be discussed and designed by SA2, not in RAN2.

Observation 2: Upper-layer encryption/decryption for device/group ID is necessary which results in upper-layer device ID invisible to A-IoT enabled gNB.

Proposal 1: To carry the upper-layer data (e.g. non-AS layer device/group ID) in A-IoT paging message, a field/header, which includes type and length of upper-layer data etc., should be added to the upper-layer data unit which indicating target device(s).

### 2.1.2 Identifier for avoiding duplicate response

According to TR 38.769 [2], the following agreements have been made in SI:

- It is supported that the reader can send multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN. The duplicated response from devices for the same service request should be avoided. The A-IoT paging message can include information to avoid this duplicated response from the device to a reader. It needs to be further discussed on how to design this information in A-IoT paging message (e.g., including stage-3 details and considering the related aspects from other WGs). Then, based on this information, the device determines whether to skip sending the response to A-IoT paging message or not (if the device had successfully responded the same service before). This information should be short and simple. This information is one ID, while it needs to be further discussed whether the ID is generated by the reader or by the core network. It needs to be further discussed on the size of this information.

- In RAN2, it has been agreed that one ID which should be short and simple is contained in A-IoT paging message to avoid the duplicate response. Note that the transaction ID which indicates the service request has been widely discussed in SA2. Since the length of transaction ID has not been defined yet, RAN2 is asked to determine the identifier for avoiding duplicated response based on the following cases:

- Case 1：the length of transaction ID is determined not to be longer than N bit(s)

- Note that N denotes the length which satisfies the restrictions of short and simple and should be determined by RAN2. For case 1, since transaction ID is short enough, it can be reused as the identifier for avoiding duplicated response directly. What’s more, another benefit of reusing transaction ID is that its duplication is easier to be avoided because it is allocated by CN.

- Case 2：the length of transaction ID is determined to be longer than N bit(s)

- For case 2, since transaction ID is not short enough, it can not be reused as the identifier to avoid duplicated response. To this end, the A-IoT enabled gNB may need to generate the identifier for avoiding duplicated response itself. It brings some benefits that the generated identifier can be shorter than transaction ID, but there may be some issues to be discussed to avoid its duplication. For example, to avoid different readers which are close to each other using same paging ID for different CN requests in overlapping time, some information exchanged between A-IoT enabled gNBs brings benefits for topology 1, but it may result in RAN3 impacts. While note that one more effective solution is to avoid the case that different readers which are close to each other performing different CN requests in overlapping time from happening, which depends on SA2.

Observation 3: The transaction ID which indicates the service request has been widely discussed in SA2, but the length of transaction ID has not been defined yet.

Proposal 2: RAN2 is asked to determine the identifier for avoiding duplicated response based on the following cases:

- If the length of transaction ID is determined not to be longer than N bit(s), transaction ID can be reused.

- If the length of transaction ID is determined to be longer than N bit(s), A-IoT enabled gNB can generate the identifier itself. FFS generation scheme

- where the value of N should be discussed and determined by RAN2.

### 2.1.3 Identifier of D2R transmission resources

During SI, for A-IoT device(s) determining the D2R resources and the following agreement of A-IoT paging are captured in TR 38.769:

- As to the A-IoT paging message, it can additionally indicate the information from which the device(s) can determine the resource(s) to be used for D2R response message(s). It can be further considered on more details for the discussion in clause 6.1.

Based on the study in RAN1, the D2R transmission resources may include at least time-domain (such as access occasion) and/or frequency-domain (such as the frequency-shift). From our point of view, the D2R transmission resources can be shared D2R resources or dedicated D2R resources, in which dedicated D2R resources represents the access resources assigned to specific Ambient IoT device, while shared D2R resources denote the common resources which should be shared and competed by all paged A-IoT devices.

Note that in order to allocate dedicated D2R resources, there are some limitations on the device IDs in A-IoT paging, e.g. one device ID or a list of device IDs should be contained. Therefore, the type of device IDs, that is, specific device ID, group ID or no ID, should be visible to A-IoT enabled gNB, so that A-IoT enabled gNB determines to allocate dedicated or shared D2R resources. Besides, for A-IoT devices determining dedicated D2R resources, the mapping ruled between each device ID and dedicated D2R resource, which is explicit or implicit, is supported.

In terms of allocating shared D2R resources, the number of paged devices is really helpful for A-IoT enabled gNB to allocate resources in A-IoT paging. For example, if ten devices are expected to be paged, allocating at least ten access occasions is suitable to avoid collisions as much as possible. Therefore the number of target A-IoT devices should be visible to gNB if available. In case that CN can not provide the number of target A-IoT devices (such as paging all devices in coverage of a reader), the A-IoT enabled gNB should estimates the device number.

For each A-IoT device determining its access occasion, which denotes frequency-domain and time-domain resource, based on the indicated shared D2R resources, it random select one between all available occasions which may be in TDM and/or FDM.  An example is given in Figure 1, where available access occasions which includes 2 frequency-domain resources and 3 time-domain resources are depicted. Then the target A-IoT device can random select one occasion among the 6 available access occasions. In other words, if shared D2R resources includes more than one frequency-domain resources, the behavior of A-IoT device randomly selecting one frequency-domain resource is same as its behavior in time-domain which has been discussed.

Figure 1. An illustration of available access occasions allocated with shared D2R resources

Observation 4: In order to allocate dedicated D2R resources, there are some limitations on the device IDs in A-IoT paging, e.g. one device ID or a list of device IDs should be contained.

Observation 5: In terms of allocating shared D2R resources, the number of paged devices is really helpful for A-IoT enabled gNB to allocate resources in A-IoT paging.

Observation 6: In case that CN can not provide the number of target A-IoT devices (such as paging all devices in coverage of a reader), estimation which can be performed by the A-IoT enabled gNB brings benefits.

Proposal 3: The dedicated or shared D2R resources can be indicated in an A-IoT paging.

Proposal 4: The type of device IDs, i.e., device ID, group ID or no ID, should be visible to A-IoT enabled gNB, so that A-IoT enabled gNB determines to allocate dedicated or shared D2R resources.

Proposal 5: For A-IoT devices determining its access occasion based on dedicated D2R resources, the mapping rule between each device ID and dedicated D2R resource, which is explicit or implicit, is supported.

Proposal 6: For each A-IoT device determining its access occasion based on shared D2R resources, it randomly selects one among all available occasions which may be in TDM and/or FDM.

### 2.1.4 Identifier of random access type

In WID, the following contents are in the RAN2’s scope of A-IoT random access:

- A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

CBRA and CFRA may be distinguished by whether dedicated D2R resource is allocated or only shared D2R transmission resource is indicated in an A-IoT paging. For CBRA, it is clear that there are following two options to be down-selected:

- Option 1: Only support 3-step CBRA

For Option 1, since only one type of random access is supported, no identifier of CBRA type is needed in A-IoT paging. That is, if only 3-step CBRA is supported, there is no any additional identifier which indicating the CBRA type is needed.

- Option 2: Support unified solution

For Option 2, it is controlled by the A-IoT enabled gNB on whether to also include the upper layer data in Msg1 resulting in 3-step or 2-step CBRA. Therefore, an identifier indicating the CBRA type is necessary for A-IoT device(s) determining the subsequent CBRA procedure. While for 2-step CBRA, it brings benefits only when the collision does not occur frequently. Considering that the Msg1 which contains both random number and device ID in 2-step CBRA is longer than that of 3-step CBRA, the reader have to wait for longer time when it detects collision during the reception of Msg1. Therefore, if collision occurs frequently in 2-step CBRA, more time will be wasted.

With the consideration that Option 2 may bring additional complexity and overhead to A-IoT paging message and 2-step CBRA does not bring obvious benefits, Option 1 is preferred.

Observation 7: CBRA and CFRA may be distinguished by whether dedicated D2R transmission resource is allocated or only shared D2R transmission resource is indicated in an A-IoT paging.

Observation 8: If only 3-step CBRA is supported, no identifier of CBRA type is needed in A-IoT paging.

Observation 9: If unified solution is supported, an identifier indicating whether to include the upper layer data in Msg1 or not should be contained in A-IoT paging.

Proposal 7: To decrease the additional complexity and overhead, only 3-step CBRA is supported, thus, no identifier of CBRA type is needed in A-IoT paging.

## 2.2 Device unavailability due to energy harvesting

In WID, the following item is within the scope of device availability as:

- H.Device (un)availability via Direction 1 in TR 38.769 only.

According to TR 38.769, Direction 1 represents that:

Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable.

In terms of Direction 1, we do not see any RAN2 impact since A-IoT enabled gNB does not need to provide any information for determining device availability/unavailability.

Observation 10: For Direction 1 of device (un)availability, there seems to be no RAN2 impact.

Proposal 8: There is no RAN2 impact in terms of device (un)availability, since for Direction 1, reader does not need to provide any information to a device.

## 2.3 Additional message for helping device(s) determining transmission/access occasion

It is clear that the determination the corresponding D2R transmission/access occasion after A-IoT paging is closely related to the timing accuracy of A-IoT devices. In R19 WI, since there is only device 1 in the general scope and the initial clock accuracy of device 1 is about 10^4 ~ 10^5 ppm which is the evaluation assumption in RAN1 as captured in TR 38.769, it should be really difficult for device 1 determining the transmission/access occasion efficiently. To this end, a R2D message, similar to QueryRep in RFID, which indicates the occasion boundary, is really essential especially when a large number of transmission/access occasions indicated in an A-IoT paging. For the case that A-IoT paging provides all subsequent scheduling information without any R2D message indicating the occasion boundary, it may cause higher signaling overhead and its performance is affected by the timing accuracy of devices seriously. In summary, RAN2 is kindly asked to support a R2D message which indicates the boundary of transmission/access occasions unless RAN1 excludes this.

Observation 11: Considering the limited timing accuracy for device type 1, a R2D message, similar to QueryRep in RFID, which indicates the occasion boundary, is essential for device to access especially when a large number of transmission/access occasions indicated by an A-IoT paging.

Observation 12: For the case that A-IoT paging provides all subsequent scheduling information without any R2D message indicating the occasion boundary, it may cause higher signaling overhead and its performance is affected by the timing accuracy of devices seriously.

Proposal 9: RAN2 is kindly asked to support a R2D message which indicates the boundary of transmission/access occasions unless RAN1 excludes this.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on paging for Ambient IoT:

Observation 1: The format of the three types of device IDs, that is, full device ID, group ID and ID targeting to all devices, should be discussed and designed by SA2, not in RAN2.

Observation 2: Upper-layer encryption/decryption for device/group ID is necessary which results in upper-layer device ID invisible to A-IoT enabled gNB.

Observation 3: The transaction ID which indicates the service request has been widely discussed in SA2, but the length of transaction ID has not been defined yet.

Observation 4: In order to allocate dedicated D2R resources, there are some limitations on the device IDs in A-IoT paging, e.g. one device ID or a list of device IDs should be contained.

Observation 5: In terms of allocating shared D2R resources, the number of paged devices is really helpful for A-IoT enabled gNB to allocate resources in A-IoT paging.

Observation 6: In case that CN can not provide the number of target A-IoT devices (such as paging all devices in coverage of a reader), estimation which can be performed by the A-IoT enabled gNB brings benefits.

Observation 7: CBRA and CFRA may be distinguished by whether dedicated D2R transmission resource is allocated or only shared D2R transmission resource is indicated in an A-IoT paging.

Observation 8: If only 3-step CBRA is supported, no identifier of CBRA type is needed in A-IoT paging.

Observation 9: If unified solution is supported, an identifier indicating whether to include the upper layer data in Msg1 or not should be contained in A-IoT paging.

Observation 10: For Direction 1 of device (un)availability, there seems to be no RAN2 impact.

Observation 11: Considering the limited timing accuracy for device type 1, a R2D message, similar to QueryRep in RFID, which indicates the occasion boundary, is essential for device to access especially when a large number of transmission/access occasions indicated by an A-IoT paging.

Observation 12: For the case that A-IoT paging provides all subsequent scheduling information without any R2D message indicating the occasion boundary, it may cause higher signaling overhead and its performance is affected by the timing accuracy of devices seriously.

Proposal 1: To carry the upper-layer data (e.g. non-AS layer device/group ID) in A-IoT paging message, a field/header, which includes type and length of upper-layer data etc., should be added to the upper-layer data unit which indicating target device(s).

Proposal 2: RAN2 is asked to determine the identifier for avoiding duplicated response based on the following cases:

- If the length of transaction ID is determined not to be longer than N bit(s), transaction ID can be reused.

- If the length of transaction ID is determined to be longer than N bit(s), A-IoT enabled gNB can generate the identifier itself. FFS generation scheme

- where the value of N should be discussed and determined by RAN2.

Proposal 3: The dedicated or shared D2R resources can be indicated in an A-IoT paging.

Proposal 4: The type of device IDs, i.e., device ID, group ID or no ID, should be visible to A-IoT enabled gNB, so that A-IoT enabled gNB determines to allocate dedicated or shared D2R resources.

Proposal 5: For A-IoT devices determining its access occasion based on dedicated D2R resources, the mapping rule between each device ID and dedicated D2R resource, which is explicit or implicit, is supported.

Proposal 6: For each A-IoT device determining its access occasion based on shared D2R resources, it randomly selects one among all available occasions which may be in TDM and/or FDM.

Proposal 7: To decrease the additional complexity and overhead, only 3-step CBRA is supported, thus, no identifier of CBRA type is needed in A-IoT paging.

Proposal 8: There is no RAN2 impact in terms of device (un)availability, since for Direction 1, reader does not need to provide any information to a device.

Proposal 9: RAN2 is kindly asked to support a R2D message which indicates the boundary of transmission/access occasions unless RAN1 excludes this.

Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN2#106, Madrid, Spain, RAN1 Vice-chair (Huawei).

- 3GPP TR 38.769 V2.0.0, “Study on solutions for ambient IoT (Internet of Things) (Release 19)”.