3GPP TSG RAN WG2 Meeting #129	R2-2501029

Athens, Greece, Feb 17th – 21st, 2025

Agenda Item:	8.11.2

Source:	CMCC

Title:	Discussion on random access in SBFD

Document for:	Discussion

Introduction

In RAN2#128 meeting, the following agreements have been made for random access in SBFD, shown as:

CFRA The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed). CBRA FFS on the following options Option 1 - Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one. RACH configuration Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

Then in this contribution, we would like to provide our further views on RAN2 impacts to support random access in SBFD.

Discussion

## 2.1 General

In RAN2#127 meeting, the following working assumptions are made for RA events:

- 1. Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Some companies think that RACH resources for Position SI request and SI request may not need to be extended, since it is common for all UEs. While, supporting Position SI request and SI request in SBFD symbol is technically feasible and has the potential to support SBFD-aware UE in obtaining latest Position SI and SI faster. To this end, aim to reduce the random access latency and the PRACH collision probability, we do not find any strong reason to drop any RA event, even for Position SI request and SI request. Therefore, RAN2 confirms the working assumption “Random access procedure in SBFD symbols is supported for all the existing RACH trigger events”.

Proposal 1: RAN2 confirms the working assumption “Random access procedure in SBFD symbols is supported for all the existing RACH trigger events”.

For legacy NR, there are different CBRA and CFRA events and corresponding PRACH resource indication/configuration signallings as summarized in Table 1.

Table 1. RA events and corresponding PRACH resource indication/configuration signallings

| RA event | CBRA PRACH resource indication/configuration signalling | CFRA PRACH resource indication/configuration signalling |
| --- | --- | --- |
| Initial access from RRC_IDLE | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| RRC Connection Re-establishment procedure | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| DL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronization status is "non-synchronize" | - CBRA resource configured by RACH-ConfigCommon - PDCCH order trigger | - CFRA resource configured by RACH-ConfigCommon - PDCCH order trigger |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronization status is "non-synchronized" | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| SR failure | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| Request by RRC upon synchronous reconfiguration (e.g. handover) | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource and preamble configured by RACH-ConfigDedicated |
| RRC Connection Resume procedure from RRC_INACTIVE | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| To establish time alignment for a secondary TAG | - N/A | - CFRA resource configured by RACH-ConfigCommon - PDCCH order indicating the used PRACH resource and preamble |
| Request for Other SI | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource and preamble configured by SI-RequestConfig or SI-RequestConfigRepetition |
| Beam failure recovery | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource, preamble and root sequence configured by BeamFailureRecoveryConfig |
| SDT in RRC_INACTIVE | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource configured by RACH-ConfigCommon - PDCCH order trigger |
| Early UL synchronization with an LTM candidate cell | - N/A | - CFRA resource configured by EarlyUL-SyncConfig - PDCCH order trigger |
| RACH-based LTM cell switch | - CBRA resource configured by RACH-ConfigCommon - LTM Cell Switch Command MAC CE trigger | - CFRA resource configured by RACH-ConfigDedicated or RACH-ConfigCommon - LTM Cell Switch Command MAC CE trigger |

From Table 1, it can be observed that there are multiple events which can trigger random access procedures  and different RA procedures (CBRA or CFRA) may be triggered by different RA events. Besides, it can also be found that CBRA PRACH resource is configured by RACH-ConfigCommon and CFRA PRACH resource can be configured by different resource configuration signallings such as SI-RequestConfig, BeamFailureRecoveryConfig etc., for different RA events.

Observation 1: CBRA PRACH resource is configured by RACH-ConfigCommon.

Observation 2: CFRA PRACH resource can be configured by different resource configuration signallings such as SI-RequestConfig, BeamFailureRecoveryConfig etc., for different RA events.

## 2.2 	Configuration of RACH resource

In RAN1#118 meeting, it has been agreed that [1]:

Agreement For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following: - For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration. Agreement For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs).

It is clear that both RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-3 have been agreed to support in RAN1 without distinguish CBRA or CFRA configuration. Since all existing RA events are supported by RAN2, one important issue to be discussed is whether both RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-3 is supported for each PRACH resource configuration signalling.

Proposal 2: RAN2 is asked to discuss whether both RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-3 are supported for each RA event.

Since whether to support 2-step RACH is still being discussed in RAN1, RAN 2 can study the RACH configurations for 4-step CBRA and 4-step CFRA firstly at this stage. For a comprehensive view, the detailed analysis of the 4-step CBAR and 4-step CFRA PRACH configuration based on Option 1 with Alt 1-1 or Option 2 with Alt 2-3 is conducted as following.

### 2.2.1 Configuration for 4-step CBRA

- Option 1 with Alt 1-1

- In RAN2#127bis meeting, it has been agreed that [2]:

- From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

- Since SBFD is not considered as a new feature, 4-step CBRA configuration shares the existing parameters configured in RACH-ConfigCommon, while note that RAN1 support separate power control parameters or PRACH transmission in SBFD symbols and non-SBFD symbols. With the consideration that details of the separate power control parameters are still being discussed by RAN1, RAN2 can wait for RAN1’s progress to determine the additional power control parameters for PRACH transmission in SBFD symbols.

- Option 2 with Alt 2-3

- For CBRA configuration with Option 2 with Alt 2-3, two separate RACH configurations are needed. Therefore, a new IE called rach-ConfigCommonSBFD-r19 can be introduced to indicate the additional 4-step CBRA configuration. An example is given as:

BWP-UplinkCommon information element

-- ASN1START

-- TAG-BWP-UPLINKCOMMON-START

BWP-UplinkCommon ::=                SEQUENCE {

genericParameters                   BWP,

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

pusch-ConfigCommon                  SetupRelease { PUSCH-ConfigCommon }                                     OPTIONAL,   -- Need M

pucch-ConfigCommon                  SetupRelease { PUCCH-ConfigCommon }                                     OPTIONAL,   -- Need M

...,

[[

rach-ConfigCommonIAB-r16            SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

useInterlacePUCCH-PUSCH-r16         ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

msgA-ConfigCommon-r16               SetupRelease { MsgA-ConfigCommon-r16 }                                  OPTIONAL    -- Cond SpCellOnly2

]],

[[

enableRA-PrioritizationForSlicing-r17 BOOLEAN                                                    OPTIONAL, -- Cond RA-PrioSliceAI

additionalRACH-ConfigList-r17       SetupRelease { AdditionalRACH-ConfigList-r17 }               OPTIONAL, -- Cond SpCellOnly2

rsrp-ThresholdMsg3-r17              RSRP-Range                                                   OPTIONAL, -- Need R

numberOfMsg3-RepetitionsList-r17    SEQUENCE (SIZE (4)) OF NumberOfMsg3-Repetitions-r17                  OPTIONAL,  -- Cond Msg3Rep

mcs-Msg3-Repetitions-r17            SEQUENCE (SIZE (8)) OF INTEGER (0..31)                               OPTIONAL   -- Cond Msg3Rep

]],

[[

additionalRACH-perPCI-ToAddModList-r18   SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF  RACH-ConfigTwoTA-r18

OPTIONAL, -- Cond 2TA-Only

additionalRACH-perPCI-ToReleaseList-r18  SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF AdditionalPCIIndex-r17

OPTIONAL,  -- Need N

rsrp-ThresholdMsg1-RepetitionNum2-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

rsrp-ThresholdMsg1-RepetitionNum4-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

rsrp-ThresholdMsg1-RepetitionNum8-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

preambleTransMax-Msg1-Repetition-r18     ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}      OPTIONAL   -- Cond Msg1Rep1

]]

[[

rach-ConfigCommonSBFD-r19            SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

]]

}

AdditionalRACH-ConfigList-r17 ::=       SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17

AdditionalRACH-Config-r17 ::=       SEQUENCE {

rach-ConfigCommon-r17               RACH-ConfigCommon                                                   OPTIONAL,  -- Need R

msgA-ConfigCommon-r17               MsgA-ConfigCommon-r16                                               OPTIONAL,  -- Need R

...

}

NumberOfMsg3-Repetitions-r17::=         ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16}

-- TAG-BWP-UPLINKCOMMON-STOP

-- ASN1STOP

- While, it still needs to be discussed that whether all parameters in RACH-ConfigCommon IE is needed for rach-ConfigCommonSBFD-r19.

Proposal 3: For 4-step CBRA configuration Option 1 with Alt 1-1, RAN2 can wait for RAN1’s progress to determine the additional power control parameters for PRACH transmission in SBFD symbols.

Proposal 4: For 4-step CBRA configuration Option 1 with Alt 2-3, RAN2 can introduce a new IE rach-ConfigCommonSBFD-r19 which parallels to rach-ConfigCommon. FFS details of parameters in ConfigCommonSBFD-r19 IE

### 2.2.2 Configuration for 4-step CFRA

- Option 1 with Alt 1-1

- Compared to the configuration for 4-step CBRA, which depends on whether SBFD is considered as a new feature, configuration for 4-step CFRA based on Option 1 with Alt 1-1 is much easier. To be specific, 4-step CFRA shares the same existing parameters in resource configuration signallings such as SI-RequestConfig, BeamFailureRecoveryConfig etc. With consideration that there are multiple different resource configuration signallings for the 4-step CFRA configuration triggered by different RA events, RAN2 is asked to study whether/which parameter(s) need to be reinterpreted by SBFD-aware UEs in each signalling.

- Option 2 with Alt 2-3

- For 4-step CFRA configuration based on Option 2 with Alt 2-3, additional CFRA configuration is needed. An easy solution is introducing a new IE parallel to each legacy IE. For example, a new IE named BeamFailureRecoveryConfigSBFD-r19 can be introduced to provide the additional 4-step CFRA configuration for SBFD-aware UEs to be used when beam failure occurs.

- For CFRA, one specific issue to be discussed is whether it is necessary to configure both addition ROs and legacy ROs for the one RA event or only to configure one type of ROs, that is, either additional ROs or legacy ROs, is enough.

Proposal 5: For 4-step CFRA configuration based on Option 1 with Alt 1-1, RAN2 is asked to study whether/which parameter(s) need to be reinterpreted by SBFD-aware UEs in each signalling.

Proposal 6: For 4-step CFRA configuration based on Option 2 with Alt 2-3, a new IE parallel to each legacy IE can be introduced. FFS the behavior of SBFD-aware UEs when some parameters in new IE is absent

Proposal 7: For CFRA, one specific issue to be discussed is whether it is necessary to configure both addition ROs and legacy ROs for one RA event or only to configure one type of ROs, that is, either additional ROs or legacy ROs, is enough.

## 2.3 Determination of RO type

SBFD-aware UEs need to determine the available RACH resources, that is whether legacy-ROs and/or additional-ROs can be used. Similar to the case analyzed in section 2.2, the behavior of SBFD-aware UE is also divided into two situations: CBRA or CFRA.

### 2.3.1 Determination of RO type for CBRA

The available RACH resources of SBFD-aware UE for CBRA depends on both its capability and the RACH configuration. To be specific, it has been agreed that UE is not required to support both options in RAN1#116bis meeting. Further considering that supporting Option 2 is more demanding for UE capability, that is, for SBFD-aware UE supports Option 2, it supports Option 1 naturally. Thus, from UE side, there may be two types of SBFD-aware UE based on their capabilities, that is:

- Cap 1: SBFD-aware UE only supports Option 1

- Cap 2: SBFD-aware UE supports both Option 1 and Option 2

Further in RAN2#128 meeting, the following agreement are made [3]:

RACH configuration - Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

Therefore, from network side, there may be three cases of RACH configuration as following:

- Case 1: Neither Option 1 nor Option 2 is configured

- Case 2: Option 1 is configured

- Case 3: Option 2 is configured

Note that based on the latest progress in RAN1, case 1 and case 2 mentioned above can already be indicated by network with an explicit indication(1-bit SIB1 indication and/or dedicated signalling to convey cell-specific configuration) [4]. Based on the capabilities of SBFD-aware UEs and RACH configuration in a cell, the available RACH resources of SBFD-aware UEs for CBRA are analyzed in Table 2.

Table 2. The available RACH resources of SBFD-aware UEs with different capabilities under different network configuration cases

| RACH configuration cases | SBFD-aware UE capability |  |
| --- | --- | --- |
|  | Cap 1 | Cap 2 |
| Case 1 | only legacy ROs available | only legacy ROs available |
| Case 2 | legacy ROs and additional ROs configured by RACH configuration option 1 available | legacy ROs and additional ROs configured by RACH configuration option 1 available |
| Case 3 | only legacy ROs available | legacy ROs and additional ROs configured by RACH configuration option 2 available |

From Table 2, it can be found that the available RACH resources of SBFD-aware UE for CBRA under different configuration cases is very clear.

- Observation 3: Since supporting Option 2 is more demanding for UE capability, for SBFD-aware UE supports Option 2, it supports Option 1 naturally.

Observation 4: From UE side, there may be two types of SBFD-aware UE based on their capabilities, that is:

- Cap 1: SBFD-aware UE only supports Option 1 with Alt 1-1

- Cap 2: SBFD-aware UE supports both Option 1 with Alt 1-1 and Option 2 with Alt 2-3.

- Observation 5: From network side, there may be also three types of configuration cases as following:

- Case 1: Neither Option 1 nor Option 2 is configured

- Case 2: Option 1 is configured

- Case 3: Option 2 is configured

- Observation 6: The available RACH resources of SBFD-aware UE for CBRA under different configuration cases is very clear.

Based on capabilities of SBFD aware UEs and RRACH configuration, if both legacy ROs and additional ROs are available, the initial PRACH transmission attempt and re-attempt should be discussed.

#### 2.3.1.1 Initial PRACH transmission attempt

In RAN1#119 meeting, the following agreement is made:

- Agreement - It is up to RAN2 to determine the detailed specified/configured conditions/prioritizations for RO selection for initial PRACH transmission attempt in one random access procedure.

In RAN2#128 meeting, the following two options are discussed and agreed:

FFS on the following options Option 1 - Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one.

It is clear that the remaining issue is that whether to prioritize the type indicated by the network or the configured priority and how the certain specified/configured conditions/prioritization is designed. From our point of view, Option 1 is preferred, because the network may have better estimates of the loads of different RO types and thus provide such indications. As for the detailed design of prioritization when there is no indication from network, DL RSRP is an important metric which needs to take into consideration. In legacy NR, the RSRP of the downlink pathloss reference is taken into consideration when selecting between 4-step RA and 2-step RA, if the BWP selected for Random Access procedure is configured with both 2-step and 4-step RA type Random Access Resources. For SBFD, how to define the initial PRACH resource selection rule based on DL RSRP depends on the motivation of enabling PRACH transmission in SBFD symbols. For example, if the motivation is to improve PRACH capacity and reduce PRACH collision probability, it would be better that cell centre SBFD-aware UEs use the additional ROs, which implies UE should select additional ROs if DL RSRP is higher than a threshold. If the motivation is to improve the PRACH coverage by using long PRACH format or using PRACH repetition, it is suitable that cell edge SBFD-aware UEs use the additional ROs, which implies UE should select additional-ROs if DL RSRP is lower than a threshold. Therefore, RAN2 is asked to discuss the the motivation of enabling PRACH transmission in SBFD symbols when determining how to design the specified prioritization rule based on DL RSRP. While note that within one cell, only one selection rule for initial RO type is configured/enabled to SBFD-aware UEs.

Observation 7: If there is no explicit indication from network, how to define the initial PRACH resource selection rule based on DL RSRP depends on the motivation of enabling PRACH transmission in SBFD symbols.

- Proposal 8: For the initial PRACH transmission attempt, Option 1, that is prioritizing the RO type indicated by the network, is preferred.

- Proposal 9: For CBRA, RAN2 is asked to take the motivation of enabling PRACH transmission in SBFD symbols into consideration when designing the prioritization rule based on DL RSRP, if no explicit indication of RO type from network.

- Proposal 10: For CBRA, only one selection rule for initial RO type is supported/enabled in one cell.

#### 2.3.1.2 PRACH transmission re-attempt

For PRACH transmission re-attempt, it has been agreed in RAN2#127 meeting that:

Agreement For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

- In RAN2, it has been agreed to support additional ROs fallback to legacy ROs, with the consideration that the interference level in SBFD symbols is often more serious than that in non-SBFD symbols. For the other direction, that is, legacy ROs fallback to additional ROs, it has been discussed in RAN2#128 meeting but no agreement has been reached. With the consideration that the factors which cause the failure of random access attempt includes high interference, collision between multiple UEs, etc., allowing SBFD-aware UEs which select legacy ROs as initial attempt fallback to additional ROs may offload PRACH transmission and reduce probability of PRACH collision more efficiently. Besides, since additional ROs may be configured in different format compared to legacy ROs for Option 2, allowing SBFD-aware UEs which select legacy ROs as initial attempt fallback to additional ROs may also bring benefits for some cases. For example, if additional ROs are configured in long format while legacy ROs are in short format, allowing SBFD-aware UEs which is far away from gNB to fallback to additional ROs can bring some benefits. Therefore, RAN2 is asked to support SBFD-aware UEs which select legacy ROs as initial attempt fallback to additional ROs. For the sake of simplicity, within one cell, only one direction, that is, either legacy ROs fallback to additional ROs or additional ROs fallback to additional ROs is supported, thus the fallback direction in one cell can be uniformly indicated through simple identification.

Observation 8: Allowing SBFD-aware UEs which select leagcy ROs as initial attempt fallback to additional ROs may offload PRACH transmission and reduce probability of PRACH collision more efficiently.

Observation 9: Since additional ROs may be configured in different format compared to legacy ROs for Option 2, allowing SBFD-aware UEs which select legacy ROs as initial attempt fallback to additional ROs may also bring benefits for some cases.

Proposal 11: RAN2 is asked to support SBFD-aware UEs which select leagcy ROs as initial attempt switch to additional ROs after certain (configured) number of times of RACH attempt.

Proposal 12: Within one cell, only one direction, that is, either legacy ROs fallback to additional ROs or additional ROs fallback to additional ROs is supported.

### 2.3.2 Determination of RO type for CFRA

In RAN1#119 meeting [4], the following agreements are made for CFRA:

Agreement For CFRA triggered by PDCCH order for SBFD aware-UEs, SBFD-aware UE is explicitly indicated to use additional-RO or legacy-RO via a 1-bit field in the DCI of PDCCH order

- And in RAN2#128 meeting, similar agreement has also been made as following:

CFRA The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed).

- Therefore it is clear that for CFRA triggered by PDCCH order, such as: DL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronization status is "non-synchronize", etc., 1-bit field indicating whether to use additional-RO or legacy-RO is contained in the DCI of PDCCH. For CFRA not triggered by PDCCH order, such as: RACH-based LTM cell switch, etc., whether to use additional-RO or legacy-RO should be indicated via signaling. The detailed signaling design depends on the specific RA event that triggers CFRA. For example, for RACH-based LTM cell switch, the RO type indication can be provided in LTM Cell Switch Command MAC CE. For Request by RRC upon synchronous reconfiguration, the RO type indication can be provided in RRC message. While note that if one SBFD-aware UE is only configured with one type of ROs for CFRA or only one type of ROs for CFRA is enabled, it is possible that the network does not indicate the RO type explicitly. To this end, RAN2 is asked to design the indication of RO-type corresponding to each RA event which can trigger CFRA. Based on above analysis, the available RACH resources of SBFD-aware UE for CFRA follows the indication from network. And if the SBFD-aware UE can not follow the indication from network, for example it don not support the PRACH configuration Option indicated by network, its behavior can be further discussed or just depends on UE implementation.

Proposal 13: RAN2 is asked to design the indication of RO type corresponding to each RA event which can trigger CFRA.

Proposal 14: For the available RO type for CFRA, SBFD-aware UE follows the indication from network, which is explicit or implicit.

## 2.4 Msg3-based early indication

Msg1-based early indication in legacy ROs has been agreed not to be supported in RAN2. For Msg3-based early indication, it has been widely discussed in RAN2#128 but no consensus was reached. If Msg3-based early indication is supported, dedicated LCID(s) can be allocated to SBFD-aware UEs. With that, gNB can identify a SBFD-aware UE based on the logical channel ID in Msg3 and thus schedule subsequent uplink transmission in SBFD symbol(s) for SBFD-aware UE which brings benefits. Considering using reserved LCID(s) in present NR, there is almost no influence on the SBFD-aware UE itself and other UEs. If Msg3-based early indication is not supported, gNB can only identify a SBFD-aware UE through UE capability transfer which is later than that through Msg3-based early indication.

- From our point of view, RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs, because there is no impact on PRACH capacity of non-SBFD-aware UEs and can bring some benefits in increasing the performance of SBFD operation.

Observation 10: With Msg3-based early indication, there is almost no impact on PRACH capacity of non SBFD-aware UEs.

Observation 11: If Msg3-based early indication is not supported, gNB can only identify a SBFD-aware UE through UE capability transfer which is later than that through Msg3-based early indication.

Proposal 15: RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs.

## 2.5 SBFD operation for 2-step RACH

In legacy, 2-step RACH can reduce the latency of random access procedure compared to 4-step RACH. However, since supporting SBFD operation for 4-step RACH can already reduce the latency of random access procedure, the benefit of supporting SBFD operation for 2-step RACH seems to be marginal. While note that whether to support SBFD opertaion for 2-step RACH is still being discussed by RAN1, thus RAN2 can just wait for RAN1 progress on whether to support SBFD operation for 2-step RACH.

Proposal 16: RAN2 is asked to wait for RAN1 progress on whether to support SBFD operation for 2-step RACH.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on random access in SBFD:

Observation 1: CBRA PRACH resource is configured by RACH-ConfigCommon.

Observation 2: CFRA PRACH resource can be configured by different resource configuration signallings such as SI-RequestConfig, BeamFailureRecoveryConfig etc., for different RA events.

- Observation 3: Since supporting Option 2 is more demanding for UE capability, for SBFD-aware UE supports Option 2, it supports Option 1 naturally.

Observation 4: From UE side, there may be two types of SBFD-aware UE based on their capabilities, that is:

- Cap 1: SBFD-aware UE only supports Option 1 with Alt 1-1

- Cap 2: SBFD-aware UE supports both Option 1 with Alt 1-1 and Option 2 with Alt 2-3.

- Observation 5: From network side, there may be also three types of configuration cases as following:

- Case 1: Neither Option 1 nor Option 2 is configured

- Case 2: Option 1 is configured

- Case 3: Option 2 is configured

- Observation 6: The available RACH resources of SBFD-aware UE for CBRA under different configuration cases is very clear.

Observation 7: If there is no explicit indication from network, how to define the initial PRACH resource selection rule based on DL RSRP depends on the motivation of enabling PRACH transmission in SBFD symbols.

Observation 8: Allowing SBFD-aware UEs which select leagcy ROs as initial attempt fallback to additional ROs may offload PRACH transmission and reduce probability of PRACH collision more efficiently.

Observation 9: Since additional ROs may be configured in different format compared to legacy ROs for Option 2, allowing SBFD-aware UEs which select legacy ROs as initial attempt fallback to additional ROs may also bring benefits for some cases.

Observation 10: With Msg3-based early indication, there is almost no impact on PRACH capacity of non SBFD-aware UEs.

Observation 11: If Msg3-based early indication is not supported, gNB can only identify a SBFD-aware UE through UE capability transfer which is later than that through Msg3-based early indication.

Proposal 1: RAN2 confirms the working assumption “Random access procedure in SBFD symbols is supported for all the existing RACH trigger events”.

Proposal 2: RAN2 is asked to discuss whether both RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-3 are supported for each RA event.

Proposal 3: For 4-step CBRA configuration Option 1 with Alt 1-1, RAN2 can wait for RAN1’s progress to determine the additional power control parameters for PRACH transmission in SBFD symbols.

Proposal 4: For 4-step CBRA configuration Option 1 with Alt 2-3, RAN2 can introduce a new IE rach-ConfigCommonSBFD-r19 which parallels to rach-ConfigCommon. FFS details of parameters in ConfigCommonSBFD-r19 IE

Proposal 5: For 4-step CFRA configuration based on Option 1 with Alt 1-1, RAN2 is asked to study whether/which parameter(s) need to be reinterpreted by SBFD-aware UEs in each signalling.

Proposal 6: For 4-step CFRA configuration based on Option 2 with Alt 2-3, a new IE parallel to each legacy IE can be introduced. FFS the behavior of SBFD-aware UEs when some parameters in new IE is absent

Proposal 7: For CFRA, one specific issue to be discussed is whether it is necessary to configure both addition ROs and legacy ROs for one RA event or only to configure one type of ROs, that is, either additional ROs or legacy ROs, is enough.

- Proposal 8: For the initial PRACH transmission attempt, Option 1, that is prioritizing the RO type indicated by the network, is preferred.

- Proposal 9: For CBRA, RAN2 is asked to take the motivation of enabling PRACH transmission in SBFD symbols into consideration when designing the prioritization rule based on DL RSRP, if no explicit indication of RO type from network.

- Proposal 10: For CBRA, only one selection rule for initial RO type is supported/enabled in one cell.

Proposal 11: RAN2 is asked to support SBFD-aware UEs which select leagcy ROs as initial attempt switch to additional ROs after certain (configured) number of times of RACH attempt.

Proposal 12: Within one cell, only one direction, that is, either legacy ROs fallback to additional ROs or additional ROs fallback to additional ROs is supported.

Proposal 13: RAN2 is asked to design the indication of RO type corresponding to each RA event which can trigger CFRA.

Proposal 14: For the available RO type for CFRA, SBFD-aware UE follows the indication from network, which is explicit or implicit.

Proposal 15: RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs.

Proposal 16: RAN2 is asked to wait for RAN1 progress on whether to support SBFD operation for 2-step RACH.

Reference

- “RAN1 Chair’s Notes”, RAN1#118, Maastricht, NL.

- “Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD” RAN2#127bis, Hefei, China.

- “Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD”, RAN2#128, Orlando, USA.

- “RAN1 Chair’s Notes”, RAN1#119, Orlando, USA.