# 3GPP TSG-WG2 Meeting #129                                            R2-2501030

Athens, Greece, Feb. 17th – 21st, 2025

Source: 			CMCC

Title:  		    Discussion on data transmission for A-IoT

Agenda item:	8.2.4

Document for: 	Discussion

Introduction

With the past RAN plenary meeting#106, new WID are approved to support Ambient IoT in NR. RAN scope are listed below:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

In this contribution, we will discuss the data transmission functionalities of A-IoT.

Discussion

Segmentation

As agreed in the study item and captured in TR 38.769, RAN2 agreed to focus D2R segmentation and some preliminary guidance on it. However, we notice that segmentation in the WID is described as “at least in D2R”. The design of R2D segmentation and D2R segmentation are different for the low complexity of A-IoT device compared to network base station. So, we suggest to avoid discussing the D2R and R2D segmentation simultaneously at the beginning of work item. Instead, we should follow the conclusion in SI, only focus D2R segmentation for R19.

Proposal 1: RAN2 confirm only support D2R segmentation for R19 A-IoT.

As we agreed in Study item, the AS layer buffer is not supported for segmentation. Solutions for devices to obtain or maintain the information of segmentation are still open. As proposed by some companies in SI, the D2R data packet to be transmitted can be indicated by either or both of “start point” and “end point” information, or by the length of the data size by the reader.

Although how to specify the “start point” or “end point” has not yet been fully discussed in RAN2, we believe the conception of “start point” or “end point” are specific on A-IoT radio only. We prefer not to equate the “points” with memory address of the devices. Generally, the memory address information is provided by the CN for A-IoT command, which is always encrypted and not available to RAN for security reasons. We suggest RAN2 to confirm such assumption firstly.

Proposal 2: RAN2 assumes that the memory address of A-IoT devices may not be visible to RAN for security reasons.

Proposal 3: Further discuss the how to specify the “start point” or “end point” for each packet or segment.

The device acquires the position of pending data with the memory address information provided in the A-IoT command from the CN. After receiving the D2R data size indicated in the R2D scheduling message, the devices segment the pending data if the data size is smaller than the pending data size. For subsequent segment transmission, the devices should follow the reader control for the segment size.

Proposal 4: The size of each segment is decided and indicated by the reader with R2D message.

Regarding the “start point”, we understand that it can either be provided by the reader for each transmission or be maintained at devices and updated when the reader informs the last segment has been successfully received. The two alternatives can be listed as follows:

- Alternative 1: Reader indicates the “start point” of segment to be transmitted in each R2D message.

- Alternative 2: A-IoT device maintains and updates the “start point” for each D2R transmission based on the acknowledgement of the latest segment.

For Alternative 1, the reader should indicate the “start point” in each R2D message, which may bring overhead to R2D message. However, the A-IoT devices can just follow the scheduling from the reader.

For Alternative 2, the reader should indicate the acknowledgement of last segment in each R2D message, the overhead is slightly smaller than Alternative 1. However, the A-IoT devices have to maintain the “start point”.

Proposal 5: Considering the following solutions for D2R segmentation,

- Alternative 1: Reader indicates the “start point” of segment to be transmitted in each R2D message.

- Alternative 2: A-IoT device maintains and updates the “start point” for each D2R transmission based on the acknowledgement of the latest segment.

For segment transmission, it is beneficial to introduce feedback for segmentation. If feedback indication for segmentation is not supported, the probability of transmission failure after introducing segmentation may be higher than that of transmitting the entire block, because each segment transmission failure may lead to entire block failure. For segmentation Alternative 1, the re-transmission can be implemented by reader repeat the “start point” in R2D message. For segmentation Alternative 2, if device receives transmission failure indicated by reader, it can continue the last “start position” without updated.

Proposal 6: Support optional explicit R2D failure/success feedback indication for D2R segments.

Proposal 7: Support buffer-less re-transmission of D2R segmentation.

Message size reporting

The expected D2R message size is beneficial for reader to perform effective resource allocation and scheduling. As concluded in SA2 SI, the AIoTF may provide approximate D2R message size based on AF request. With such information, the RAN could obtain the rough D2R message size. However, we understand that the message size information is optionally provided by the AIoTF implementation. Solution that device reports the message size is helpful if gNB does not receive the expected message size from CN. One of the direct solutions is device informs the reader about whether the current data packet is the last packet, with an end marker like indicator. In the end marker solution, 1bit is enough. Such an end marker should be carried in every data packet. Even if CN provides the approximate data size information, the end marker is beneficial to confirm that there is no remaining data at device. So, we think end marker solution is always needed. Another solution is to provide total data size reporting by devices roughly or exactly. Because the CN may provide data size to gNB, the total data size reporting from device can be handled under network control. For example, if the gNB receives size information from the CN, it can inform the device to avoid data size reporting. While exact message size reporting can be helpful for the gNB to allocate D2R resources more precisely, considering that the possible data size could be around 1000 bits, the overhead associated with such reporting cannot be ignored.

In summary, 2 alternatives for message size reporting are as follows:

Alternative 1: End marker to indicate whether it is the last packet.

Alternative 2: Total data size reporting, which is under network control. FFS for rough or exact data size.

Proposal 8: Considering the following solutions for message size reporting:

- Alternative 1: End marker to indicate whether it is the last packet.

- Alternative 2: Total data size reporting, which is under network control. FFS for rough or exact data size.

Alternative 1 is suggested as baseline.

AS ID

"AS ID" is used at least for the purpose of D2R scheduling and R2D reception. From higher layer perspective, it is assumed that this "AS ID" should be a short AS layer ID, rather than the full upper layer device ID. Following options are captured in TR38.769 for this "AS ID",

-	Option 1: a random ID (if used in first D2R message) can be reused;

-	Option 2: the reader assigns this "AS ID" to the device. It needs to be further discussed via which R2D message.

-	Option 3: It is up to the reader whether to reuse the random ID (if used in first D2R message) as the "AS ID" or to assign a new "AS ID". It needs to be further discussed via which R2D message.

In option 1, the 16 bits random ID is reused as “AS ID”. Considering that the AS identifier needs to be carried in every data transmission, 16 bits would impose a burden on data transmission. In addition, if the random ID generated by device may be reported by other devices within the same paging around, i.e., multiple devices report the same random ID in Msg1. How to handle the collision should be further discussed. One possible solution is reader informs A-IoT about the collision with a 1 bit indicator. If no collision, the indicator can be “true”, the random ID in Msg 1 is confirmed as “AS ID”; if random ID collision, the indicator should be set to “false”, the A-IoT device can re-generate a random ID in the same paging around or avoid performing re-access by implementation.

In option 2, reader assign “AS ID” to device. With option 2, the “AS ID” can be short than 16 bits. For the subsequent transmission, it may decrease the transmission payload. However, the “AS ID” should be carried in Msg 2 along with random ID to indicate the match information. Compared to option 1, the length of Msg 2 is larger.

In option 3, the “AS ID” can be random ID or assigned by the reader, which rely on reader implementation without considering the collision issue. For details, option 3 has different implementation methods. For example, the indicator of “reuse” or “new assign” can be implicit or explicit. With implicit solution, the devices need to justify whether the new “AS ID” are assigned based on the length of the Msg2. For “reuse” case, no more “ID” is followed, the total length of the Msg 2 can be shorter; for “new assign” case, the “new assign” “AS ID” is carried in Msg2 along with random ID.

The formats can be listed as follows:

|  | Msg2 Format | Length of Msg2 | Length of “AS ID” used for subsequent transmission |
| --- | --- | --- | --- |
| Option 1 | RN16+indicator (confirm or re-generate RN16) | 17 bits | 16 bits |
| Option 2 | RN16+ new “AS ID” (Note: the new “AS ID” can be shorter than 16 bits) | Less than 32 bits | Less than 16 bits |
| Option 3 | RN16+indicator (reuse) RN16+indicator (new)+16 bits “AS ID” (Note: indicator can be explicit or implicit) | 17 bits or 33 bits (explicit indicator) 16 bits or 32 bits (implicit indicator) | 16 bits |

Table-1 Solutions for “AS ID”

Note: New assigned AS ID can be (re)configured in Msg2 or following R2D message.

The above analyses are about how the reader handle the “AS ID” assignment. Something needs to be clarified before we discuss and down-select the optional solutions is that whether “AS ID” is always needed. In our understanding, for inventory-only case, the “AS ID” is not necessary. The device can use RN16 to identify itself during D2R transmission scheduling.

Proposal 9: RAN2 is kindly asked to discuss whether AS ID is always needed, i.e., AS ID is not necessary for inventory-only case.

As preliminary discussion on “AS ID”, we suggest aligning the understanding of the 3 optional solutions in Table-1 firstly. For solution down-selection, we think we should consider the following criteria: the length of Msg2; the length of “AS ID”; device implementation complexity; impacts on supporting if concatenation of Msg2 of multi devices.

Proposal 10: RAN2 is kindly asked to discuss and confirm the understanding of the 3 options in Table-1.

Proposal 11: RAN2 is suggested to consider following criteria for solution down-selection:

- length of Msg2

- length of “AS ID”

- device implementation complexity

- impacts on supporting if concatenation of Msg2 of multi devices

Data transmission failure

How to handle data transmission failure has been discussed in SI. The guidance is A-IoT device just follow the reader subsequent R2D instructions. From device perspective, how to identify the R2D message type and how to response to “repeat” message should be discussed in WI. Before that, we need make it clear whether it is necessary to distinguish the “repeat” R2D message or just acknowledge as initial R2D message. As we discussed and concluded during segmentation issue, the AS buffer is not considered in A-IoT. So, devices can not store the transmitted data for so-called “re-send”. For gNB reader, it can retransmit Msg2 if it fails to receive Msg3. In that case, the reader should include AS ID in Msg2 as discussed above. Considering network implementation, the A-IoT enable gNB can update the allocated AS ID along with Msg2. Correspondingly, the device needs to update the AS ID along with Msg3. As conclusion, data transmission failure can be handled by reader (A-IoT enable gNB) implementation, for example, re-send the data with R2D scheduling in same slot, or access in following access round or paging round when receive NACK for Msg3 from reader.

Proposal 12: Data transmission failure can be handled by reader (A-IoT enable gNB) implementation, for example, re-send the data with R2D scheduling in same slot, or access in following access round or paging round.

Proximity determination

2 solutions on proximity determination have been captured in the TR. According to the TR, device side measurement has been excluded. Potential specification impacts or not will be discussed in work item.

Solution 1: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader. Solution 2: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader based on measurements at the reader side.

Considering the two solutions above, the reader judges the devices are nearby once the reader receives the response from the device or the quality of the response is higher than some thresholds. From procedure perspective, both solution 1 and solution 2 can be covered by current inventory and command procedure. The D2R transmission received by A-IoT enabled gNB can be Msg1, Msg3 or subsequent data transmission. Other types of D2R message or data, which are included in procedures other than inventory and command, should be avoided. In additional, for the scope of R19 WID, only topology 1 is considered. It is the A-IoT enabled gNB that judges and makes decision. So, the behaviours after receiving D2R transmission can be left to network implementations. No additional impacts are identified from RAN2’s perspective. Whether and how to trigger proximity determination procedure from CN to A-IoT enabled gNB is RAN3 scope.

Obsevation1: Solution 1 and solution 2 on proximity determination can be covered by current inventory and command procedure

Proposal 13: A-IoT enabled gNB behaviours after receiving D2R transmission can be left to network implementations.

Proposal 14-1: No additional impacts on proximity determination are identified in RAN2.

Proposal 14-2: Whether and how to trigger proximity determination procedure from CN to A-IoT enabled gNB is RAN3 scope.

Conclusion

According to the above discussion, the following observation and proposals are given:

segmentation

Proposal 1: RAN2 confirm only support D2R segmentation for R19 A-IoT.

Proposal 2: RAN2 assumes that the memory address of A-IoT devices may not be visible to RAN for security reasons.

Proposal 3: Further discuss the how to specify the “start point” or “end point” for each packet or segment.

Proposal 4: The size of each segment is decided and indicated by the reader with R2D message.

Proposal 5: Considering the following solutions for D2R segmentation,

- Alternative 1: Reader indicates the “start point” of segment to be transmitted in each R2D message.

- Alternative 2: A-IoT device maintains and updates the “start point” for each D2R transmission based on the acknowledgement of the latest segment.

Proposal 6: Support optional explicit R2D failure/success feedback indication for D2R segments.

Proposal 7: Support buffer-less re-transmission of D2R segmentation.

Message size reporting

Proposal 8: Considering the following solutions for message size reporting:

- Alternative 1: End marker to indicate whether it is the last packet.

- Alternative 2: Total data size reporting, which is under network control. FFS for rough or exact data size.

Alternative 1 is suggested as baseline.

AS ID

Proposal 9: RAN2 is kindly asked to discuss whether AS ID is always needed, i.e., AS ID is not necessary for inventory-only case.

Proposal 10: RAN2 is kindly asked to discuss and confirm the understanding of the 3 options in Table-1.

|  | Msg2 Format | Length of Msg2 | Length of “AS ID” used for subsequent transmission |
| --- | --- | --- | --- |
| Option 1 | RN16+indicator (confirm or re-generate RN16) | 17 bits (fix) | 16 bits |
| Option 2 | RN16+ new “AS ID” (Note: the new “AS ID” can be shorter than 16 bits) | Less than 32 bits | Less than 16 bits |
| Option 3 | RN16+indicator (reuse) RN16+indicator (new)+16 bits “AS ID” (Note: indicator can be explicit or implicit) | 17 bits or 33 bits (explicit indicator) 16 bits or 32 bits (implicit indicator) | 16 bits |

Note: New assigned AS ID can be (re)configured in Msg2 or following R2D message.

Proposal 11: RAN2 is suggested to consider following criteria for solution down-selection:

- length of Msg2

- length of “AS ID”

- device implementation complexity

- impacts on supporting if concatenation of Msg2 of multi devices

Data transmission failure

Proposal 12: Data transmission failure can be handled by reader (A-IoT enable gNB) implementation, for example, re-send the data with R2D scheduling in same slot, or access in following access round or paging round.

Proximity determination

Obsevation1: Solution 1 and solution 2 on proximity determination can be covered by current inventory and command procedure

Proposal 13: A-IoT enabled gNB behaviours after receiving D2R transmission can be left to network implementations.

Proposal 14-1: No additional impacts on proximity determination are identified in RAN2.

Proposal 14-2: Whether and how to trigger proximity determination procedure from CN to A-IoT enabled gNB is RAN3 scope.

Reference

[1] RP-243326, New work item: Solutions for Ambient IoT (Internet of Things) in NR