3GPP TSG RAN WG2 Meeting #129	R2-2501032

Athens, GR, Feb. 17 – 21, 2025

Agenda Item:	8.8.3

Source:	CMCC

Title:	Discussion on uplink capacity/throughput enhancement for NR NTN

Document for:	Discussion, Discussion

Introduction

For uplink capacity/throughput enhancement for NR NTN, according to RAN1 discussion progress, two OCC code lengths are support at least and related agreements are achieved as following:

- RAN1 #118:

Agreement At least one of the OCC techniques when PUSCH repetitions are used will be specified: - Inter-slot time-domain OCC with OCC length 2 - Inter-slot time-domain OCC with OCC length 2 and 4 - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 and 4 - Note: combination of techniques is not precluded - PUSCH repetition Type B is not considered Conclusion Multiplexing of 8 UEs with PUSCH OCC is not discussed in RAN1 until the work for multiplexing of less than 8 UEs has been completed.

- Then in RAN1 #119 meeting, RAN1 confirm the WA of RAN1 #118bis with revision as follows:

Agreement RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows: - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences - RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact. - RAN1 assumes no specification impact for CFO grouping - RAN1 does not pursue closed-loop frequency adjustment commands. - RAN1 assumes that RAN4 does not define new UE requirements for CFO.

In this contribution, we would like to provide some discussion on RAN2 aspects about uplink capacity/throughput enhancement based on above agreements reached in RAN1.

Discussion

Access control

Similar to existing cell bar mechanism, for R19 UEs supporting OCC, a new bar bit could be introduced in SIB1 to apply the access control. And if the new bar bit is set to notbarred, it means that the current cell allows the R19 UE supporting OCC to access, otherwise, not allowed. Then is the new bar bit is not present, the R19 UEs supporting OCC could follow the existing NTN bar bit (i.e. cellBarredNTN).

However, some companies may have some concerns that the bar bits introduced is a little too much and new bar bit for R19 UEs supporting OCC may not be necessary. And we could just follow the existing access control mechanism for legacy NTN UEs as the following.

- 5.2.2.4.2 Actions upon reception of the SIB1 Upon receiving the SIB1 the UE shall: 1> store the acquired SIB1; 1> if the access is for NTN: 2> if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running: 3> if the cellBarredNTN in the acquired SIB1 is set to barred or the cellBarredNTN is not included in the acquired SIB1: 4> consider the cell as barred in accordance with TS 38.304 [20]; 4> perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends; 3> if the UE is a fixed VSAT UE and the cellBarredFixedVSAT in the acquired SIB1 is set to barred or the cellBarredFixedVSAT is not included in the acquired SIB1, or 3> if the UE is a mobile VSAT UE and the cellBarredMobileVSAT in the acquired SIB1 is set to barred or the cellBarredMobileVSAT is not included in the acquired SIB1: 4> consider the cell as barred in accordance with TS 38.304 [20]; 4> perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

Therefore, RAN2 could discuss the access control mechanism for R19 UEs supporting OCC between the two options: introduce new bar bit with code points {barred, notbarred} or reuse the existing cellBarredNTN.

Proposal 1: Kindly suggest RAN2 to discuss the access control mechanism for R19 UEs supporting OCC between the two options:

Option 1: introduce new bar bit in SIB1

Option 2: reuse the existing cellBarredNTN.

UE capability

In last RAN1 meeting, the UE capability aspects is confirmed as follows:

Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

Then in RAN2, we need to introduce new UE capability to support OCC to keep alignment. Define one UE capability with OCC length 2 and other one capability with OCC length 4 which means also support OCC length 2 simultaneously.

Proposal 2: Introduce 2 new UE capabilities to support OCC length 2 and OCC length 4 respectively, for OCC length 4, simultaneously supporting OCC length 2 is mandatory.

ASN.1 impact

Existing OCC related configuration in PUCCH-Config is as following:

<<omitted>> PUCCH-Resource ::= SEQUENCE { pucch-ResourceId PUCCH-ResourceId, startingPRB PRB-Id, intraSlotFrequencyHopping ENUMERATED { enabled } OPTIONAL, -- Need R secondHopPRB PRB-Id OPTIONAL, -- Need R format CHOICE { format0 PUCCH-format0, format1 PUCCH-format1, format2 PUCCH-format2, format3 PUCCH-format3, format4 PUCCH-format4 } } PUCCH-ResourceExt-v1610 ::= SEQUENCE { interlaceAllocation-r16 SEQUENCE { rb-SetIndex-r16 INTEGER (0..4), interlace0-r16 CHOICE { scs15 INTEGER (0..9), scs30 INTEGER (0..4) } } OPTIONAL, --Need R format-v1610 CHOICE { interlace1-v1610 INTEGER (0..9), occ-v1610 SEQUENCE { occ-Length-v1610 ENUMERATED {n2,n4} OPTIONAL, -- Need M occ-Index-v1610 ENUMERATED {n0,n1,n2,n3} OPTIONAL -- Need M } } OPTIONAL, -- Need R ..., [[ format-v1700 SEQUENCE { nrofPRBs-r17 INTEGER (1..16) } OPTIONAL, -- Need R pucch-RepetitionNrofSlots-r17 ENUMERATED { n1,n2,n4,n8 } OPTIONAL -- Need R ]], [[ applyIndicatedTCI-State-r18 ENUMERATED {first, second, both, spare1} OPTIONAL, -- Need R multipanelSFN-Scheme-r18 ENUMERATED {enabled} OPTIONAL, -- Need R dl-DataToUL-ACK-r18 SetupRelease { DL-DataToUL-ACK-r18 } OPTIONAL, -- Need M dl-DataToUL-ACK-DCI-1-2-r18 SetupRelease { DL-DataToUL-ACK-DCI-1-2-r18} OPTIONAL -- Need M ]] } PUCCH-ResourceId ::= INTEGER (0..maxNrofPUCCH-Resources-1) PUCCH-format0 ::= SEQUENCE { initialCyclicShift INTEGER(0..11), nrofSymbols INTEGER (1..2), startingSymbolIndex INTEGER(0..13) } PUCCH-format1 ::= SEQUENCE { initialCyclicShift INTEGER(0..11), nrofSymbols INTEGER (4..14), startingSymbolIndex INTEGER(0..10), timeDomainOCC INTEGER(0..6) } PUCCH-format2 ::= SEQUENCE { nrofPRBs INTEGER (1..16), nrofSymbols INTEGER (1..2), startingSymbolIndex INTEGER(0..13) } PUCCH-format3 ::= SEQUENCE { nrofPRBs INTEGER (1..16), nrofSymbols INTEGER (4..14), startingSymbolIndex INTEGER(0..10) } PUCCH-format4 ::= SEQUENCE { nrofSymbols INTEGER (4..14), occ-Length ENUMERATED {n2,n4}, occ-Index ENUMERATED {n0,n1,n2,n3}, startingSymbolIndex INTEGER(0..10) } <<omitted>>

PUCCH-Resource, PUCCH-ResourceExt field descriptions

<<omitted>>

format Selection of the PUCCH format (format 0 – 4) and format-specific parameters, see TS 38.213 [13], clause 9.2. format0 and format1 are only allowed for a resource in a first PUCCH resource set. format2, format3 and format4 are only allowed for a resource in non-first PUCCH resource set. The network can only configure format-v1610 when format is set to format2 or format3. The network only configures format-v1700 when format is set to format0, format1 or format4.

<<omitted>>

occ-Index Indicates the orthogonal cover code index (see TS 38.213 [13], clause 9.2.1). This field is applicable when useInterlacePUCCH-PUSCH-16 is configured.

occ-Length Indicates the orthogonal cover code length (see TS 38.213 [13], clause 9.2.1). This field is applicable when useInterlacePUCCH-PUSCH-16 is configured.

Similarly, to provide OCC parameters configuration for PUSCH, we could consider to introduce a new IE (e.g. occ-v19) including OCC parameters (e.g. occ-length and occ-index ) in PUSCH-Config. And network could indicate specific configuration based on UE capability reporting.

Proposal 3: It is proposed to introduce a new IE (e.g. occ-v19) including OCC parameters (e.g. occ-length and occ-index ) in PUSCH-Config to provide OCC related configuration for PUSCH as following:

PUSCH-Config information element

-- ASN1START

-- TAG-PUSCH-CONFIG-START

PUSCH-Config ::=                        SEQUENCE {

<<omitted>>

[[

occ-v19                        SEQUENCE {

occ-Length-v19       ENUMERATED {n2,n4}                                            OPTIONAL, -- Need M

occ-Index-v19       ENUMERATED {n0,n1,n2,n3}                                      OPTIONAL  -- Need M

]]

}

<<omitted>>

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on RAN2 aspects about uplink capacity/throughput enhancement for NR NTN and have the following proposals:

Proposal 1: Kindly suggest RAN2 to discuss the access control mechanism for R19 UEs supporting OCC between the two options:

Option 1: introduce new bar bit in SIB1

Option 2: reuse the existing cellBarredNTN.

Proposal 2: Introduce 2 new UE capabilities to support OCC length 2 and OCC length 4 respectively, for OCC length 4, simultaneously supporting OCC length 2 is mandatory.

Proposal 3: It is proposed to introduce a new IE (e.g. occ-v19) including OCC parameters (e.g. occ-length and occ-index ) in PUSCH-Config to provide OCC related configuration for PUSCH as following:

PUSCH-Config information element

-- ASN1START

-- TAG-PUSCH-CONFIG-START

PUSCH-Config ::=                        SEQUENCE {

<<omitted>>

[[

occ-v19                        SEQUENCE {

occ-Length-v19       ENUMERATED {n2,n4}                                            OPTIONAL, -- Need M

occ-Index-v19       ENUMERATED {n0,n1,n2,n3}                                      OPTIONAL  -- Need M

]]

}

<<omitted>>

Reference

- Chair notes RAN1#118 eom0

- Chair notes RAN1#118bis eom0

- Chair notes RAN1#119 eom0

- 3GPP TS 38.331 V18.3.0 NR; Radio Resource Control (RRC) protocol specification (Release 18)