3GPP TSG RAN WG2 Meeting #129	R2-2501035

Athens, GR, Feb. 17 – 21, 2025

Agenda Item:	8.9.4

Source:	CMCC

Title:	Support of PWS messages for NB-IoT

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on support of PWS for NB-IoT and some agreements were achieved as follows [1]:

Agreements:

1.	We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.

2.	We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed

3.	Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication.

In this contribution, we would like to provide some further considerations on the PWS for NB-IoT.

Discussion

SIB1-NB acquisition

We have agreed to extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. For whether support SIB1-NB acquisition is FFS. Per the existing system information acquisition procedure, UE acquires the corresponding ETWS or CMAS system information block respectively. However there is no “acquiring SystemInformationBlockType1 immediately” related description.

Then for paging reception, the following mechanism is supported currently. If etws-Indication and/or cmas-Indication is included in Paging message and the UE is ETWS/CMAS capable, UE would re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary.

<<omitted>> if the etws-Indication is included and the UE is ETWS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary; 2> if the schedulingInfoList indicates that SystemInformationBlockType10 is present: 3> acquire SystemInformationBlockType10; NOTE 2: If the UE is in CE, it is up to UE implementation when to start acquiring SystemInformationBlockType10. 2> if the schedulingInfoList indicates that SystemInformationBlockType11 is present: 3> acquire SystemInformationBlockType11; 1> if the cmas-Indication is included and the UE is CMAS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary as specified in 5.2.1.5; 2> if the schedulingInfoList indicates that SystemInformationBlockType12 is present: 3> acquire SystemInformationBlockType12; <<omitted>>

Therefore, we could just extend the existing RRC procedure about PWS notification to NB-IoT, and no need for SIB1-NB acquisition.

Proposal 1: No further enhancement is needed for SIB1-NB acquisition.

UE capability

In existing TS 36.306, the PWS feature is optional to support without UE radio access capability.

- 6.2 PWS features - 6.2.1 ETWS It is optional for UE to support ETWS reception as specified in TS 36.331 [5]. - 6.2.2 CMAS It is optional for UE to support CMAS reception as specified in TS 36.331 [5]. It is optional for a CMAS-capable UE to support Geofencing information (warningAreaCoordinates-r15) as specified in TS 36.331 [5]. - 6.2.3 KPAS It is optional for UE to support KPAS reception as specified in TS 36.331 [5]. The Korean Public Alert System (KPAS) uses the same AS mechanisms as defined for CMAS. Therefore a KPAS-capable UE shall support all behaviour that is included in TS 36.331 [5] and TS 36.304 [14] for a CMAS-capable UE. - 6.2.4 EU-Alert It is optional for UE to support EU-Alert reception as specified in TS 36.331 [5]. The Europearn Union Warning System EU-Alert uses the same AS mechanisms as defined for CMAS. Therefore a EU-Alert-capable UE shall support all behaviour that is included in TS 36.331 [5] and TS 36.304 [14] for a CMAS-capable UE.

Then section 4 provides optional features applicable in NB-IoT without UE radio access capability parameters. However there is no PWS feature, that is the PWS feature in section 6.2 depicted above is not applicable in NB-IoT, as description in section 4.

The following optional features without UE radio access capability parameters specified in clause 6 are applicable in NB-IoT: - RRC Connection Re-establishment for the Control Plane CIoT EPS Optimization (clause 6.7.5) - System Information Block Type 16 (clause 6.8.1) - Enhanced random access power control (clause 6.8.3) - MT-EDT for Control Plane CIoT EPS Optimisation (clause 6.8.10) - MT-EDT for User Plane CIoT EPS Optimisation (clause 6.8.11) - EDT for Control Plane CIoT EPS Optimization (clause 6.8.4) - Enhanced PHR (clause 6.8.6) - Carrier specific NRSRP thresholds for NPRACH resource selection (clause 6.8.15) - Radio Link Failure Report for NB-IoT (clause 6.10.2) - SC-PTM in Idle mode (clause 6.16.1) - Multiple TB scheduling for SC-PTM in Idle mode for NB-IoT (clause 6.16.2) - Relaxed monitoring (clause 6.17.1) - DL channel quality reporting in Msg3 for the anchor carrier (clause 6.17.2) - Serving cell idle mode measurements reporting (clause 6.17.3) - NSSS-Based RRM measurements (clause 6.17.4) - NPBCH-Based RRM measurements (clause 6.17.5) - RRM measurements on non-anchor paging carriers (clause 6.17.6) - NRS presence on non-anchor paging carriers (clause 6.17.7) - DL channel quality reporting in Msg3 for non-anchor carrier (clause 6.17.8) - Assistance information for inter-RAT cell selection to/from NB-IoT (clause 6.17.9) - RRC Connection Re-establishment for the Control Plane CIoT 5GS Optimisation (clause 6.18.3) - NB-IoT/5GC (clause 6.18.4) - MO-EDT for Control Plane CIoT 5GS Optimisation (clause 6.18.5) - AS RAI (clause 6.18.6) - Cell reselection measurements triggering based on service time (clause 6.19.1) - Discontinuous coverage (clause 6.19.2). - Early RLF triggering based on service time (clause 6.19.3). - Neighbour cell measurements based on service start time of the neighbour cell (clause 6.19.4). - UE autonomous release based on service time (clause 6.19.5). - Cell reselection measurements triggering based on location for (quasi-)fixed cell (clause 6.19.6). - Cell reselection measurements triggering based on location for earth moving cell (clause 6.19.7). - GNSS measurements during inactive time (clause 6.19.8). - SystemInformationBlockType33(-NB) reception in a TN cell (clause 6.19.9). The optional features without UE radio access capability parameters specified in clause 6 are not applicable in NB-IoT, unless they are listed above.

Therefore, to support PWS in NB-IoT, a simple solution is to add support PWS in section 4 of TS 36.306, then the PWS feature in 6.2 could be apply to NB-IoT directly.

Proposal 2: Introduce PWS reception in section 4 of TS 36.306, then the PWS feature in 6.2 could be apply to NB-IoT directly.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on broadcast of PWS messages for NB-IoT and have the following proposals:

Proposal 1: No further enhancement is needed for SIB1-NB acquisition.

Proposal 2: Introduce PWS reception in section 4 of TS 36.306, then the PWS feature in 6.2 could be apply to NB-IoT directly.

Reference

- R2-128 NR-NTN-IoT-NTN (Sergio)_ EOM

- RP-242397 WID Rev R19 IOT-NTN cl

- 3GPP TS 36.300 V18.2.0 Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 18)

- 3GPP TS 36.331 V18.2.0 Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 18)