3GPP TSG RAN WG2 Meeting #129	R2-2501036

Athens, GR, Feb. 17 – 21, 2025

Agenda Item:	8.17

Source:	CMCC

Title:	Support of IoT-NTN TDD mode

Document for:	Discussion, Decision

Introduction

In RAN #105 new WID on introduction of IoT-NTN TDD mode is agreed, the latest WID scope is approved in RAN #106 based on the study of RAN1/4, and RAN2 related work is excerpted as following[1]:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2]

In this contribution, we would like to provide some considerations on support of IoT-NTN TDD mode.

Discussion

User plane impact

  - 2.1.1 Windows/timers enhancements

Per the existing specification, for IoT-NTN FDD, considering the long propagation delay, there are several UP timers/windows are extended or delay start with UE-eNB RTT. As shown in the Table 1, the start of ra-ResponseWindow/mac-ContentionResolutionTimer/pur-ResponseWindowTimer is delayed with UE-eNB RTT. And UL HARQ RTT timer is extended with UE-eNB RTT. Then for RLC t-Reordering timer and PDCP discardTimer, new values are introduced.

Table 1 Existing enhanced UP window/timer for IoT-NTN FDD

| Layer 2 | Enhanced window/timer | Specific enhancements |
| --- | --- | --- |
| MAC | ra-ResponseWindow | delay start with UE-eNB RTT |
|  | mac-ContentionResolutionTimer | delay start with UE-eNB RTT |
|  | pur-ResponseWindowTimer | delay start with UE-eNB RTT |
|  | UL HARQ RTT timer | Extension with UE-eNB RTT |
| RLC | t-Reordering | adding new values {ms2200, ms3200} |
| PDCP | discardTimer(only for eMTC UE) | adding new value ms2000 |

To support TDD for NB-IoT NTN with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period, the transmission delay would be further increase due to the new TDD pattern, therefore, the windows/timers enhancements of user plane are needed to discuss. Although for PDCP discardTimer, enhancement were only made for eMTC in previous release, and the current maximum value for NB-IoT is 81920ms, to support the new TDD pattern with a longer period would be a different story, PDCP discardTimer extension for NB-IoT could also be considered. Therefore, at least, the UP windows/timers mentioned above as well as HARQ RTT timer could be the starting point.

Proposal 1: Suggest RAN2 to discuss the enhancements for UP windows/timers. And a-ResponseWindow, mac-ContentionResolutionTimer, pur-ResponseWindowTimer, (UL) HARQ RTT timer, RLC t-Reordering timer, PDCP discardTimer could be the starting point.

  - 2.1.2 HARQ

And for HARQ of NB-IoT NTN FDD, in R18, we supported DL HARQ feedback disabled to support long propagation delay based on network configuration. For NB-IoT NTN TDD, considering the characteristic of the new TDD structure, for one HARQ process, the interval between a new transmission and a retransmission or two retransmission would be too long. That is a feedback used to determine whether a retransmission is needed may be not important. And we could consider an solution (e.g. add related description in TS 36.300) that if the NB-IoT NTN system operates in TDD mode, the HARQ feedback is disabled as default.

Proposal 2: It is proposed that if the NB-IoT NTN system operates in TDD mode, the HARQ feedback is disabled by default, and to be simple, we could just add related description in TS 36.300.

Control plane impact

  - 2.2.1 System information scheduling

Based on the scope, there are only 8 consecutive DL subframes in the TDD pattern, then SIB1-NB only could be transmitted at most once in one TDD pattern period, which could not even support the repetition mechanism against the poor coverage that some IoT devices deployed. Then for other SIB-NB information scheduled by SIB1-NB, the corresponding SI window may be also affected. As in the current TS 36.331, for NB-IoT UE, the related SI window determination is as following:

- 5.2.3a Acquisition of an SI message by BL UE or UE in CE or a NB-IoT UE When acquiring an SI message, the BL UE or UE in CE or NB-IoT UE shall: - determine the start of the SI-window for the concerned SI message as follows: 2> if the concerned SI message is configured in the schedulingInfoList, schedulingInfoListExt (if present) or if the concerned SI message is configured in the posSchedulingInfoList and si-posOffset is not configured; 3> for the concerned SI message, determine the number n which corresponds to the order of entry in the concatenated list of SI messages configured by schedulingInfoList, schedulingInfoListExt (if present) in SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) and posSchedulingInfoList in SystemInformationBlockType1-BR; 3> determine the integer value x = (n – 1)*w, where w is the si-WindowLength-BR (or si-WindowLength in NB-IoT); 3> if the UE is a NB-IoT UE: 4> the SI-window starts at the subframe #0 in the radio frame for which (H-SFN * 1024 + SFN) mod T = FLOOR(x/10) + Offset, where T is the si-Periodicity of the concerned SI message and, Offset is the offset of the start of the SI-Window (si-RadioFrameOffset); - <<omitted>>

However, for NB-IoT TDD mode, the start calculation of SI-window needs to take the new TDD pattern into account. For example, the input parameter x affected by the si-WindowLength whose value range may not enough for the new TDD pattern. And based on the agreement of RAN1 #119, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods (Iridium TDMA structure is shown in figure 1), which would lead to that the actual calculated subframe #0 may not be used. Therefore we need to discuss the impacts for SI acquisition based on the new TDD pattern.

Figure 1 Legacy system with TDD pattern deployed in the 1616-1626.5 MHz band

Proposal 3: SI acquisition needs to consider the new TDD pattern.

  - 2.2.2 Paging

Per the existing TS 38.304, one Paging Frame (PF) is one Radio Frame, which may contain one or multiple Paging Occasion(s). PO, PF are determined by following formula:

PF is given by following equation: SFN mod T= (T div N)*(UE_ID mod N) Index i_s pointing to PO from subframe pattern defined in 7.2 will be derived from following calculation: i_s = floor(UE_ID/N) mod Ns Parameters used for the calculation of the PF, i_s: - T: DRX cycle of the UE. - nB: 4T, 2T, T, T/2, T/4, T/8, T/16, T/32, T/64, T/128, and T/256, and for NB-IoT also T/512, and T/1024. - N: min(T,nB)

The current subframe patterns for both NB-IoT TDD and FDD are provided as the following:

- 7.2 Subframe Patterns FDD: - If P-RNTI is transmitted on PDCCH or NPDCCH, or if P-RNTI is transmitted on MPDCCH with system bandwidth > 3MHz: Ns; PO when i_s=0; PO when i_s=1; PO when i_s=2; PO when i_s=3. 1; 9; N/A; N/A; N/A. 2; 4; 9; N/A; N/A. 4; 0; 4; 5; 9. TDD (all UL/DL configurations): - If P-RNTI is transmitted on PDCCH or NPDCCH, or if P-RNTI is transmitted on MPDCCH with system bandwidth > 3MHz: Ns; PO when i_s=0; PO when i_s=1; PO when i_s=2; PO when i_s=3. 1; 0; N/A; N/A; N/A. 2; 0; 5; N/A; N/A. 4; 0; 1; 5; 6.

Based on the new TDD pattern, firstly the paging delay would increase obviously due to the limited DL resources. Then with N=9 and D=8, and the specific DL subframes options within D=8 discussed in RAN1, the actual PF and PO needs to be reconsidered due to the actual available DL subframes.

Proposal 4: Paging enhancement (e.g. actual available PF/PO) is needed based on the new TDD pattern.

  - 2.2.3 Measurement Gap

Based on the current TS 36.133, in E-UTRA-NR dual connectivity mode, NR - E-UTRA dual connectivity mode and E-UTRA standalone mode, all gap patterns #0~11 in Table 8.1.2.1-1 can be configured  for measurements of NR carriers only, and gap pattern#0, 1, 2, 3, 4, 6, 7, 8, 10 can be configured for measurements of E-UTRA carriers with the applicability as specified in Table 8.1.2.1-1.

Table 8.1.2.1-1: Gap Pattern Configurations supported by the UE

| Gap Pattern Id | MeasurementGap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) | Minimum available time for inter-frequency and inter-RAT measurements during 480ms period (Tinter1, ms) | Measurement Purpose |
| --- | --- | --- | --- | --- |
| 0 | 6 | 40 | 60 | Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x, inter-RAT NR |
| 1 | 6 | 80 | 30 | Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x, inter-RAT NR |
| 2 | 3 | 40 | 24NOTE 1,2 | Inter-Frequency E-UTRAN FDD and TDD for cells with time difference as specified below. inter-RAT NR |
| 3 | 3 | 80 | 12NOTE 1,2 | Inter-Frequency E-UTRAN FDD and TDD for cells with time difference according as specified below. inter-RAT NR |
| 4 | 6 | 20 | 120 Note 1 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 5 | 6 | 160 | Note 3 | inter-RAT NR |
| 6 | 4 | 20 | 72 Note 1, 5, 7 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 7 | 4 | 40 | 36 Note 1, 5, 8 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 8 | 4 | 80 | 18Note 1, 5, 9 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 9 | 4 | 160 | Note 3 | inter-RAT NR |
| 10 | 3 | 20 | 48 Note 1, 5 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 11 | 3 | 160 | Note 3 | inter-RAT NR |
| 24 | 10 | 80 | 54Note 1 | Inter-Frequency E-UTRAN FDD and TDD Note 11 |
| 25 | 20 | 160 | Note 3 | Note 11 |
| NOTE 1: When determing UE requirements using Tinter1 for GP2, 3, 4, 6, 7, 8, 10, Tinter1 = 60 for GP2, GP4, GP6, GP7, GP10 and Tinter1 = 30 for GP3, GP8 and GP24 shall be used. NOTE 2: Void. NOTE 3: This gap pattern can be used only for measurement of NR carriers, and Tinter is not applicable. NOTE 4: Void NOTE 5: Void. NOTE 6: This gap pattern is supported by UEs which are configured to perform both E-UTRA inter-frequency measurement and inter-RAT NR measurement or supported by UEs configured to perform inter-RAT NR measurement only. NOTE 7: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 48ms corresponding to the first 3ms of the 4ms gap NOTE 8: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 24ms corresponding to the first 3ms of the 4ms gap NOTE 9: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 12ms corresponding to the first 3ms of the 4ms gap NOTE 10: For UE only supporting measGapPatterns-NRonly-r16 for the corresponding gap patterns among GP2-11, the corresponding gap patterns are only applicable to measurement of inter-RAT NR. NOTE 11: Measurement gap patterns #24 and #25 can be requested by NR PCell [38] only when the UE is configured at least with any of RSTD, UE Rx-Tx, or PRS-RSRP measurements requiring such gaps and can only be used during the corresponding positioning measurement period |  |  |  |  |

From our perspective, the parameters MeasurementGap Length and measurement gap repetition period would be affected by the available DL subframes. And based on the current measurement gap configuration, the value range of MeasurementGap Length may be suitable to the new TDD pattern based on network configuration. While for the measurement gap repetition period, some new values could be considered based on the available DL subframes in one new TDD pattern period, for example, 90ms (i.e. corresponding to the period of the new TDD pattern) and multiples of 90ms. Therefore, measurement gap configuration enhancements could be discussed to match the new TDD pattern.

Proposal 5: It is proposed that measurement gap configuration enhancements (e.g introduction of new measurement gap repetition period values) could be discussed to match the new TDD pattern.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on IoT-NTN TDD mode and have the following proposals:

Proposal 1: Suggest RAN2 to discuss the enhancements for UP windows/timers. And a-ResponseWindow, mac-ContentionResolutionTimer, pur-ResponseWindowTimer, (UL) HARQ RTT timer, RLC t-Reordering timer, PDCP discardTimer could be the starting point.

Proposal 2: It is proposed that if the NB-IoT NTN system operates in TDD mode, the HARQ feedback is disabled by default, and to be simple, we could just add related description in TS 36.300.

Proposal 3: SI acquisition needs to consider the new TDD pattern.

Proposal 4: Paging enhancement (e.g. actual available PF/PO) is needed based on the new TDD pattern.

Proposal 5: It is proposed that measurement gap configuration enhancements (e.g introduction of new measurement gap repetition period values) could be discussed to match the new TDD pattern.

Reference

- RP-243293 Revised WID on introduction of IoT-NTN TDD_rm

- 3GPP TS 36.300 V18.4.0 Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 18)

- 3GPP TS 36.331 V18.4.0 Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 18)

- 3GPP TS 36.304 V18.3.0 Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode (Release 18)

- 3GPP TS 36.133 V18.8.0 Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 18)