3GPP TSG-WG2 Meeting #129                                                                            R2-2501037

Athens, GR, 17th – 21st February 2025

Agenda item:	8.8.2

Title:	Analysis on DL coverage enhancements due to extended SIB periodicity

Source:	CMCC

Document for:	Discussion

- Introduction

In previous RAN#106 meetings, the WID scope of downlink coverage enhancement in NR NTN had been discussed,  the following agreements has acheived:

- System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - ... - Rel-18 network energy saving techniques should be considered as baseline in the system level study

This means RAN has agreed that the maximum of the extended default SSB dectetion periodicity is 160ms. As we konw, the current RRC signalling can support the SSB periodicity value of 160ms,  hence, in this contribution, we will further provide our views on on RAN2 aspects of DL coverage enhancements under this limitation.

- SMTC impacts for DL Coverage enhancement

In previous RAN2#128 meetings, SMTC impacts for downlink coverage enhancement in NR NTN had been discussed,  the following agreements has acheived:

Agreements:

1.	As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs)

2.	RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

According to the WID objective,  the maximum of the extended default SSB dectetion periodicity is 160ms:

Currentlty, the main argument was whether to extend the SMTC number.

Firstly, there is no need to extend the number of SMTC due to default SSB dectetion periodicity extension, since current SMTC/SSB configuration siganalling can support the periodicity value of 160ms.

Secondly, as a serving cell has more than one neighbour cells typically, and a separate SMTC may be needed for each of the neighbour cell, because the offsets of neighbour cells' SMTCs may be not synchronized between each other, in case of the beam hopping. However, before discussing the required number of SMTCs, we need to analyze how beam hopping affects the SMTC configuration, which requires a precise understanding of how the SSBs are transmitted via beam hopping.

In our understanding, for the NTN cell, according to the agreed assumption in RAN1, a satellite may have large satellite footprint and limited feeder link bandwidth, especially in Set1-2 in RAN1 agreement. For Set1-2, the number of active beams is just 16, which is constrained by satellite payload capability, it is not be able to provide NR service for all beam footprints, the total number of the beam footprints is 1058.

Figure 1 Two scenarios of the satellite target coverage area in NTN system

As shown in Figure1, there are mainly 2 options which can be considered for the mapping of NR beams and NR cells to one satellite beams:

- Scenario 1: Different PCIs for mutiple beam footprints in one satellite cell (1-to-1 mapping);

- Scenario 2: Single PCI for Multi-beam footprints in one satellite cell (multiple-to-1 mapping)

Regarding Scenario 1, which might be considered as a baseline consist in one-to-one mapping of one beam (similar to one Terrestrial-network cell) to Satellite cell. In option 1, the beam hopping can be modelled as cell switching. Hence, NR Beam management operation is not needed and the current SMTC number and pattern definition is sufficient.

Proposal 1: for senario1, i.e. different PCIs for mutiple beam footprints in one satellite cell (1-to-1 mapping), the current SMTC number and pattern definition is sufficient.

Then for scenario 2, multiple NR beams per NR cell, with the same cell (PCI) is mapped to partial numbers of satellite beams (e.g., quarter beams in one satellite cell). This means for different beam/footprint (group) in a single cell,  there will be different SSB start offset. Therefore,  the current granulartiy of the current SMTC number and SSB pattern definition is possible not sufficient at least for the UE in idle/inactive mode, even for the serving cell measurement itself.  Since for the connected UE, the RRC signalling can be indicated in UE-specific level to address the issue.

Figure 2 illustrates the time-domain positioning information of SSB in satellite scenarios with beam hopping

Figure 3 illustrates diverse SSB delivery modes in satellite scenarios with beam hopping

Observation 1: Since for different beam/footprint (group) in a single cell,  there will be different SSB start offset,  the current granulartiy of the current SMTC number and SSB pattern definition is possible not sufficient at least for the UE in idle/inactive mode, even for the serving cell measurement itself.

Moreover, since the Rel-18 NES DTX/DRX pattern had been excluded in last RAN plenary, there is no way to combine the multiple NES DTX/DRX pattern (for example, in beam/footprint level) and current SMTC/SSB pattern in cell/carrier level.

Observation 2: since the Rel-18 NES DTX/DRX pattern had been excluded in last RAN plenary, there is no way to combine the multiple NES DTX/DRX pattern (for example, in beam/footprint level) and current SMTC/SSB pattern in cell/carrier level.

Therefore, there are the following candidates for the issue:

- Introduce multiple SMTC/SSB patterns for each beam/footprint (group) in a single cell with different SSB start offset;

- Remaining the current SMTC/SSB pattern defintion with some additial information to enable the UE idle/inactive mode to identity the specific SSB reception location/window to avoid unnecessary power consumption.

- Proposal 2: for senario2, i.e. Single PCI for Multi-beam footprints in one satellite cell (multiple-to-1 mapping)

, RAN2 should consider the following candidates to address the issue:

Alt 1: introduce multiple SMTC/SSB patterns for each beam/footprint (group) in a single cell with different SSB start offset;

Alt 2: Remaining the current SMTC/SSB pattern defintion with some additial information, e.g., beam/footprint/location information,  to enable the UE idle/inactive mode to identity the specific SSB reception location/window to avoid unnecessary power consumption.

- Summary

Observation 1: Since for different beam/footprint (group) in a single cell,  there will be different SSB start offset,  the current granulartiy of the current SMTC number and SSB pattern definition is possible not sufficient at least for the UE in idle/inactive mode, even for the serving cell measurement itself.

Observation 2: since the Rel-18 NES DTX/DRX pattern had been excluded in last RAN plenary, there is no way to combine the multiple NES DTX/DRX pattern (for example, in beam/footprint level) and current SMTC/SSB pattern in cell/carrier level.

Proposal 1: for senario1, i.e. different PCIs for mutiple beam footprints in one satellite cell (1-to-1 mapping), the current SMTC number and pattern definition is sufficient.

- Proposal 2: for senario2, i.e. Single PCI for Multi-beam footprints in one satellite cell (multiple-to-1 mapping)

, RAN2 should consider the following candidates to address the issue:

- Alt 1: introduce multiple SMTC/SSB patterns for each beam/footprint (group) in a single cell with different SSB start offset;

- Alt 2: Remaining the current SMTC/SSB pattern defintion with some additial information, e.g., beam/footprint/location information,  to enable the UE idle/inactive mode to identity the specific SSB reception location/window to avoid unnecessary power consumption.

- References

- Chairman's Notes RAN2#128

- RP-243300 Rel-19 WID NR NTN phase 3