3GPP TSG-RAN WG2 Meeting #129                           	                                 R2-2501038

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.7.5

Source: 	CMCC

Title:  	Discussion on the RLC AM enhancements for XR

WID/SID:	         NR_XR_Ph3-Core

Document for:	Discussion and Decision

Introduction

In the last RAN2 #128 meeting, the RLC enhancements has been discussed and the following related agreements have been approved.

- Agreements on AL-FEC (related to LS from SA2) - There is no consensus in RAN2 that AL-FEC ratio information is useful for the gNB for both RLC AM and RLC UM. - RAN2 understanding is that in case this information would be provided to the gNB, it is up to gNB how/whether to consider it, i.e. no impact on RAN2 specifications Agreements on avoiding unnecessary retransmissions - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced - Special handling to avoid PDCP control PDU discard is not needed. - A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. - The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report. Agreements on RLC timely retransmissions - Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

In this contribution, we provide our views on the RLC enhancements.

Discussion

Avoidance of Unnecessary Retransmission

In the last RAN2 meeting,  the details of combined approach have not reach a consensus in last meeting and still need to be discussed.

The UE as transmitting side could acquire the remaining time of an SDU. Since the PDCP layer is the upper layer and could derive the remaining time of an SDU, it is more natural and accurate that PDCP indicates the RLC to trigger the discard procedure based on the remaining time.  RAN2 should confirm that the TX side should stop the transmissions of an outdated RLC SDU based on the indication from upper layer.

Proposal 1: In combined approach, TX side should stop the transmissions of an outdated RLC SDU based on the indication from upper layer.

In the combined approach, it is agreed that RX side abandons the SDU based on a local timer and the timer starts when the gap is detected at RLC layer. One proposed solution is that the local timer for abandoning the obsolete SDU in RX RLC entity could be set like t-Reassembly. When new timer expires, the RLC entity discards the received segments of unsuccessfully received PDU and sends back ACK for the unsuccessfully received PDU in STATUS report, moves the lower edge of  RX window to SN marked with the new timer related state variable. This method may bring delays on sending back the STATUS report and stall the TX windows when several gaps are found in the RX sides. Another proposed solution is that the local timer for abandoning the obsolete SDU in RX was triggered for each missing RLC PDU, which may fasten to send back the STATUS report. It seems to be more accurate to sync the Tx and Rx side and avoid the window stalling. Therefore The local timer for abandoning the obsolete SDU in Rx side may consider to be set per RLC PDU.

Proposal 2: The local timer for abandoning the obsolete SDU in Rx side may consider to be set per RLC PDU.

The RLC failure is indicated from RLC to upper layer when max retransmission has been reached, which may lead Radio Link Failure (RLF) and bring the interruption. If the RLC AM entity discards an obsolete RLC SDU and the maximum number of retransmissions is reached, the indication from RLC to upper layer, which indicates max retransmission has been reached as RLC failure, shall not be triggered due to the retransmissions discarding.

Proposal 3: The indication for RLC failure from RLC to upper layer shall not be triggered due to the retransmissions discarding.

Timely RLC Retransmission

In the last meeting, it was agreed that both polling enhancements and autonomous retransmission were supported to achieve the timely RLC retransmissions. The details of solutions need to be discussed.

Autonomous Retransmission

In the autonomous retransmission mechanism, the transmitter could retransmit the PDUs without waiting for the NACKs indicated in STATUS report from receiving side. Indeed the autonomous retransmission may fasten the retransmission procedure by reducing the report waiting time. Meanwhile the autonomous retransmission could blindly retransmit the PDUs already successfully received in case that the STATUS report including the ACK of a PDU was not sent back timely, which may increase the capacity burden.

In order to reduce the impact on capacity and timely retransmit the data becoming outdated, one proposed solution is to retransmit the selected PDUs with short remaining time. In this case, the RLC PDUs or the segments of the RLC PDU could be early retransmitted based on some new triggers to reduce the retransmission delay.

Since the PDCP layer could acquire the remaining time based on the discardTimer, one solutions is to use the new indication from PDCP to indicate the remaining time is below a threshold and trigger the retransmission. Another solutions is to introduce a new timer associated with a particular RLC PDU in RLC. The new timer could start when the RLC SDU is received from PDCP or the RLC PDU is submitted to lower layer. Since other features, e.g. DSR also use the remaining time as the trigger mechanism, it is better to use dedicated threshold for each feature. The following mechanism could be considered to trigger the autonomous retransmissions.

- Option 1: A new indication from PDCP could be used to indicate the remaining time is below a threshold.

- Option 2: A new timer associated with a particular RLC PDU could be introduced in RLC to trigger the retransmission action.

- Proposal 4: The following mechanism could be considered to trigger the autonomous retransmission:

- Option 1: A new indication from PDCP could be used to indicate that the remaining time of the SDU is below a dedicated threshold to trigger the autonomous retransmission.

- Option 2: A new timer associated with a particular RLC PDU could be introduced in RLC to trigger the autonomous retransmission.

Multiple retransmissions in autonomous retransmission could also be considered. The retransmissions could be transmitted in the same carrier or different carriers with more than one MAC entities, e.g. CA configuration. However if the retransmissions are transmitted 	in different carriers, the RLC entity could be restricted to serve the smallest resource of transmission opportunity indicated by lower layer from different MAC entities, which leads the resource waste. So it is suggested that the autonomous retransmissions could be transmitted in the same carrier.

Observation 1: The retransmissions could be transmitted in the same carrier or different carriers with more than one MAC entities,e.g. CA configuration. However if the retransmissions are transmitted 	in different carriers, the RLC entity could be restricted to serve the smallest resource of transmission opportunity indicated by lower layer from different MAC entities, which leads the resource waste.

If multiple autonomous retransmissions is supported, one option is to periodically transmit the multiple retransmissons till the maximum number of retransmissions is reached or the ACK of a PDU is received. Another option is to use a new prohibit timer in the transmitting side of an AM RLC entity for autonomours retransmissions. A prohibit timer could start after the autonomous retransmission to avoid excessive retransmissions. For example, when the NACK of a PDU is received after an autonomous retransmission, a new retransmission could be sent after the expiry of the new prohibit timer, which could avoid excessive retransmissions.

Proposal 5: Multiple autonomous retransmission transmitted in the same carrier could also be considered.

Proposal 6: If Multiple autonomous retransmission is supported, periodically transmit the multiple autonomous retransmissons or use a new prohibit timer after the autonomous retransmission could be considered to avoid excessive retransmission.

Polling Enhancement

In legacy, when the total number of PDUs or the total data volume of PDUs is accumulated larger than the configured value pollPDU or  pollByte, the polling could be included in AMD PDU to request the status report from receiving side. In order to reduce the polling delay for the RLC SDUs with small remaining time value, the remaining time could be considered in the polling trigger mechanism. Indeed, when the remaining time is below a threshold for a PDU, which means this PDU turns to be a delay-critical data, the polling may be triggered.

Several solutions for polling enhancement was proposed in previous meetings.

One solution is to include the polling bit in the PDU containing delay-critical data. But this solution may result lots of pollings or STATUS reports if consecutive delay-critical data come when the congestion occurs.

Another solution is to configure a dedicated value pollPDU or pollByte for delay-critical data in the RLC entity. Two options listed below could be considered to apply a new dedicated value pollPDU or pollByte for delay-critical data.

- Option 1: If the any PDU containing the delay-critical data is found, and when the total number of PDUs or the total data volume of PDUs is accumulated larger than a new dedicated value pollPDU or  pollByte, the polling could be included in the PDU to request the status report from receiving side.

- Option 2: When the total number of PDUs or the total data volume of PDUs containing the delay-critical data is accumulated larger than a new dedicated value pollPDU or  pollByte, the polling could be included in the PDU to request the status report from receiving side.

Compared to option1, this option 2 could fasten the polling trigger procedure by additional polling for delay-critical data and not impact the legacy polling procedure.

Proposal 7: The remaining time could be considered to enhance the polling trigger mechanism.

- Proposal 8: The following mechanism could be considered to trigger the polling enhancement:

- Option 1:  If the any PDU containing the delay-critical data is found, and when the total number of PDUs or the total data volume of PDUs is accumulated larger than a new dedicated value pollPDU or  pollByte, the polling could be included in the PDU to request the status report from receiving side.

- Option 2: When the total number of PDUs or the total data volume of PDUs containing the delay-critical data is accumulated larger than a new dedicated value pollPDU or  pollByte, the polling could be included in the PDU to request the status report from receiving side. The legacy polling procedure is not impacted.

The timer t-PollRetransmit is used by the transmitting side of an AM RLC entity in order to retransmit a poll. The typical end-to-end latency requirement for the Mobile Metaverse for immersive gaming and live shows is in the range of  [5~20] ms. Considering the short PDB, the value below 5ms for the timer t-PollRetransmit should be added to reduce the delay of retransmitting the polling. The additional values could be introduced for short PBD of XR traffic, including [1,2,3,4]in milliseconds value, which is simple way to implement the timely retransmission.

T-PollRetransmit ::= ENUMERATED { ms1,ms2,ms3,ms4,ms5, ms10, ms15, ms20, ms25, ms30, ms35, ms40, ms45, ms50, ms55, ms60, ms65, ms70, ms75, ms80, ms85, ms90, ms95, ms100, ms105, ms110, ms115, ms120, ms125, ms130, ms135, ms140, ms145, ms150, ms155, ms160, ms165, ms170, ms175, ms180, ms185, ms190, ms195, ms200, ms205, ms210, ms215, ms220, ms225, ms230, ms235, ms240, ms245, ms250, ms300, ms350, ms400, ms450, ms500, ms800, ms1000, ms2000, ms4000, ms1-v1610, ms2-v1610, ms3-v1610, ms4-v1610, spare1}

Observation 2: The typical end-to-end latency requirement for the Mobile Metaverse for immersive gaming and live shows is in the range of  [5~20] ms. Considering the short PDB, the value below 5ms for the timer t-PollRetransmit should be added to reduce the delay of retransmitting the polling.

Proposal 9: The additional values of the timer t-PollRetransmit could be introduced for short PBD of XR traffic, including [1,2,3,4] in milliseconds value.

Conclusion

In this contribution, we discussed the RLC enhancements related issues. We propose:

Proposal 1: In combined approach, TX side should stop the transmissions of an outdated RLC SDU based on the indication from upper layer.

Proposal 2: The local timer for abandoning the obsolete SDU in Rx side may consider to be set per RLC PDU.

Proposal 3: The indication for RLC failure from RLC to upper layer shall not be triggered due to the retransmissions discarding.

- Proposal 4: The following mechanism could be considered to trigger the autonomous retransmission:

- Option 1: A new indication from PDCP could be used to indicate that the remaining time of the SDU is below a dedicated threshold to trigger the autonomous retransmission.

- Option 2: A new timer associated with a particular RLC PDU could be introduced in RLC to trigger the autonomous retransmission.

Observation 1: The retransmissions could be transmitted in the same carrier or different carriers with more than one MAC entities,e.g. CA configuration. However if the retransmissions are transmitted 	in different carriers, the RLC entity could be restricted to serve the smallest resource of transmission opportunity indicated by lower layer from different MAC entities, which leads the resource waste.

Proposal 5: Multiple autonomous retransmission transmitted in the same carrier could also be considered.

Proposal 6: If Multiple autonomous retransmission is supported, periodically transmit the multiple autonomous retransmissons or use a new prohibit timer after the autonomous retransmission could be considered to avoid excessive retransmission.

Proposal 7: The remaining time could be considered to enhance the polling trigger mechanism.

- Proposal 8: The following mechanism could be considered to trigger the polling enhancement:

- Option 1:  If the any PDU containing the delay-critical data is found, and when the total number of PDUs or the total data volume of PDUs is accumulated larger than a new dedicated value pollPDU or  pollByte, the polling could be included in the PDU to request the status report from receiving side.

- Option 2: When the total number of PDUs or the total data volume of PDUs containing the delay-critical data is accumulated larger than a new dedicated value pollPDU or  pollByte, the polling could be included in the PDU to request the status report from receiving side. The legacy polling procedure is not impacted.

Observation 2: The typical end-to-end latency requirement for the Mobile Metaverse for immersive gaming and live shows is in the range of  [5~20] ms. Considering the short PDB, the value below 5ms for the timer t-PollRetransmit should be added to reduce the delay of retransmitting the polling.

Proposal 9: The additional values of the timer t-PollRetransmit could be introduced for short PBD of XR traffic, including [1,2,3,4] in milliseconds value.

Reference

- 3GPP TS 38.323 V18.3.0 (2024-09), Packet Data Convergence Protocol (PDCP) specification(Release 18)

- 3GPP TR 23.700-70 V19.0.0 (2024-09), Study on architecture enhancement for Extended Reality and Media service (XRM);Phase 2(Release 19)

- 3GPP TS 38.322 V18.1.0 (2024-06), Radio Link Control (RLC) protocol specification (Release 18).