3GPP TSG-RAN WG2 Meeting #129	R2-2501040

Athens, GR, Feb 17th – 21th, 2025

Agenda Item:	8.3.2.1

Source:	CMCC

Title:	Potential specification impact for RRM measurement prediction

WID/SID:				FS_NR_AIML_Mob

Document for:	Discussion

# 1 Introduction

In the previous RAN2 meeting, there were discussion on Simulation assumptions and evaluation methodology for RRM measurement prediction. The agreements about simulation assumptions for RRM measurement prediction were achieved [1][2][3] with the following agreed evaluation scenarios to start the relevant simulations.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In last RAN2#127bis [4] and RAN2#128 [5], the following agreements/observations on intra-frequency temporal domain prediction (Case 2/4) and inter-frequency prediction (Case 3) were achieved as below.

RAN2#127bis:

Agreements on intra-frequency

- For intra-frequency temporal domain, higher UE speeds result in larger prediction errors

- Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold, further increases do not yield significant benefits. Conversely, for PW, longer durations correlate with decreased prediction accuracy.  RAN2 will not define the actual threshold and fast fading assumption.

- Majority of companies observe that among sub cases 1, 2, and 3, at least with shorter prediction window sub case 2 demonstrates the highest prediction accuracy

- Companies can provide multiple real time RSRP value(s) and/or average RSRP value over the entire window and should indicate in their simulation results what they have used.  The companies should at least provide the results of only one value it should be the last value at the end of the PW.   We will add two columns in the spreadsheet to capture the last value and the average value.

Agreements on inter-frequency

- Companies need to report whether earlier predicted results are also used as inputs for future RRM prediction.

- Companies should report with their simulation the correlation coefficient

- Higher-to-lower and lower-to-higher frequency prediction is comparable

- For co-located scenario,  the UE speed in the inter-frequency case has minor impact on prediction accuracy

RAN2#128:

Agreements/Observations to be captured in TR

- The prediction accuracy for intra-frequency temporal domain case B reduces as MRRT increases.

- For temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance.  Companies may report the adopted skipping pattern when providing simulation results.   Companies are not required to run new simulations but can clarify in the spreadsheet.    We can capture in the TR how the skipping patter affects the performance.

- For the inter-frequency prediction, the evaluation results show that the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy.   FFS on observations on low correlations.

- For inter-frequency, cluster as input (i.e. measurements from different cells as inputs )can improve the prediction accuracy than single cell as input.    For temporal domain, the gains are unclear.   RAN2 will focus on frequency domain for cluster based approach.

- In cluster approach the model takes measurements from more than one cell as inputs.

Agreements/observations

- Companies can compare results with non-AI approaches.   Temporal domain, sample and hold, and frequency domain, pathloss offset.   Companies can consider other simple models (e.g. ARIMA).

- For FR1 intra-frequency temporal domain case B, when PW is short, the performance between AI and sample-and-hold is not significant. However, when PW becomes larger, AI outperforms sample-and-hold.

- In frequency domain prediction cases (2GHz and 4GHz), cell-based approaches achieve limited gain compared to pathloss offset without the help of neighbor cell measurement. The cluster-based shows better performance compared to pathloss offset.

- For intra-frequency temporal domain case A, AI can provide gains (in terms of L3 cell RSRP difference) compared to sample and hold. The gain improves with UE speed.

In this contribution, we provide our initial consideration on potential specification impact for RRM measurement prediction.

# 2 Discussion

Considering the evaluation progress of RRM measurement prediction, we think we could consider to start the study on potential specification impact of RRM measurement prediction.

Based on the simulation results from companies during the previous RAN2 meetings, we can observe that both intra-frequency temporal domain Case A and Case B for RRM measurement prediction based on cell level results could achieve stable prediction performance. Therefore, the specification impact could be further considered to support intra-frequency temporal domain of RRM measurement prediction for measurement overhead reduction or handover performance enhancement.

Intra-frequency temporal domain (i.e., Case A and Case B) for RRM measurement prediction could be supported by UE-side model or NW-side model.

- For UE-side model, UE reports the prediction results (AI/ML model output) and the time information of prediction generation to network for Handover Decision;

- For NW-side model, UE reports the RRM measurement results and UE assistant information (if needed) to network as AI/ML model input.

For intra-frequency temporal domain Case A, it focuses on the enhancement for handover performance and no reduction in measurement overhead in this case. An example of Case A is shown in Figure 1.

Figure 1: Temporal domain Case A example [6]

For intra-frequency temporal domain Case B, it focuses on measurement overhead reduction. Two examples of Case B are shown in Figure 2 and Figure 3.

Figure 2: Temporal domain Case B example 1 [7]

Figure 3: Temporal domain Case B example 2 [7]

AI/ML model input could include:

- Actual RRM measurement results, e.g., RSRP, RSRQ and SINR;

- UE assistant information, e.g., UE speed, UE location information;

AI/ML model output could include:

- Cell level RRM measurement prediction results based on cell level results or beam level results, e.g., RSRP, RSRQ and SINR;

Proposal 1: Intra-frequency temporal domain for RRM measurement prediction could be supported by UE-side model or NW-side model.

- For UE-side model, UE reports the prediction results (AI/ML model output) and the time information of prediction generation to network for Handover Decision;

- For NW-side model, UE reports the RRM measurement results and UE assistant information (if needed) to network as AI/ML model input.

Proposal 2: The potential specification impact could be considered for intra-frequency temporal domain for RRM measurement prediction based on cell level results.

AI/ML model input could include:

- Actual RRM measurement results, e.g., RSRP, RSRQ and SINR;

- UE assistant information, e.g., UE speed, UE location information;

AI/ML model output could include:

- Cell level RRM measurement prediction results based on cell level results or beam level results, e.g., RSRP, RSRQ and SINR.

# 3 Conclusion

In this paper, we provide our initial consideration on potential specification impact for RRM measurement prediction. We have the following proposals:

Potential specification impact

Proposal 1: Intra-frequency temporal domain for RRM measurement prediction could be supported by UE-side model or NW-side model.

- For UE-side model, UE reports the prediction results (AI/ML model output) and the time information of prediction generation to network for Handover Decision;

- For NW-side model, UE reports the RRM measurement results and UE assistant information (if needed) to network as AI/ML model input.

Proposal 2: The potential specification impact could be considered for intra-frequency temporal domain for RRM measurement prediction based on cell level results.

AI/ML model input could include:

- Actual RRM measurement results, e.g., RSRP, RSRQ and SINR;

- UE assistant information, e.g., UE speed, UE location information;

AI/ML model output could include:

- Cell level RRM measurement prediction results based on cell level results or beam level results, e.g., RSRP, RSRQ and SINR.

# Reference

- R2_125bis_ChairNotes_24-04-19_final, Apr 2024

- R2_126_ChairNotes_24-05-24_final, May 2024

- R2_127_ChairNotes_24-08-23_17-00_eom, Aug 2024

- R2_127bis_ChairNotes_24-10-18_15-45_eom_clean, Oct 2024

- R2_128_ChairNotes_24-11-22_17-00_eom, Nov 2024

- R2-2407781, Summary of offline [AT127][026][AI Mob] Simulation assumptions, OPPO

- R2-2407885, Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions-phase 2, OPPO