3GPP TSG-RAN WG2 Meeting #129       	                      R2-2501048

Athens, GR, 17th-21st February 2025

Agenda item:	8.1.2.2

Source:	CMCC

Title:	Discussion on LCM for UE-sided model for BM

WID/SID:	NR_AIML_air-Core

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 discussed LCM for UE-sided model for BM and achieved the following agreements.

Agreements

- When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

- Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

- Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

- For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

- For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

- The network can provide the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

RAN1 sent reply LS [2] on applicable functionality reporting for beam management UE-sided model in last meeting.

This contribution will discuss the remaining issues on UE-sided model LCM for beam management use case.

- Discussion

- 2.1 Applicable functionality reporting

- In last meeting, RAN2 agreed that when a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network. FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability. We understand that a smart network can be aware of the non-applicable functionality based on latest applicable functionality reporting and last reporting. There is no need to report the non-applicable functionality explicitly.

- Proposal 1: An explicit non-applicable functionality reporting to the network is not needed, since it can be indicated implicitly based on latest and last applicable functionality reporting.

- In RAN1 reply LS [2], RAN1 discussed two options to decide applicable functionality, A) CSI-ReportConifg for inference configuration or B) inference related parameters which is not for inference. If the UE has already been configured with CSI-ReportConfig in Step 3, i.e. option A) is used, Step 5 may be optional, which aligns with RAN2 agreements before.

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction

- However, RAN1 has no consensus on the following issues, such as which option used in Step 3, the granularity of functionality, the content and granularity of NW-side additional condition, etc. At this stage, we can wait for RAN1’s further progress on applicable functionality.

- Proposal 2: The discussion on the issues, such as the granularity of functionality, the content and granularity of NW-side additional condition, etc. are postponed until RAN1 has further progress.

- 2.2 Data collection

- In last meeting, there was a FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration.

- In our understanding, although RAN2 has no agreements on the details of data collection configuration, it is provided to the UE by RRCReconfiguration message since the UE stays in RRC_CONNECTED state. As specified in TS 38.331[3], if the UE cannot comply with RRCReconfiguration, the UE shall continue using the configuration used prior to the reception of RRCReconfiguration message or perform the actions upon going to RRC_IDLE or initiate the connection re-establishment procedure.

- 5.3.5.8 Reconfiguration Failure - 5.3.5.8.2 Inability to comply with RRCReconfiguration The UE shall: - <omitted> 1> else if RRCReconfiguration is received via NR (i.e., NR standalone, NE-DC, or NR-DC): - <omitted> 2> else if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over the SRB1 or if the upper layers indicate that the nas-Container is invalid: NOTE 0a: The compliance also covers the SCG configuration carried within octet strings e.g. field mrdc SecondaryCellGroupConfig. I.e. the failure behaviour defined also applies in case the UE cannot comply with the embedded SCG configuration or with the combination of (parts of) the MCG and SCG configurations. NOTE 0b: The compliance also covers the V2X sidelink configuration carried within an octet string, e.g. field sl ConfigDedicatedEUTRA. I.e. the failure behaviour defined also applies in case the UE cannot comply with the embedded V2X sidelink configuration. 3> if the RRCReconfiguration message was received as part of ConditionalReconfiguration; or, 3> if the RRCReconfiguration message was received as part of ltm-Config: 4> continue using the configuration used prior to when the inability to comply with the RRCReconfiguration message was detected; 3> else: 4> continue using the configuration used prior to the reception of RRCReconfiguration message; 3> if AS security has not been activated: 4> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other' 3> else if AS security has been activated but SRB2 and at least one DRB or multicast MRB or, for IAB and NCR, SRB2, have not been setup: 4> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'; 3> else: 4> initiate the connection re-establishment procedure as specified in 5.3.7, upon which the reconfiguration procedure ends; - <omitted>

- For data collection for model training, we think it is a separate procedure from legacy operation. If the UE cannot perform data collection based on received configuration, there is no need to initiate the connection re-establishment procedure or perform the actions upon going to RRC_IDLE, and the legacy operation should be performed as usual. However, we think the network needs to know when the UE cannot perform data collection. Therefore, an indication is needed from UE to network to indicate that the UE cannot perform data collection based on received configuration. Furthermore, the indication can be included in RRCReconfigurationComplete or UEAssistanceInformation message.

- Proposal 3: An indication is introduced in RRCReconfigurationComplete or UEAssistanceInformation message when UE can’t perform data collection based on received configuration.

- 2.3 Functionality identification

- During R18 study phase, RAN1 agreed to take the UE capability reporting as starting point for functionality identification and the UE can report the applicable functionality after functionality identification, which were captured in TR 38.843 [3].

- 4.2.1 LCM Flavours The LCM procedure is studied for the case that an AI/ML model has a model ID with associated information and/or for the case that a given functionality is provided by some AI/ML operations. Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion. From RAN1 perspective, an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation. When distinction is necessary for discussion purposes, companies may use the term a logical AI/ML model to refer to a model that is identified and assigned a model ID, and physical AI/ML model(s) to refer to an actual implementation of such a model. For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Legacy 3GPP framework of feature is taken as a starting point. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. <omitted> After functionality identification, necessity, mechanisms, for UE to report updates on applicable functionality(es) among functionality(es) are studied, where the applicable functionalities may be a subset of all functionalities. Applicable functionalities can be reported by the UE. <omitted>

- In R19, RAN2 discussed the basic procedure of applicable functionality reporting which includes the UE capability reporting containing the supported functionalities in UE side. In our understanding, functionality identification is achieved in step 1 and step 2. Therefore, we think there is no need to introduce an additional definition and procedure for functionality identification.

- Proposal 4: There is no need to introduce the definition and procedure for functionality identification since it is achieved in UE capability reporting (i.e. step1/2 of applicable functionality reporting).

# 3	Conclusion

Here are the observations and proposals for LCM for UE-sided model for beam management.

Applicable functionality reporting

- Proposal 1: An explicit non-applicable functionality reporting to the network is not needed, since it can be indicated implicitly based on latest and last applicable functionality reporting.

- Proposal 2: The discussion on the issues, such as the granularity of functionality, the content and granularity of NW-side additional condition, etc. are postponed until RAN1 has further progress.

Data collection

- Proposal 3: An indication is introduced in RRCReconfigurationComplete or UEAssistanceInformation message when UE can’t perform data collection based on received configuration.

Functionality identification

- Proposal 4: There is no need to introduce the definition and procedure for functionality identification since it is achieved in UE capability reporting (i.e. step1/2 of applicable functionality reporting).

# 4	Reference

- Draft_R2_128_Meeting_Report_v1

- R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface