3GPP TSG RAN WG2 #129			R2-2501066

Athens, GR, February 17th – 21st, 2025

Source: 	Fujitsu

Agena item:	8.8.5

Title:	Discussion on regenerative payload

Document for:	Discussion and decision

Background

In this contribution, we would like to discuss the remaining issue on the Regenerative payload.

Discussion

Time-based measurement

In Rel-17 NTN, only transparent payload case was considered. In such case, the feeder link switches may happen regardless of service link quality. To address this issue, the t-Service which indicates the time information on when the cell is going to stop serving the NTN UE due to feeder link switches and service link switches, and the time-based measurement were introduced to initiate the measurement on NTN UE before t-Service, allowing for IDLE mode NTN UE to have enough time to reselect the best cell.

Observation 1: Time-based measurement can be initiated by the NTN UE based on the t-Service.

In Rel-19 NTN, satellites with regenerative payload are considered. Different from the transparent payload, the time information due to feeder link switches may not be indicated by t-Service, because feeder link switches may not change the serving cell. In other words, the timing of the cell going to stop serving the NTN UE may not be related to feeder link switches.

Observation 2: In regenerative payload case, the timing of the cell going to stop serving the NTN UE may not be related to feeder link switches.

According to the above observations, the time-based measurement is based on the time the cell will stop serving the NTN UE, so the time-based measurement should not always apply to feeder link switches in regenerative payload.

Proposal 1: The time-based measurement should not always apply to feeder link switches case in regenerative payload.

At this point, looking into the t-Service that triggers the time-based measurement, the current description in stage 3 is defined as follows [1]:

t-Service

Indicates the time information on when a cell provided via NTN is going to stop serving the area it is currently covering. This field applies for both service link switches in NTN quasi-Earth fixed cell and feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell.

According to the description, t-Service indicates the time information on when a cell provided via NTN is going to stop serving the area it is currently covering and applies to both service link switches and feeder link switches. Meanwhile, in regenerative payload, the timing of the cell going to stop serving the NTN UE may not include the time information due to feeder link switches. In other words, the current description in stage 3 may not consider regenerative payload.

Observation 3: The current stage 3 description does not consider regenerative payload.

Even in case of regenerative payload, service link switches will occur in earth-moving system, so the time-based measurement is useful for the NTN UE to indicate appropriate time for measurements.

Therefore, to support the time-based measurement in regenerative payload, RAN2 clarifies the t-Service does not always apply to feeder link switches case.

Proposal 2: In order to support time-based measurement in regenerative payload, RAN2 clarifies the t-Service does not always apply to feeder link switches case.

To make t-Service to support regenerative payload, the current definition of the t-Service needs to be modified. For example, as the feeder link switches without the serving cell change may or may not happen, the field description should be updated to delete “both” and add “/or”.

t-Service

Indicates the time information on when a cell provided via NTN is going to stop serving the area it is currently covering. This field applies for both service link switches in NTN quasi-Earth fixed cell and/or feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell.

Proposal 3: RAN2 to update the field description of the t-Service as follows:

- This field applies for both service link switches in NTN quasi-Earth fixed cell and/or feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell.

Conclusion

In this contribution, the following proposals and observations were made:

Observation 1: Time-based measurement can be initiated by the NTN UE based on the t-Service.

Observation 2: In regenerative payload case, the timing of the cell going to stop serving the NTN UE may not be related to feeder link switches.

Proposal 1: The time-based measurement should not always apply to feeder link switches case in regenerative payload.

Observation 3: The current stage 3 description does not consider regenerative payload.

Proposal 2: In order to support time-based measurement in regenerative payload, RAN2 clarifies the t-Service does not always apply to feeder link switches case.

Proposal 3: RAN2 to update the field description of the t-Service as follows:

- This field applies for both service link switches in NTN quasi-Earth fixed cell and/or feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell.

References

- 3GPP TS38.331 V18.4.0, Radio Resource Control (RRC) protocol specification (Release 18)