3GPP TSG-RAN WG2 Meeting #129                                                                             R2-2501067

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.2.2

Source:	China Telecom

Title:	Further discussions on Ambient IoT Paging

Document for:	Discussion

- Introduction

During the RAN#106 meeting, a new work item (RP-243326) was approved for Ambient IoT (Internet of Things) in NR. The scope includes specifying the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedures to enable DO-DTT and DT data transmission. RAN2 aims to design a paging message format that accommodates multiple identifiers within a single paging message to ensure forward compatibility. A-IoT paging involves repeated paging attempts for the same service. According to the objectives of this new WID, RAN2 need to support two options: a paging message either contains a single identifier or none. Temporary identifiers are not supported in paging messages unless explicitly required by SA WGs. During the most recent RAN2#128 meeting, the following agreements were made.

Agreements on paging

- An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size.

- Capture the scenario that different readers may page the same device for same service request.  We will discuss this in WI phase (if in scope of WI) and consider progress from all the WGs

- Whether we include further information in paging message to indicate service type/command type or to indicate that further subsequent messages are coming can be discussed in WI phase once we understand more the paging design and AS ID.

Based on the above progress, this paper mainly analyses the details of subsequent A-IoT paging and paging contents.

- Discussion

2.1 Multi-Devices CFRA Paging Scenario

To initiate the multi-device CFRA procedure, the A-IoT paging message need to indicate the mapping between each device ID and the corresponding access occasion index, ensuring that each device can accurately determine its designated access occasion. However, explicitly specifying a one-to-one mapping in the paging message introduces significant overhead. An alternative approach involves using a group or mask ID. When multiple A-IoT devices are activated for the CFRA procedure through group or mask IDs, their respective access timing indices are determined by their positions relative to the assigned group or mask ID. However, if the target devices are assigned to distinct groups, individual device IDs must be explicitly included in the A-IoT paging message. A third approach involves using a PF/PO-like mechanism to allocate access timing to each device ID. In this method, an initial access timing is assigned to the target A-IoT device based on a computation of its device ID, which is modelled as a parameter defining the access timing size within a cycle. This strategy confines conflicts to A-IoT devices sharing the same collision outcome. Each device is required to determine its ranking within the group of devices sharing this outcome and adjust its computation accordingly to identify its accurate access timing. Using a PF/PO-like access index assignment method, an A-IoT device only needs to include in its paging message a list of device IDs sharing the same collision outcome and the number of accesses per cycle.

- Proposal 1: To enable A-IoT devices to identify their appropriate access occasions in the A-IoT CFRA procedure, three potential options can be considered in the A-IoT paging message:

- One-to-one mapping of the device ID to the access occasion

- PF/PO-like access index allocation approach

- Mask ID

2.2 Paging Round

AIoT devices may operate in the following states and can transition between states depending on their energy harvesting status:

- ON state: transmission, reception, and a clock running

- OFF state: energy harvesting only

- SLEEP sate: energy harvesting, monitoring, maintaining memory, and a rough clock running

The issue on whether Device 1 can support sleep state in addition to the ON and OFF state was still discussed in RAN1. However, it is evident that Device 2b can support the ON, OFF, and SLEEP states.

During RAN1#118 meeting, RAN1 reached the following conclusions on energy harvesting/device unavailability:

Agreement For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection.

Regarding Direction 2, the reader can transmit information in the R2D message, allowing the device to select its availability for energy harvesting. When the device is not required to respond to any access rounds during that paging cycle, it may switch to the OFF or SLEEP state for energy harvesting until the end of that paging round. Limited by capacity, it may be difficult for the device to make predictions about the duration of a paging round based on other information. Therefore, the reader should explicitly include time indication information in the A-IoT paging message, which indicates the duration of this paging round to help the device determine their respective available/unavailable times.

- Proposal 2: To include time indication information in the A-IoT paging message which indicates the duration of each access round in the device's current paging round, so that the device can easily determine their respective available/unavailable times.

2.3 Multiple/Subsequent A-IoT Paging

In general, the current agreements presume that paging necessitates a response in all cases. This assumption is particularly valid for paging scenarios where the reader must conduct an inventory of one or more devices, retrieve data from devices, or establish communication for data exchange. However, a single round of paging may not always reach all target devices within the coverage area. This may occur because some devices are insufficiently charged, preventing them from responding to paging requests. Alternatively, a high density of devices may lead to contention for uplink resources when multiple devices attempt to respond simultaneously. Consequently, for each CN service request for paging, the reader can initiate multiple paging messages.

A paging round refers to the duration of a sequence of paging messages triggered by the same CN request. Each paging message may constitute a paging round. To minimize complexity, subsequent paging messages within the same paging round are identical to the initial trigger message. The method for initiating multiple or subsequent A-IoT paging still requires further discussion. Three potential implementation options are as follows:

- Upon receiving the CN request, the reader sets the timing for subsequent A-IoT paging and specifies the number of paging repetitions.

- The CN service request includes a periodic A-IoT paging configuration (e.g., paging round duration and the number of repetitions). Based on this configuration, the reader schedules subsequent A-IoT paging and defines the number of repetitions.

- The CN triggers each instance of multiple or subsequent A-IoT paging independently, and the reader executes the transmission upon receiving the paging trigger from the upstream node.

Option 1 may not be feasible, as the reader typically lacks information regarding devices within the coverage area. Compared to Option 3, Option 2 may reduce CN requests and associated overhead, as it allows the reader to perform A-IoT paging periodically. However, according to conclusions from SA2, reader-triggered periodic paging requires the CN to send a termination request to stop paging repetitions, and any reconfiguration necessitates an additional CN request. Therefore, to simplify the A-IoT protocol, it is advisable for the CN to trigger A-IoT paging on demand instead of implementing periodic paging.

- Proposal 3: It is proposed that the CN initiate A-IoT paging on demand rather than configuring periodic paging.

2.4 Contents of A-IoT Paging Message

RAN2 has agreed on the following cases for A-IoT paging messages:

- A message containing the ID of a single A-IoT device.

- A message containing a group ID that maps to multiple A-IoT devices.

- A message broadcasted to all devices capable of receiving the A-IoT message.

- A message containing multiple A-IoT device IDs, although its necessity requires confirmation through SA2 discussions.

RAN2 did not reach a consensus on the need for multiple IDs and recommended verifying its necessity through SA2 discussions. However, no relevant discussions or agreements have taken place in recent SA2 meetings. The use of multiple IDs is relevant from both the CN/AF and A-IoT air interface perspectives. From the CN/AF perspective, it may require conducting an inventory or performing read/write operations on a small subset of devices that do not belong to any configured group. From the perspective of A-IoT air interface, after multiple paging rounds, the reader may need to inventory remaining unresponsive devices with non-contiguous device IDs.

For instance, each paging message may target the same batch of devices, but certain devices may not have the opportunity to respond during a specific paging round. If multiple device IDs with non-contiguous numbers are included in the service request message and subsequently incorporated into A-IoT air interface messages, retransmitting identical paging messages with the same device ID list would be redundant and inefficient. Considering that some devices may have successfully responded in previous rounds of paging, consistently including them in subsequent paging messages is unnecessary. Instead, all successfully paged devices should be excluded from subsequent paging messages. Depending on TB size, it may not always be feasible to include multiple device IDs. However, given the above requirements, this issue needs further discussion during the WI phase.

Proposal 4: It is proposed to support paging messages containing multiple A-IoT device IDs. These multiple IDs can be used to indicate devices with non-contiguous IDs.

At RAN2 #126, it was established that A-IoT paging messages can be directed toward a single device, a group of devices, or all devices. Different paging functions may be defined for these cases. At least two-step and three-step random access procedures should be supported for "inventory only" and "inventory and command" use cases, as discussed in RAN2. Since A-IoT access procedures are triggered by the reader, it is responsible for allocating access resources and selecting the appropriate random-access type, depending on service requirements.

If the reader intends to send commands to a small number of A-IoT devices, it may opt for a contention-free access method. If a simple acknowledgment is required after executing a command, the reader can select a two-step A-IoT access. For cases involving large-scale inventorying with limited resources, the reader may instruct A-IoT devices to perform three-step or four-step A-IoT access. Consequently, the reader should clearly define the type of A-IoT access within the paging message.

RAN2 #127 Agreements

5   For all use cases, device determines the random access type from the paging message.   FFS whether it is explicitly or implicitly.   FFS whether we will down selection for all RA types (2step, 3step, CFRA) or try to unify the design for 2step, 3step

In RAN2#127, it was agreed that, for all use cases, the device determines the random access type based on the paging message. For implicit indication, CBRA and CFRA may be distinguished by whether the reader allocates dedicated or shared resources. However, for CBRA, implicitly indicating the RACH type through resource allocation is inaccurate because the response resources for a 2-step RACH may also be small. For instance, factors such as target coverage, data rate requirements, energy consumption, or device availability can influence the TB size. Given the potentially small TB size, at least for 2-step and 3-step CBRA, the A-IoT paging message should explicitly indicate the RACH type.

Proposal 5: At least for CBRA, the A-IoT paging message should indicate the access type (2-step or 3-step CBRA).

According to SA2, an A-IoT service may be transferred to multiple A-IoT readers using the same higher-layer session ID. For example, an inventory operation targeting the same set of devices may be passed to multiple readers within a localized area. From the device perspective, it is unnecessary to repeatedly respond to all paging procedures triggered by the same service. Additionally, storing the session ID and tracking the response status on the device helps to prevent duplicate responses and optimize power consumption. Furthermore, since access failures may still occur, the reader may trigger multiple rounds of paging for the same service via the air interface. Consequently, paged devices must recognize whether they have already been successfully paged. Specifically, if a device receives a paging request identified by the same session ID as one it previously responded to, it can determine that the current paging attempt is a subsequent round of the same request and, therefore, does not need to respond again. Thus, introducing a lower-layer session ID can help mitigate unnecessary power consumption from the device's perspective. By including a session ID that indicates the paging request originates from a specific service, the device can selectively respond.

Regarding paging causes, the paging causes "Inventory" and "Command" may be incorporated. Commands refer to instructions (e.g., Read, Write, Disable) issued by an AF to an A-IoT device, which are protected end-to-end between the AF and the A-IoT device. Consequently, the AMF may include one or more command details in the paging message. Additionally, command information, formatted as a string, can be incorporated into the paging message. In conclusion, the following information can be included in the A-IoT paging message:

- UE paging ID (an ID, a group ID or multiple ID)

- Paging cause (inventory or command)

- Session ID

- NAS data: e.g., Command

- Proposal 6: The following information can be included in A-IoT paging message:

- UE paging ID (an ID, a group ID or multiple ID)

- Paging cause (inventory or command)

- Session ID

- NAS data: e.g., Command

- Conclusions

Based on the discussion in the previous sections, we propose the following:

- Proposal 1: To enable A-IoT devices to identify their appropriate access occasions in the A-IoT CFRA procedure, three potential options can be considered in the A-IoT paging message:

- One-to-one mapping of the device ID to the access occasion

- PF/PO-like access index allocation approach

- Mask ID

- Proposal 2: To include time indication information in the A-IoT paging message which indicates the duration of each access round in the device's current paging round, so that the device can easily determine their respective available/unavailable times.

- Proposal 3: It is proposed that the CN initiate A-IoT paging on demand rather than configuring periodic paging.

Proposal 4: It is proposed to support paging messages containing multiple A-IoT device IDs. These multiple IDs can be used to indicate devices with non-contiguous IDs.

Proposal 5: At least for CBRA, the A-IoT paging message should indicate the access type (2-step or 3-step CBRA).

- Proposal 6: The following information can be included in A-IoT paging message:

- UE paging ID (an ID, a group ID or multiple ID)

- Paging cause (inventory or command)

- Session ID

- NAS data: e.g., Command

- References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- R2-25xxxxx, Draft Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA

- TR 23.700-13	Study on Architecture support of Ambient power-enabled Internet of Things