3GPP TSG-RAN WG2 Meeting #129 	R2-2501068

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.4.3

Source:	China Telecom

Title:	Discussion on RRM measurement relaxation and offloading for RRC_IDLE/INACTIVE

Document for:	Discussion

- Introduction

In the RAN #102 meeting, a new work item on Low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved. According to the objective of WID, the following agreements and open issues were achieved in RAN2#128 meeting:

The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements

FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

This paper aims to further discuss the criteria for RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE modes.

- Discussion

RAN4 has only agreed on case #1 and case #3 so far when serving cell measurement by LP-WUR is being performed:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Based on progress of RAN4, RAN2 need to focus on the criteria for RRM measurement cases that have been agreed by RAN4, i.e., case#1 and case#3. As outlined in TR 38.869, using the MR for RRM measurements significantly impacts UE energy consumption and diminishes power-saving benefits. Therefore, RRM measurements via MR should either be disabled (Case #1: full offloading) or relaxed (Case #3: relaxation) when the UE monitors LP-WUS, in alignment with RAN4 agreements. This implies that when the UE begins monitoring LP-WUS, the frequency of transitions from LR to MR should be minimized to enhance energy-saving efficiency.

- Observation 1: If the UE is monitoring LP-WUS, the number of transitions from LR to MR should be minimized to maximize power-saving efficiency.

In RAN2#127 and RAN2#127bis meeting, the agreements on RRM measurement relaxation and offloading are summarized as below:

- RAN2#127 RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results. - RAN2#127bis Working assumption For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

Based on the above agreements, it is evident that RAN2 discussed the entry and exit conditions for RRM measurement full offloading (Case #1) without considering Case #3, which was concurrently agreed upon by RAN4 during the previous meeting. Currently, with both Case #1 and Case #3 supported, RAN2 must further evaluate the entry and exit conditions, particularly regarding the transition between these two cases.

If the entry conditions for LP-WUS monitoring are fulfilled, before conducting serving cell measurements on MR, the system should assess whether the criteria for performing neighboring cell measurements are met. Consequently, based on the aforementioned cases, RRM relaxation and RRM full offloading should have distinct entry conditions when configured to operate with LP-WUS. Moreover, RRM full offloading entails halting both serving cell and neighboring cell measurements on MR; therefore, the criteria required for full offloading should be more stringent than those for RRM relaxation.

RRM relaxation and RRM full offloading should have distinct entry conditions when configured to operate with LP-WUS.

In previous meetings, RAN2 agreed that for neighboring cell measurement relaxation, no additional MR-based criteria would be defined beyond the Release-16 criteria. However, it remains FFS whether to reuse the "low mobility" criterion for neighboring cell measurement relaxation in UEs capable of LP-WUS. The WID does not explicitly specify mobility requirements. However, it is presumed that WUR is not restricted to low-mobility scenarios, such as NB-IoT, and is instead applicable to standard broadband UEs. Therefore, mobility considerations must still be addressed for UEs supporting LP-WUS.

Additionally, the "UE with low mobility" criterion is linked to the likelihood of cell reselection. When this criterion is met, the UE may not require cell reselection, thereby allowing neighboring cell measurement to be relaxed from the MR side. Consequently, the legacy Release-16 "UE with low mobility" criterion can be reused for neighboring cell measurement relaxation.

To introduce the low-mobility criterion for neighboring cell measurement relaxation in MR.

- Conclusions

Based on the discussion in the previous sections, we provide the following observation and proposals:

- Observation 1: If the UE is monitoring LP-WUS, the number of transitions from LR to MR should be minimized to maximize power-saving efficiency.

Proposal 1: RRM relaxation and RRM full offloading should have distinct entry conditions when configured to operate with LP-WUS.

- Proposal 2: To introduce the low-mobility criterion for neighboring cell measurement relaxation in MR.

- References

- RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- R2-25xxxxx, Draft Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA

- TR 38.869, Study on low-power wake up signal and receiver for NR.