3GPP TSG-RAN WG2 Meeting #129	R2-2501070

Athens, Greece, Feb 17th – Feb 21st, 2025

Agenda item:	8.3.3

Source:	Qualcomm Incorporated

Title:	Measurement Event prediction and handover modelling

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# Introduction

In the last RAN2 meeting (RAN2 #128 [1]), the focus was on completing the discussion on the simulation framework, assumptions, and parameters, for Measurement Event prediction, RLF prediction, Model Generalization, and System Level Simulations. Many agreements were captured in the Chair notes for the meeting [1]. It was agreed that for the RAN2 #129 meeting, the focus would be on discussing simulation results for Measurement Event prediction, Model Generalization, and System Level Simulations, for the agreed simulation scenarios.

In this contribution, we discuss how Measurement Event prediction reports can be utilized to configure the UE to perform handover.

# Measurement Event prediction and handover modelling

The following was agreed in the previous RAN2 meeting (RAN2 #128 [1]):

- For both temporal domain RRM measurement prediction Case A and Case B, upon receiving a Measurement Event prediction report from the UE, e.g., containing a prediction that an event A3 will occur at a future time, the network initiates handover preparation.

- There are two options regarding when the network transmits the Handover Command to the UE:

Option 1: Network waits to receive a Measurement Report from the UE based on actual measurements and when the measurement event, i.e., event A3 in this case, is triggered.

Option 2: UE transmits a Measurement Event prediction report to the network when the event (i.e., event A3 in this case) entering conditions are met based on actual measurements, and the UE predicts that the event entering conditions will be met by the predicted measurements for the duration of TTT into the future.

The proposed schemes above rely on Measurement Event prediction reports from the UE to initiate early handover preparation on the network side. However, the network relies on reports from the UE based on actual measurements in some way to trigger the transmission of Handover Command to the UE.

Observation 1. It was agreed in the last RAN2 meeting (RAN2 #128 [1]) that the network may initiate early handover preparation based on receiving Measurement Event prediction reports from the UE.

Two options were agreed regarding when the network transmits the Handover Command to the UE, and these options rely on reports from the UE based on actual measurements in some way.

An issue with relying on receiving actual measurements from the UE before transmitting the Handover Command to the UE is that, in some scenarios it might be too late, i.e., the network cannot complete the handover preparation procedure and send the Handover Command in time, e.g., before the UE experiences RLF.

Observation 2. An issue with relying on receiving actual measurements from the UE before transmitting the Handover Command to the UE is that, in some scenarios it might be too late, i.e., the network cannot complete the handover preparation procedure and send the Handover Command to the UE in time.

We discuss below a method in which the network transmits a Handover Command to the UE based on reports from the UE that can, in principle, consist only of predicted measurements of the serving and candidate cells. The method can be summarized by a couple of proposals, as discussed below.

The method consists of the following:

- UE transmitting a Measurement Event prediction report to the network that can contain one or more candidate cells for handover, and for each indicated candidate cell, a future time of occurrence of the handover event and an associated probability of occurrence of the handover event.

- Network initiating a Conditional Handover (CHO) preparation procedure for a subset of the candidate cells indicated in the Measurement Event prediction report and transmitting to the UE a CHO configuration for handover upon completion of the preparation. Network can take advantage of information regarding probability and time of occurrence of handover event associated with each candidate cell in the Measurement Event prediction report, to provide the UE with a CHO configuration consisting of the most likely candidate cells for handover.

Proposal 1. The UE transmits to the source gNB a Measurement Event prediction report corresponding to a handover event, e.g., event A3, indicating one or more candidate cells. The following information is provided for each candidate cell:

- Candidate cell PCI,

- In case of Indirect Measurement Event prediction, optionally a time interval in the future [T1, T1 + delta] in which the event is predicted to occur, and an associated probability of occurrence of the event,

- In case of Direct Measurement Event prediction, optionally the time interval from the current time until a time T2 into the future, within which the event is predicted to occur, and an associated probability of occurrence of the event,

- Optionally, actual RRM measurements and predictions of RRM measurements of the candidate cell.

The Measurement Event prediction report may also contain actual RRM measurements and predictions of RRM measurements of the serving cells.

Proposal 2. Based on the Measurement Event prediction report, the source gNB initiates CHO preparation for one or more candidate cells and transmits an RRC reconfiguration message to the UE with a CHO configuration for handover, upon completion of the preparation procedure.

Observation 3. The proposed method above, summarized in Proposals 1 and 2, has the following benefits that are associated with CHO:

- The procedure provides allows a handover to any one of the configured candidate cells. This can be helpful when the target cell is incorrectly predicted.

- The UE does not need to rely on an explicit handover command from the network. This is helpful if the prediction timeline is incorrect (the cell does not appear as fast as predicted) or if the target cell never becomes better than the source cell.

- The procedure ensures reliability while avoiding an actual measurement report. Sending actual measurements would void most of the timeline gains provided by measurement predictions.

- The procedure minimizes the time CHO is configured and thus the time other cells are configured with resources to enable CHO.

- The procedure minimizes the number of candidate cells in the CHO and thus the amount of preparation needed in the network.

Whether the proposed method above works well can be studied through simulations, and we may consider doing this in the future.

# 3 Conclusions

Based on the above discussions, we recommend that RAN2 discuss the following proposals which provide the following benefits:

- Better resilience to prediction errors,

- Better resilience to accelerated or delayed realizations of predictions,

- Realizing reliability and delay enhancements compared to reverting to legacy handover procedure after the prediction, and

- Optimizing resources reserved by CHO.

Proposal 1: The UE transmits to the source gNB a Measurement Event prediction report corresponding to a handover event, e.g., event A3, indicating one or more candidate cells. The following information is provided for each candidate cell:

- Candidate cell PCI,

- In case of Indirect Measurement Event prediction, optionally a time interval in the future [T1, T1 + delta] in which the event is predicted to occur, and an associated probability of occurrence of the event,

- In case of Direct Measurement Event prediction, optionally the time interval from the current time until a time T2 into the future, within which the event is predicted to occur, and an associated probability of occurrence of the event,

- Optionally, actual RRM measurements and predictions of RRM measurements of the candidate cell.

The Measurement Event prediction report may also contain actual RRM measurements and predictions of RRM measurements of the serving cells.

Proposal 2: Based on the Measurement Event prediction report, the source gNB initiates CHO preparation for one or more candidate cells and transmits an RRC reconfiguration message to the UE with a CHO configuration for handover, upon completion of the preparation procedure.

# References

[1] Chair notes, RAN2 #128 meeting.