3GPP TSG-RAN WG2 #129	R2-2501318

Athens, Greece, 17th – 21st Feb. 2025

Agenda Item:	8.8.2

Source:	Ericsson

Title:	DL coverage enhancements

Document for:	Discussion/Decision

# 1	Introduction

The WID downlink CE objective was updated in RAN#106 [1]:

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4]

- Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth.

- Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements.

- Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint.

  - Link level enhancements are to be specified for the following channels:

    - PDCCH at least for CSS (except for Type-3) via PDCCH repetition

    - PDSCH with Msg4 via PDSCH repetition

    - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration

  - System-level enhancements are to be specified for the following:

    - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

      - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms

- Notes for this objective:

  - SSB channel enhancement other than SSB periodicity extension is not considered

    - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension

  - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation

  - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE

  - NGSO to be considered in priority: LEO Set-1 @ 600 km

And the Agenda has the following instructions for this agenda item:

Contributions should focus on RAN2 aspects of DL coverage enhancements due to extended SIB periodicity (up to 160ms), including e.g. possible SMTC impacts (while no contributions are expected on cell level / beam level DTX/DRX mechanism).

# 2	Discussion

The updated WID now states:

- Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint.

  - Link level enhancements are to be specified for the following channels:

    - PDCCH at least for CSS (except for Type-3) via PDCCH repetition

    - PDSCH with Msg4 via PDSCH repetition

    - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration

  - System-level enhancements are to be specified for the following:

    - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

      - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms

These objectives will have very limited impact on RAN2, and mainly form the changes that are to be defined by RAN1. The schemes for Satellite beam power sharing and feeder link rate limits, including link and system level enhancements, may have impact but RAN1 will need to do the work before RAN2 can include them in any further studies.

## 2.1	Extension of SMTCs

Following RAN1's agreement to support a 160 ms additional default SSB periodicity, RAN2 has explored the potential impact on SMTC configurations, considering enhancements like UE autonomous location-based SMTC selection.

In the scenario depicted in Figure 1, we assume a satellite employs a power-sharing mechanism where each beam corresponds to a cell, and not all neighbouring cells are active simultaneously. A sensible network implementation might utilize a specific frequency re-use factor, considering the limited spectrum available for NTN, alongside a beam-hopping pattern that minimizes interference within the same satellite. It is crucial to clarify whether RAN2's scope includes the configuration of different SSB periodicities for cells operating on the same frequency layer and served by the same satellite. Specifically, we need to determine if the network can configure two neighbouring cells on the same frequency, one with an SSB periodicity of 20 ms and another of 160 ms, when utilizing the beam-hopping feature.

RAN2 should clarify whether different SSB periodicities are expected in the same frequency layer.

Figure 1. Potential beam hopping pattern where each colour represents a frequency.

In the example illustrated in Figure 1, it is evident that when beam-hopping patterns are sensibly combined and all beams share the same periodicity, implementing 4 parallel SMTCs would suffice for a UE to ensure efficient operation. The specification allows configuring a maximum of 4 frequency layers with information relevant to cell reselection in SIB4. From Release 17, each frequency layer can accommodate a total of 4 SMTCs to account for different offsets caused by satellite movement and varying propagation delays. Therefore, if all beams have the same periodicity, the current number of SMTCs is deemed sufficient.

- If all beams have the same SSB periodicity, the current number of SMTCs (4) is sufficient to support the new default 160ms SSB periodicity.

No enhancements to the number of SMTCs are needed in Rel-19 to support 160ms default SSB periodicity.

Additionally, location-based SMTC selection has been under consideration, marked as a comeback issue in RAN2#128. As per TS 38.331, since Release 17, a UE is permitted to select SMTCs when it supports fewer than the 4 provided in the list, based on implementation. Therefore, a well-designed UE implementation would use information like satellite ephemeris, estimated propagation delay, and its position to select the most suitable SMTC. Consequently, we do not anticipate the need for enhancements in this area for Release 19.

smtc4list

Measurement timing configuration list for NTN deployments, see clause 5.5.2.10. The offset of each SSB-MTC4 in smtc4list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in this list, it is up to the UE to select which SMTCs to consider.

- A UE can choose its SMTCs based on implementation.

No enhancements to SMTC selection are needed in Rel-19 to support 160ms default SSB periodicity.

## 2.2	Cell level DTX

- Notes for this objective:

  - SSB channel enhancement other than SSB periodicity extension is not considered

    - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension

  - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation

  - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE

  - NGSO to be considered in priority: LEO Set-1 @ 600 km

In RAN2#128, potential enhancements to cell level DTX/DRX based on the Release 18 NES solutions were discussed to support power sharing and beam hopping. As indicated in the excerpt from [1], the downlink coverage enhancements note related to Release 18 NES was removed after RAN#106. Therefore, we consider that cell DTX enhancements based on Release 18 NES are beyond the scope of Release 19, and RAN2 should not continue discussions on this feature.

Cell DTX enhancements based on Rel-18 NES are not within the scope of Rel-19 downlink coverage enhancements.

## 2.3	Other aspects

We believe there are no other RAN2 aspects of DL coverage enhancements that are not dependent on the further evaluations in RAN1. This includes the UE power consumption aspects (briefly discussed last meeting), barring of UEs, and the RAN1 answers to the unanswered questions in the LS to RAN1. The discussions will be fast if RAN2 do not need to speculate on what RAN1 agrees on. Therefore, RAN2 needs to wait for the results in RAN1.

RAN2 wait for the RAN1 conclusions on downlink coverage enhancements.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	If all beams have the same SSB periodicity, the current number of SMTCs (4) is sufficient to support the new default 160ms SSB periodicity.

Observation 2	A UE can choose its SMTCs based on implementation.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 should clarify whether different SSB periodicities are expected in the same frequency layer.

Proposal 2	No enhancements to the number of SMTCs are needed in Rel-19 to support 160ms default SSB periodicity.

Proposal 3	No enhancements to SMTC selection are needed in Rel-19 to support 160ms default SSB periodicity.

Proposal 4	Cell DTX enhancements based on Rel-18 NES are not within the scope of Rel-19 downlink coverage enhancements.

Proposal 5	RAN2 wait for the RAN1 conclusions on downlink coverage enhancements.

# References

RP-243300, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, 3GPP TSG RAN#106, Dec. 2024.

R2-2405767, “LS on DL coverage enhancements”, Contact: CMCC, To:RAN1, May. 2024.

R1-2407538, “Reply LS on DL coverage enhancements”, Contact: CMCC, To: RAN2, Aug. 2024.

# Appendix A RAN2 agreements

RAN2#126 made these agreements:

Agreements;

- Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements.

- RAN2 assumes that both EFC and EMC are supported

RAN2 has started to study on RAN2 aspects of DL coverage enhancement. To progress the study, RAN2 has identified questions to RAN1 for aspects where the input is required.

Related to this, RAN2 would like to request RAN1 to provide feedback on the following questions.

Question1 : Can RAN1 provide the information on their progress on whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values?

Question2 : Can RAN1 provide the information on whether/how the solution RAN1 is investigating is expected to impact common control signalling for UEs in RRC idle / RRC inactive?

Question3 :  Can RAN1 provide the feedback on whether UL beam hopping is also being studied in RAN1 (and whether this is separate from DL beam hopping)?

Question4 : RAN2 would like to remind RAN1 that satellite beams are currently not visible to UEs and any decision about different beam status (i.e. "off", "common messages only" and "active traffic") will likely have to relate to beams visible to the UE (e.g. SSB beams). RAN2 would also like to know whether RAN1 intends to define beam status for beams not visible to the UE or to define new beam status for beams visible to the UE.

Question5 : Can RAN1 provide the feedback on whether the beam status in different beams (visible to the UE) of one cell are the same or can be different in any given time, i.e.. the beam status is cell specific or beam specific?

And sent an LS to RAN1 [2] with these questions:

And RAN1 has partly replied [3]:

Answer to Q1: According to the updated WID (RP-241667), SSB enhancement other than SSB periodicity extension is not considered. Thereby, RAN1 understanding is that enhancements to the existing SSB patterns, e.g. SSB position in burst, SSB index number, are not within the scope. The extension of the SSB periodicity is still under RAN1’s discussion.

Answer to Q4: Beam footprint status in terms of "off", "common messages only" and "active traffic” in one of the RAN1 agreements is only defined for the sake of system-level evaluation methodology.

At RAN2#127, RAN2 made these agreements:

Agreements:

1.	From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects)

2.	From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms.

3.	RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity

4.	If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

Furter RAN2#127, RAN2 made these agreements:

Agreements:

1.	If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE)

2.	If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit)

3.	(also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

And at RAN2#128, RAN2 made these agreements:

Agreements:

1.	If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

Agreements:

1.	As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs)

2.	RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

Agreements:

1.	For cell level DTX for NTN DL coverage enhancement, RAN2 currently understands that there will be only two states: "cell DTX On period" and "Cell DTX Off period" (FFS if the UE behaviour will be different from the NES states or the same). FFS on Cell DRX states.

And the following discussions were marked for possible comeback in next meeting:

Observation 1: The UE should ideally consider only SMTCs corresponding to the closest neighbor cells

Proposal 4: Consider enhancements to allow location-based UE SMTC selection (broadcast of neighbor cells reference location / SSB indexes associated with SMTCs)

-	Nokia agrees but thinks we could work on a UE-centric solution

-	QC thinks there is no need to capture anything for this at this stage. LG, CATT, Samsung agree

Can come back in the next meeting

Proposal 2: If beam level DTX/DRX for NTN DL coverage enhancement is supported, beam transmission/reception state is modelled into two states: "beam DTX/DRX Active period" state and "beam DTX/DRX inactive period" state.

Continue in the next meeting