3GPP TSG-RAN WG2 Meeting #129	R2-2501326

Athens, Greece, Feburary 17th – 21st, 2025

Source:	CATT

Title:	Analysis on LP-WUS for RRC_CONNECTED

Agenda Item:	8.4.4

Document for:	Discussion and Decision

Introduction

In previous RAN1 meetings and RAN2 meetings, great progress was made for LP-WUS in RRC_CONNECTED mode and RAN1 also sent one LS to RAN2 [1]. Based on the agreements and the LS from RAN1, we will continue to analyze the LP-WUS monitoring procedure in RRC_CONNECTED mode for LP-WUS including the following aspects:

- DC/CA;

- Short DRX cycle;

- LP-WUS monitoring restriction;

- SPS/CG;

- Reply to LS of RAN1;

- LP-WUS enable/disable and activation/deactivation.

And our observations and proposals are provided.

Discussion

2.1	DC/CA

- 2.1.1	Dual DRX group

In order to reduce the power consumption when both FR1 and FR2 cells are configured via Carrier Aggregation, a secondary DRX group was introduced with a separate drx-InactivityTimer and drx-onDurationTimer configured for the FR2 cells enabling FR2 to go to sleep more quickly. However, it is FFS whether LP-WUS can be configured with dual DRX group. In RAN1#119 meeting[2] this issue and the following alternatives were discussed:

- For the case when UE is configured with NR-DC and/or CA without dual DRX groups in RRC CONNECTED mode, - LP-WUS is configured on a serving cell per cell-group - LP-WUS indication is applicable to all activated serving cells in the cell-group - For the case when UE is configured with NR-DC and/or CA with dual DRX groups in RRC CONNECTED mode, - Opt.1 - LP-WUS is configured on a serving cell per cell-group - LP-WUS carries separate indications for each DRX group(s) to trigger PDCCH monitoring of all activated serving cells - Note: This requires RAN2 coordination via LS - Opt.2 - LP-WUS is configured on a serving cell per DRX group per cell-group - LP-WUS indication is applicable to all activated serving cells in the DRX group per cell-group - Note: This requires RAN2 coordination via LS - Opt.3 - LP-WUS is configured on a serving cell per cell-group - LP-WUS indication is applicable to all activated serving cells of all DRX groups per cell-group. FFS: Whether the serving on which the LP-WUS is configured is only PCell/PSCell or RRC configured cell other than PCell/PSCell

In the following, RAN2 continue to discuss these options when UE is configured with NR-DC and/or CA without/with dual DRX groups in RRC CONNECTED mode provided by RAN1.

  - Opt.0

    - LP-WUS is not configured with dual DRX group simultaneously.

In this option, LP-WUS function and dual DRX group are not configured simultaneously which follows DCP mechanism.

  - Opt.1

    - LP-WUS is configured on a serving cell per cell-group

    - LP-WUS carries separate indications for each DRX group(s) to trigger PDCCH monitoring of all activated serving cells

In this option, one LP-WUS is configured per Cell group. It can control the UE to monitor PDCCH separately via separate indications, e.g. DRX group ID can be indicated in the LP-WUS. When the UE is indicated to wake up by LP-WUS with the DRX group ID, the UE monitors the PDCCH for the corresponding DRX group.

  - Opt.2

    - LP-WUS is configured on a serving cell per DRX group per cell-group

    - LP-WUS indication is applicable to all activated serving cells in the DRX group per cell-group

In this option, LP-WUS triggers the UE to wake up to monitor PDCCH separately for different DRX groups. It requires the LP-WUS to be configured for each DRX group, i.e. separate LP-WUS is configured on the corresponding serving cell in each DRX group.

  - Opt.3

    - LP-WUS is configured on a serving cell per cell-group

    - LP-WUS indication is applicable to all activated serving cells of all DRX groups per cell-group.

In this option, the LP-WUS controls all the activated serving cells, e.g., when the UE is indicated to wake up by LP-WUS, the UE wakes up to monitor the PDCCH of all the activated serving cells of all DRX groups but with different timer lengths.

Compared with these options:

- Option 0 follows the DCP principle brings the least spec impacts;

- Option 2 brings benefits on control flexibity but also causes higher resouce consumption, while on the opposite, Option 3 consumes less resource and brings less spec impact but lacks control flexiblity. While Option 1 is one compromised solution with less resource consumption but flexibity.

However, it is within RAN1 scope to discuss the LP-WUS payload and the LP-WUS location. Hence, it is proposed that:

Proposal 1: RAN2 postpone discussion on simultaneous configuration of LP-WUS and dual DRX group until RAN1 make further progress.

- 2.1.2	DC

In 37.340, it is specified that DCP can be configured in MR-DC.

## 6.1 MAC Sublayer /Omitted/ In MR-DC, for power saving purpose, the UE can be configured with DCP to be monitored on the PCell, if the MN is a gNB (i.e. for NE-DC and NR-DC) and/or with DCP to be monitored on the PSCell, if the SN is a gNB (i.e. for EN-DC, NGEN-DC and NR-DC). /Omitted/

And for LP-WUS for DC, RAN1 has made the following agreements in RAN1#118bis meeting:

Agreement LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

It can be seen that RAN1 has agreed that LP-WUS is supported with NR-DC. From our perspective, NR-DC can be extended to MR-DC scenario with restriction that LP-WUS is configured in the NR Cell group. And it can be restricted that the UE can be configured with LP-WUS to be monitored in MCG if the MN is a gNB or SCG if the SN is a gNB.

Proposal 2: LP-WUS is supported when the UE is configured with MR-DC in RRC CONNECTED mode and the LP-WUS can be configured in MCG if the MN is a gNB or SCG if the SN is a gNB.

2.2	Short DRX cycle

For DCP, if short DRX cycle is supported, it may require a huge overhead in the DL which may reduce the power saving gain. In RAN1 replied LS [3], it indicated RAN1 assumed that DCP was not applicable for the short DRX cycle if both long and short DRX cycles are configured for the UE. Finally, it was agreed that DCP was only applicable to long DRX cycle.

However, considering the power saving of LP-WUS is extremely low, it may not impact the power saving gain in Option 1-1. However, like DCP, this has to be confirmed in RAN1. RAN2 can send LS to RAN1 whether LP-WUS can be supported for short DRX cycle in Option 1-1.

Observation 1: RAN2 need to check with RAN1 whether short DRX cycle is applicable for LP-WUS for Option 1-1.

For Option 1-2, the LP-WUS is configured independent of the DRX cycle actually. And the drx-onDurationTimer will not be started as agreed in RAN2 last meeting. Hence, it is meaningless to configure short DRX cycle for Option 1-2. Hence, it is proposed that:

Proposal 3: Short DRX cycle is not supported for LP-WUS for Option 1-2.

2.3	LP-WUS Monitoring restriction

For DCP, it was captured in 38.300 that

# 11 UE Power Saving /Omitted/ A UE can only be configured to monitor DCP when connected mode DRX is configured, and at occasion(s) at a configured offset before the on-duration. More than one monitoring occasion can be configured before the on-duration. The UE does not monitor DCP on occasions occurring during active-time, measurement gaps, BWP switching, or when it monitors response for a CFRA preamble transmission for beam failure recovery (see clause 9.2.6), in which case it monitors the PDCCH during the next on-duration. If no DCP is configured in the active BWP, UE follows normal DRX operation. /Omitted/

It can be seen that the UE does not monitor DCP for the following occasions:

- Active-time

- Measurement gaps

- BWP switching

- Monitoring response for a CFRA preamble transmission for beam failure recovery.

For LP-WUS, the case may be different considering the LP-WUS is monitored via a different receiver and the UE may not monitor LP-WUS when MR is on. Hence, we discuss the above scenarios case by case.

DRX Active Time

Regarding DRX Active time, it is still under discussion on whether the UE needs to monitor LP-WUS during DRX Active Time in RAN1. Hence, RAN2 can wait for more details from RAN1 for the case whether the UE stops monitoring LP-WUS when the UE is in DRX Active Time.

Measurement gap

The UE performs intra/inter-frequency measurement during measurement gap in which the UE does not perform data transmission and reception. And it has been agreed in RAN1[4]-[5] that no impact on RRM/RLM/BFD measurement requirements is assumed for Option 1-1 and Option 1-2, hence the UE will turn on MR during measurement gap to perform measurement. It is similar to the case of DRX Active Time.

BWP switching

The BWP switching is controlled by the PDCCH indicating a downlink assignment or an uplink grant, by the bwp-InactivityTimer, by RRC signalling, or by the MAC entity itself upon initiation of Random Access procedure or upon detection of consistent LBT failure on SpCell. During BWP switch cases other than bwp-InactivityTimer expiration, the MR is on, i.e. the UE may not monitor LP-WUS which is similar to the case of DRX Active Time.

For the case of bwp-InactivityTimer expiration, it is unclear whether MR will be involved in this case. For example, whether there are different MR behaviors for different sleep mode. From RAN2’s perspective, this can be left to RAN1 for more details.

Monitoring response for a CFRA preamble transmission for beam failure recovery

According to the specification, the UE will monitor PDCCH scrambled by C-RNTI during ra-ResponseWindow. Hence, this case is similar to the case of DRX Active Time. RAN2 still needs to wait for RAN1 inputs.

1> if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity: 2> if the contention-free Random Access Preamble for beam failure recovery request was transmitted on a non-terrestrial network: 3> start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the PDCCH occasion as specified in TS 38.213 [6]. 2> else: 3> start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission. 2> monitor for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while ra-ResponseWindow is running.

Observation 2: RAN2 need to check with RAN1 and ask whether the UE monitors LP-WUS during the following cases for Option 1-1 and Option 1-2, including:

- DRX Active Time;

- Measurement gap;

- BWP switch;

- Monitoring response for a CFRA preamble transmission for beam failure recovery.

2.4	SPS/CG

For SPS, the gNB allocates downlink resources for downlink data transmissions to UEs and RRC defines the periodicity of the configured downlink assignments while PDCCH addressed to CS-RNTI can either signal and activate the configured downlink assignment, or deactivate it. During the SPS occasion, the UE will switch the MR to perform PDSCH reception. It depends on RAN1 to decide whether the UE can monitor LP-WUS.

For CG, the gNB allocates uplink resources for uplink data transmissions from UEs. If there is uplink transmission, the MR is on and whether the UE monitors LP-WUS depends on RAN1 discussion like SPS.

Observation 3: Considering the UE performs PDSCH reception during the SPS occasion or uplink transmission during the CG occasion which requires MR to be switched on, RAN2 need to check with RAN1 on whether the UE monitors LP-WUS during the SPS/CG occasion.

Specially, for CG, in case of no MAC PDU needs to be transmitted and the UE supports uplink skipping, the network can configure uplink skipping for the UE. So that, no MAC PDU for the HARQ entity will be generated for the configured grant. Hence, no uplink transmission will be performed. In this case, no MR will be switched on. Hence, the UE can still monitor LP-WUS to remain in low power state.

Proposal 4: If network configures uplink skipping and there is no uplink data to transmit when the UE is in low power state, the UE can keep monitoring LP-WUS in low power state.

2.5	RAN1 LS

- 2.5.1	Incoming LS

In RAN1#119 meeting, RAN1 made agreements regarding Option 1-1 and Option 1-2 in RRC CONNECTED mode. And an LS was sent to RAN1 asking the following issues:

In RAN#119, RAN1 made following conclusion and agreement to use existing L2 signalling to stop C-DRX related timers and resuming LP-WUS monitoring: Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909. Agreement - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909.. Regarding the above conclusion on Option 1-1, RAN1 would like to ask RAN2 whether RAN2 has same understanding or not. Regarding the above agreement on Option 1-2, RAN1 would like to ask RAN2 the feasibility of the above potential solution.

Mainly, there are two issues need to be addressed in the LS, (long) DRX command MAC CE and LP-WUS monitroing when the UE is not in C-DRX active time.

(Long) DRX command MAC CE

In legacy DRX operation, DRX command MAC CE and Long DRX Command MAC CE is used to stop drx-onDurationTimer and drx-InactivityTimer for each DRX group. Consequently, the intended UE behaviour for Option 1-1 is:

- In LP-WUS CONNECTED mode operation, existing DRX Command MAC CE and Long DRX Command MAC CE are used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group.

For Option 1-2, it was agreed in RAN2#128 meeting that:

For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS.

Consequently, the intended UE behaviours for Option 1-2 is:

- In LP-WUS CONNECTED mode operation, existing DRX Command MAC CE and Long DRX Command MAC CE are used to stop the new timer and drx-InactivityTimer for the DRX group.

Proposal 5: RAN2 confirm RAN1 agreement, i.e., in LP-WUS CONNECTED mode operation, existing DRX Command MAC CE and Long DRX Command MAC CE are used to stop:

- drx-onDurationTimer and drx-InactivityTimer for the DRX group in Option 1-1;

- the new timer and drx-InactivityTimer for the DRX group in Option 1-2.

Whether to monitoring LP-WUS when UE is not in DRX active time

For option 1-1, it was agreed in RAN2#126 meeting that:

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

For DCP, the UE does not monitor DCP on occasions occurring during active-time, measurement gaps, BWP switching, or when it monitors response for a CFRA preamble transmission for beam failure recovery according to 38.300. If the similar mechanism is applied, the UE does not monitor LP-WUS during the following occasions including active-time, measurement gaps, BWP switching, or when it monitors response for a CFRA preamble transmission for beam failure recovery. If these aspects are considered for LP-WUS monitoring, it may contradict with RAN1 conclusion. Considering this is in RAN1 scope based on section 2.3, it will be included in the reply LS.

- 2.5.2	Reply LS

Based on discussion in above sections, it can be seen that the reply LS can including the following aspects:

- Replying RAN1:

  - (long) DRX command MAC CE:

    - RAN2 confirm RAN1 agreement, i.e., in LP-WUS CONNECTED mode operation, existing DRX Command MAC CE and Long DRX Command MAC CE are used to stop:

      - drx-onDurationTimer and drx-InactivityTimer for the DRX group in Option 1-1;

      - the new timer and drx-InactivityTimer for the DRX group in Option 1-2.

  - Monitoring LP-WUS when UE is not in DRX active time for Option 1-1 and Option 1-2:

    - When the UE is not in DRX Active Time, the UE may be within the following occasions including measurement gap, BWP switch, monitoring response for a CFRA preamble transmission for beam failure recovery and SPS/CG occasion based on the discussion in Section 2.3 and Section 2.4. It is still questionable whether the UE monitors LP-WUS during these occasions.

    - Hence, RAN2 ask RAN1: whether the UE monitors LP-WUS during the following cases, including: DRX Active Time, Measurement gap; BWP switch; Monitoring response for a CFRA preamble transmission for beam failure recovery and SPS/CG occasion.

- Other issues identified by RAN2:

  - Whether short DRX cycle is applicable for LP-WUS for Option 1-1.

Proposal 6: The reply LS to RAN1 can include the following aspects:

- Reply RAN1:

  - (long) DRX command MAC CE:

    - RAN2 confirm RAN1 agreement, i.e., in LP-WUS CONNECTED mode operation, existing DRX Command MAC CE and Long DRX Command MAC CE are used to stop:

      - drx-onDurationTimer and drx-InactivityTimer for the DRX group in Option 1-1;

      - the new timer and drx-InactivityTimer for the DRX group in Option 1-2.

  - Monitoring LP-WUS when UE is not in DRX active time for Option 1-1 and Option 1-2:

    - When the UE is not in DRX Active Time, the UE may be within the following occasions including measurement gap, BWP switch, monitoring response for a CFRA preamble transmission for beam failure recovery and SPS/CG occasion. It is still questionable whether the UE monitors LP-WUS during these occasions.

    - Hence, RAN2 can ask RAN1: whether the UE monitors LP-WUS during the following cases for Option 1-1 and Option 1-2, including: DRX Active Time, Measurement gap, BWP switch, Monitoring response for a CFRA preamble transmission for beam failure recovery and SPS/CG occasion.

- Other issues identified by RAN2:

  - Whether short DRX cycle is applicable for LP-WUS for Option 1-1.

2.6	LP-WUS enable/disable and activation/deactivation

- 2.6.1	Enable/disable

In RAN1#115 meeting, RAN1 it was agreed that:

RAN1#115 Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance.

For RRC CONNECTED mode, from RAN1 perspective, PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling. But it is FFS whether to support UE assistance to enable/disable PDCCH monitoring triggered by LP-WUS. In our understanding, the gNB can determine when/whether to enable/disable PDCCH monitoring triggered by LP-WUS based on UE capability and the network load. Hence, no UE assistance information is needed.

Proposal 7: UE assistance is not needed to enable/disable PDCCH monitoring triggered by LP-WUS.

- 2.6.2	Activation/deactivation

In RAN1 previous meetings, it was agreed that:

RAN1#118 meeting Agreement - In RRC CONNECTED mode, LP-WUS monitoring can be activated/deactivated by at least one or more of - by gNB RRC signaling, with or without UE assistance. - by gNB L1/L2 LP-WUS activation/deactivation signaling, with or without UE assistance. - based on pre-configured condition(s), such as timer. - LP-WUS monitoring by UE is known to gNB, study whether it could be transparent to gNB. - other options are not precluded.

RAN1#119 meeting [Wed]Proposal 4-1: When LP-WUS monitoring is enabled for RRC CONNECTED mode, support UE fallback to PDCCH monitoring when UE monitors LP-WUS - FFS the condition for the fallback - For LP-WUS operation option 1-1, UE fallbacks to legacy C-DRX behavior - For LP-WUS operation option 1-2, - FFS what UE fallbacks to - FFS how to avoid misalignment between gNB and UE on the UE LP-WUS/PDCCH monitoring

It can be seen that RAN1 is still studying LP-WUS activation/deactivation/fallback to PDCCH monitoring when UE monitors LP-WUS. RAN2 can wait RAN1 for more progress for these aspects.

Proposal 8: For LP-WUS activation/deactivation/fallback to PDCCH monitoring when UE monitors LP-WUS aspects, RAN2 wait for more details from RAN1 for further discussion.

Conclusion

In this document, we analyze the remaining issues in RRC_CONNECTED for LP-WUS, and the following observations and proposals are provided:

DC/CA

Proposal 1: RAN2 postpone discussion on simultaneous configuration of LP-WUS and dual DRX group until RAN1 make further progress.

Proposal 2: LP-WUS is supported when the UE is configured with MR-DC in RRC CONNECTED mode and the LP-WUS can be configured in MCG if the MN is a gNB or SCG if the SN is a gNB.

Short DRX cycle

Observation 1: RAN2 need to check with RAN1 whether short DRX cycle is applicable for LP-WUS for Option 1-1.

Proposal 3: Short DRX cycle is not supported for LP-WUS for Option 1-2.

LP-WUS monitoring restriction

Observation 2: RAN2 need to check with RAN1 and ask whether the UE monitors LP-WUS during the following cases for Option 1-1 and Option 1-2, including:

- DRX Active Time;

- Measurement gap;

- BWP switch;

- Monitoring response for a CFRA preamble transmission for beam failure recovery.

SPS/CG

Observation 3: Considering the UE performs PDSCH reception during the SPS occasion or uplink transmission during the CG occasion which requires MR to be switched on, RAN2 need to check with RAN1 on whether the UE monitors LP-WUS during the SPS/CG occasion.

Proposal 4: If network configures uplink skipping and there is no uplink data to transmit when the UE is in low power state, the UE can keep monitoring LP-WUS in low power state.

RAN1 LS

Proposal 5: RAN2 confirm RAN1 agreement, i.e., in LP-WUS CONNECTED mode operation, existing DRX Command MAC CE and Long DRX Command MAC CE are used to stop:

- drx-onDurationTimer and drx-InactivityTimer for the DRX group in Option 1-1;

- the new timer and drx-InactivityTimer for the DRX group in Option 1-2.

Proposal 6: The reply LS to RAN1 can include the following aspects:

- Reply RAN1:

  - (long) DRX command MAC CE:

    - RAN2 confirm RAN1 agreement, i.e., in LP-WUS CONNECTED mode operation, existing DRX Command MAC CE and Long DRX Command MAC CE are used to stop:

      - drx-onDurationTimer and drx-InactivityTimer for the DRX group in Option 1-1;

      - the new timer and drx-InactivityTimer for the DRX group in Option 1-2.

  - Monitoring LP-WUS when UE is not in DRX active time for Option 1-1 and Option 1-2:

    - When the UE is not in DRX Active Time, the UE may be within the following occasions including measurement gap, BWP switch, monitoring response for a CFRA preamble transmission for beam failure recovery and SPS/CG occasion. It is still questionable whether the UE monitors LP-WUS during these occasions.

    - Hence, RAN2 can ask RAN1: whether the UE monitors LP-WUS during the following cases for Option 1-1 and Option 1-2, including: DRX Active Time, Measurement gap, BWP switch, Monitoring response for a CFRA preamble transmission for beam failure recovery and SPS/CG occasion.

- Other issues identified by RAN2:

  - Whether short DRX cycle is applicable for LP-WUS for Option 1-1.

LP-WUS enable/disable and activation/deactivation

Proposal 7: UE assistance is not needed to enable/disable PDCCH monitoring triggered by LP-WUS.

Proposal 8: For LP-WUS activation/deactivation/fallback to PDCCH monitoring when UE monitors LP-WUS aspects, RAN2 wait for more details from RAN1 for further discussion.

Reference

- R2-2500012, LS on LP-WUS operation in CONNECTED mode (R1-2410909; contact: NTT DOCOMO);

- R1-2500003 , RAN1#119 meeting report;

- R1-1911475, LS on PDCCH-based Power Saving Signal/Channel carrying indication of UE wakeup before DRX ON, CATT;

- R1-2407587, RAN1#118 meeting report;

- R1-2409341, RAN1#118bis meeting report