3GPP TSG-RAN WG2 Meeting #129					R2-2501514

Athens, Greece, Feb. 17th – 21st, 2025

Source: 	ZTE Corporation (Offline Rapporteur)

Title:	Report from offline on segment retransmissions for A-IoT

# Segment Retransmission

Solutions for Segment retransmission (only window size of 1 is considered)

| # | Solution description | Technical problems if any (i.e. does it work or not) | Implications on reader/device |
| --- | --- | --- | --- |
| 1 | NDI based: - The reader will set the NDI to 0 in the initial transmission - Toggle the NDI for successful reception - Keep the NDI bit for the failed reception - The solution enables segmentation, retransmission | - Works fine | - Device needs to remember: - previous NDI - Two offsets into data, e.g, OFFSET for NDI 0 and OFFSET for NDI 1 - 1 bit NDI is needed in the R2D grant |
| 2 | Reader indicates total number of bits received successfully - In addition to the new TB size (i.e. the grant size), the reader will indicate explicitly how many bits have been successfully received so far in each grant | - Works fine | - R2D message includes new grant size + the total number of bits received successfully so far - The device needs to remember the start point of the data |
| 3 | Implicit ACK + Explicit NACK | - Seems that this option doesn’t work (in the example shown, there is duplicated segment 1 transmission) - There may be some mis-alignment | - Similar to solution 1, the device needs to remember how much data was transmitted in the previous transmission (the TB size of the previous grant) and where to transmit from |

Observations on level of support

Overall split

Solution 1: 9

Solution 2: 4

No retransmission: 6

Slight majority for solution 1

If segmentation retransmission is supported, which solution will you support

Solution 1: 13

Solution 2: 9

Slight majority for solution 1

Should we support segmentation retransmission or not:

Yes, support: 11

No, no need: 12

Split views

Quickly discuss online if any further changes to the views happened after offline discussion by checking the following proposal.

Proposal 1: If segment retransmission is supported, choose between the following options for segment retransmission:

- Solution 1: NDI like solution (reader indicates 1 bit NDI to indicate to the device whether or not to transmit the next segment in the R2D grant)

- Solution 2: Reader based solution (Reader explicitly indicates an offset – e.g. number of bits successfully received so far, in the R2D grant)

# Segmentation indication

R2-2500253	Protocol design principle and considerations on Data transmission	Xiaomi	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 14: 1-bit indication is introduced to indicate whether the data is segmented and whether it is the last segment.

Noted

R2-2501127	Data Transmission and Other General Aspects of Ambient IoT	Qualcomm Incorporated	discussion	Rel-19	Ambient_IoT_Solutions-Core

Proposal 7:	To support segmentation, two-bit segment indication is supported. The Segmentation Info (SI) field in NR RLC PDU can be as a baseline solution.

Noted

1 bit vs 2 bit segmentation indication:

Proposal 2: A 1 bit indication is introduced to indicate whether the data is segmented and to indicate whether it is the last segment

# Conclusion

Observation 1:

The following two solutions can work for segment retransmission:

-	Solution 1: NDI like solution (reader indicates 1 bit NDI to indicate to the device whether or not to transmit the next segment in the R2D grant)

-	Solution 2: Reader based solution (Reader explicitly indicates an offset – e.g. number of bits successfully received so far, in the R2D grant)

Observation 2:  on level of support for various options

Overall split

Solution 1: 9

Solution 2: 4

No retransmission: 6

Slight majority for solution 1

If segmentation retransmission is supported, which solution will you support

Solution 1: 13

Solution 2: 9

Slight majority for solution 1

Should we support segmentation retransmission or not:

Yes, support: 11

No, no need: 12

Split views

Quickly discuss online if any further changes to the views happened after offline discussion by checking the following proposal.

Proposal 1: If segment retransmission is supported, choose between the following options for segment retransmission:

- Solution 1: NDI like solution (reader indicates 1 bit NDI to indicate to the device whether or not to transmit the next segment in the R2D grant)

- Solution 2: Reader based solution (Reader explicitly indicates an offset – e.g. number of bits successfully received so far, in the R2D grant)

Proposal 2: A 1 bit indication is introduced to indicate whether the data is segmented and to indicate whether it is the last segment