3GPP TSG RAN WG2#129bis	R2-2501739

Wuhan, China, 7th - 11th April 2025

3GPP TSG-RAN WG4 Meeting #114	R4-2502662

Athens, GR, 17th – 21st February, 2025

Title:	LS on CSSF optimization for NR RRM Phase 5

Response to:

Release:	Rel-19

Work Item:	NR_RRM_Ph5-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Jie Cui,  jie_cui@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

For NR RRM Phase 5 WI, RAN4 would like to update RAN2 the progress of the discussion on solutions for FR2-1 L3 measurement delay reduction by optimizing CSSF (carrier specific scaling factor) outside gap in CA/DC. Following agreements have been reached, which will need RAN2 signaling design (highlighted in yellow).

Agreements in RAN4#114 meeting: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band, - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there are multiple SCCs but no such SCC on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap. Agreements in RAN4#113 meeting: Agreement: - RAN4 to introduce new individual capability(s) for CSSF reduction in R19. - delay the capability detail discussion to the end of the core part. Agreement: - after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable Agreements in RAN4#112 meeting: Agreement: - RAN4 to consider following CA/DC mode for L3 measurement delay reduction by optimizing CSSFoutside_gap,i - Final applicability of the solutions can be FFS - EN-DC - EN-DC with FR2 only intra band CA - EN-DC with FR2 only inter band CA - EN-DC with FR1 +FR2 CA (FR1 PSCell) - EN-DC with FR1 +FR2 CA (FR2 PSCell) - NE-DC - NE-DC with FR2 only intra band CA - NE-DC with FR2 only inter band CA - NE-DC with FR1 +FR2 CA (FR1 PCell) - SA - FR1+FR2 CA (FR1 PCell) - FR1+FR2 CA (FR2 PCell) - FR2 only intra-band CA - FR2 only inter-band CA - NR-DC - FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell)

If any additional solutions which need signaling design are confirmed, RAN4 will provide corresponding information in the future LS. RAN4 respectfully requests RAN2 to take above agreements into account in their work.

# 2	Actions

To RAN2:

ACTION: 	RAN4 respectfully asks RAN2 to take the above into account in their future work.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #114bis	7th – 11th April 2025	Wuhan, China

TSG-RAN WG4 Meeting #115	19th – 23rd May 2025	Malta