3GPP TSG-RAN WG2 Meeting #129bis                                                           R2-2501780

Wuhan, China, April 7th– 11th, 2025

Agenda item:	8.9.4

Source:	vivo

Title:	Discussion on RAN2 Imapcts of IoT-NTN TDD mode

Document for:	Discussion and Decision

- Introduction

In RAN2#129 [1], the following agreements were made by RAN2 on IoT-NTN TDD.

Agreements: 1. RAN2 will continue studying paging aspects based, on RAN1 progress 2. RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed. 3. RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network. 4. RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated. 5. RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system. 6. RAN2 can continue the discussion also on RAR window 7. Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

Besides, RAN1 also discussed the issue in RAN1#120 [2], and the following conclusion was made.

Conclusion It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any.

In the contribution, we will analyse the open issue highlighted in yellow and give our view.

- Discussion

## 2.1 FFS cellBarred or cellBarred-NTN

According to the WID [3], the IoT-NTN TDD will be deployed in the specific band, i.e., 1616-1626.5 MHz MSS allocated band. Thus, UE can identify the corresponding cell operating in IoT-NTN TDD mode without NW indication. From this perspective, whether the legacy cellBarred or the cellBarred-NTN is used to indicate the cell-barring of the cell has no function difference.

The work objectives assume the following: - LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763) - Target the 1616-1626.5 MHz MSS allocated band - Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT) - Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO - The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with N=9 for the target MSS allocated band.

However, for legacy NTN cells, the cellBarred-NTN should always be indicated [4]. To make it consistent, it is proposed that UE also use the cellBarred-NTN for cell access control for IoT-NTN TDD.

cellBarred-NTN Barred means the cell is barred for connectivity to NTN, as defined in TS 36.304 [4]. E-UTRAN always includes cellBarred-NTN and sets cellBarred to 'barred' in an NTN cell.

Proposal 1: The cellBarred-NTN is used for cell access control for IoT-NTN TDD mode.

## 2.2 FFS to extend Kmac

In RAN2#129, there is an assumption that the Kmac may need to be extended for the scenario that several satellite relays via ISL between the UE and the eNB.

2. RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed.

Kmac means the latency between the RP and the eNB, the value of the Kmac depends on the location of the RP. That is, if the RP is located in the nearest satellite to the eNB, there is no reason why the Kmac should be extended. Thus, we do not think the Kmac shall have to be extended to address the satellite relay between UE and the eNB scenario.

Further, Kmac was originally proposed and decided by RAN1 in the R17 NTN discussion since it is related to the scheduling issue. Besides, RAN1 also discussed the issue in RAN1#120, but no enhancement will be made. During the RAN1 discussion, they concluded that this is not IoT-NTN TDD specific issue and it is out of scope. Thus, it is proposed that RAN2 follows the RAN1 conclusion that no enhancement for Kmac is needed.

Conclusion RAN1 does not further discuss the issue of Kmac extension unless triggered by RAN2 LS.

Proposal 2: RAN2 confirms that no enhancement on Kmac is needed.

## 2.3 Impacts on MAC timers

In general, the following are the summarized UP times and their units.

| MAC times in units of PDCCH Period (PP) | RAR window, mac-ContentionResolutionTimer, pur-ResponseWindowTimer, OnDurationTimer, drx-InactivityTimer, drx-RetransmissionTimer, drx-ULRetransmissionTimer |
| --- | --- |
| MAC timers depend on valid DL/UL scheduling | DL HARQ RTT Timer, UL HARQ RTT Timers |

The following is the definition of the PDCCH Period (PP) [5]. According to the definition of the PP, it can be seen that 1 PP is the interval between the start of the TWO consecutive PDCCH occasions. That is, when it applies to the IoT-NTN TDD system, the duration of the PP covers at least one TDD pattern (90ms), including the UL duration and guard time. Thus, the timers in units of PP are long enough to ensure that UE can receive the corresponding response from NW.

PDCCH period (pp): Refers to the interval between the start of two consecutive PDCCH occasions and depends on the currently used PDCCH search space, as specified in TS 36.213 [2]. A PDCCH occasion is the start of a search space and is defined by subframe k0 as specified in clause 16.6 of TS 36.213 [2]. The calculation of number of PDCCH-subframes for the timer configured in units of a PDCCH period is done by multiplying the number of PDCCH periods with npdcch-NumRepetitions-RA when the UE uses the common search space or by npdcch-NumRepetitions when the UE uses the UE specific search space. When counting a timer whose length is calculated in PDCCH-subframes, the UE shall include PDCCH-subframes that will be dropped or not required to be monitored as specified in clause 16.6 of TS 36.213 [2]. The calculation of number of subframes for the timer configured in units of a PDCCH period is done by multiplying the number of PDCCH periods with duration between two consecutive PDCCH occasions.

As for timers depending on the valid DL/UL scheduling, the following is a description of how to set the corresponding value of the timers [5]. It can be seen that the duration of the timers depends on the valid scheduling time point, i.e., the start subframe of the next valid PDCCH occasion. Take the DL HARQ RTT timer as an example, the length between the time point “3+RTT offset” and the time point “the potential 1st subframe of PDCCH occasion” already includes the duration of the UL duration and the guard time for the IoT-NTN TDD scenario.

For NB-IoT, the intended UE behaviour regarding setting the HARQ RTT Timer is shown in Figure C-1 and for the UL HARQ RTT Timer is shown in Figure C-2. Figure C-1: Setting the HARQ RTT Timer for NB-IoT Figure C-2: Setting the UL HARQ RTT Timer for NB-IoT

Observation 1: The length of the times in units of PP and the times depending on valid scheduling already cover the UL duration and the guard time.

Proposal 3: The length of the legacy MAC timers can be directly reused for IoT-NTN TDD network, no enhancement is needed.

## 2.4 Scheduling and transmission of SI message

As shown in Figure 1, due to the TDD pattern, the start position of the SI message or the duration of the SI message for a specific SI overlap with the guard time or the UL duration of the TDD pattern.

Figure 1: SI message resource conflictions with the FDD mode and TDD pattern

If the SI window is postponed to the next valid DL subframes of the TDD pattern, it may impact the next DL transmission in the next DL subframe. Correspondingly, it may lead to the timing relationship of the system becoming very complicated and even ambiguous. Moreover, there is no way to specify the UE behaviour from the spec perspective. Rather, one may argue that the SI window and the SI transmission can be dropped directly in case the SI window overlaps with the non-DL subframes, but this may impact the system performance due to the reduced resources for SI transmission. Further, as shown in Figure 1, the start position of the SI window is within the non-valid DL subframes of the TDD pattern, dropping means the whole SI window start position is ambiguous. Moreover, it is also very complicated from the UE perspective since UE may need to decide whether it is overlapped and how/whether to drop the resource.

Observation 2: The Postponement of a channel when it overlaps with non-U/D NB-IoT subframes may make the system timing relationship complicated and ambiguous.

Observation 3:  UE behaviour is unclear and cannot be specified from the spec perspective if the SI window is postponed to the next valid DL subframe.

Observation 4:  Dropping (full or partial) of SI windows when they overlap with non-valid DL NB-IoT subframes has impacts on system performance.

Observation 5:  Dropping (full or partial) of SI windows when they overlap with non-valid DL NB-IoT subframes may cause complicated UE behaviour.

Another solution is to introduce a new time resource allocation unit, i.e., the TDD frame and/or the hyper TDD frame. The  90ms length TDD frame is aligned with the IoT-NTN TDD pattern period, and the maximum of the TDD frame is X. As for the hyper TDD frame, the length is X TDD frame, and the maximum hyper TDD frame is Y. The following is an example of the TDD frame.

Figure 2: TDD frame/hyper TDD frame structure

Based on the newly introduced TDD frame and/or the hyper TDD frame, the network configures the resource periodicities and uses the new time unit to decide the SI window. More specifically, if we set the X=1024, and Y=1024, then the only change for the SI window calculation formula is to change the term “floor(x/10)” to “floor(x/90) within the formula of the SI window calculation if the SI window length configuration is ms” or “floor(x) within the formula of the SI window calculation if the SI window length configuration is TDD frame”.

Observation 6: New periodicities based on the TDD frame and/or the hyper TDD frame can save the issue of the SI window and all other resource overlap with the non-valid DL/UL subframe and have limited spec impacts on the resource calculation formula.

Proposal 4: Introduce new periodicities based on the time unit of the TDD frame and/or the hyper TDD frame to decide the SI window and SI transmission resource.

Proposal 5: Send LS to RAN1 to indicate RAN2 agreements on scheduling of the SI window and SI transmission resource.

- Conclusion

In this contribution, we have further discussed the IoT-NTN TDD system. And we make the following observations and proposals:

FFS cellBarred or cellBarred-NTN

Proposal 1: The cellBarred-NTN is used for cell access control for IoT-NTN TDD mode.

FFS to extend Kmac

Proposal 2: RAN2 confirms that no enhancement on Kmac is needed.

Impacts on MAC timers

Observation 1: The length of the times in units of PP and the times depending on valid scheduling already cover the UL duration and the guard time.

Proposal 3: The length of the legacy MAC timers can be directly reused for IoT-NTN TDD network, no enhancement is needed.

Scheduling and transmission of SI message

Observation 2:  Postponement of a channel when it overlaps with non-U/D NB-IoT subframes may make the system timing relationship complicated and ambiguous.

Observation 3:  UE behaviour is unclear and cannot be specified from the spec perspective if the SI window is postponed to the next valid DL subframe.

Observation 4:  Dropping (full or partial) of SI windows when they overlap with non-valid DL NB-IoT subframes has impacts on system performance.

Observation 5:  Dropping (full or partial) of SI windows when they overlap with non-valid DL NB-IoT subframes may cause complicated UE behaviour.

Observation 6: New periodicities based on the TDD frame and/or the hyper TDD frame can save the issue of SI window and all other resource overlap with the non-valid DL/UL subframe and have limited spec impacts on the resource calculation formula.

Proposal 4: Introduce new periodicities based on the time unit of the TDD frame and/or the hyper TDD frame to decide the SI window and SI transmission resource.

Proposal 5: Send LS to RAN1 to indicate RAN2 agreements on scheduling of the SI window and SI transmission resource.

- Reference

- Draft_RAN2_129_Meeting_Report_v1.

- Draft_RAN1_120_Meeting_Report_v1.

- RP-242415, New WID on introduction of IoT-NTN TDD mode, Iridium

- TS36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.

- TS36.321, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.