3GPP TSG-RAN WG2 Meeting #129bis	R2-2501811

Wuhan, China, Apr. 7th – 11th, 2025

Source:	vivo

Title:	Discussion on signaling feasibility of OTA solutions

Agenda Item:	8.1.5

Document for:	Discussion and Decision

- Introduction

During the RAN1#119 meeting, RAN1 discussed the options for resolving issues related to inter-vendor training collaboration of the CSI-compression use case. RAN1 concluded that standardized signaling, if feasible and specified, can be used for parameter and/or dataset exchange to alleviate/resolve the inter-vendor training collaboration complexity. In addition, to facilitate the model parameter and/or dataset sharing, RAN1 concluded the following two candidate standardized signaling approaches:

- over-the-air

- other approaches

After that, RAN1 sent the LS [1] to RAN2 on the feasibility of the standardized signaling for the down-selected options:

Action: RAN1 respectfully asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for NW-side sharing model parameters and/or dataset to the UE or UE-side for the following options - Dataset sharing consisting of {(Target CSI, CSI feedback)} - Encoder parameter sharing - Encoder parameter sharing + dataset sharing consisting of {target CSI}

At the RAN2#129 meeting, RAN2 had an initial discussion on the understanding and candidate solutions for these two approaches. However, no consensus was reached and one post-meeting email discussion was triggered. In the conclusion of the report for the email discussion [2], the challenges of OTA solutions are analyzed but the feasibility was not concluded.

In this contribution, we will provide our analysis of the feasibility of the OTA solutions. Besides, additional dependency on other WGs is also analyzed.

# Discussion

In the report [2], for the OTA approach, the following three solutions are summarized:

- OTA solution 1a: gNB -> UE via CP

- OTA solution 2: CN -> UE via gNB

- OTA solution 3: OAM -> UE via gNB

Furthermore, the following challenges and potential suitable scenarios for OTA solutions were listed:

Table 2.1-1 Challenges and potential suitable scenario for OTA solutions[2]

| OTA Solution 1a | Challenges | - Not feasible to have RRC buffer >200Mbyte for UE in 5G - Significant specification impact: - Other segmentation beyond RRC layer requires a new SRB protocol stack to perform segmentation, including functions such as handling segmentation, retransmission, etc - UE selection - Challenges to support E2E reliability, considering dataset/model parameter transfer is shared by different gNB/vendors during UE mobility and different RRC state transition - Uu overhead for data collection from UE and dataset/model parameter transfer to UE - Overloading CP with large datasets would disrupt core control message transmission (e.g. service degradation, reliability, etc) |
| --- | --- | --- |
|  | Potential suitable scenario | - Small dataset/model parameter size. However, the maximum RRC segment needs to be further studied - Split large dataset/model parameter into small pieces, and potentially send to multiple UEs, then gather by UE training entity. RAN2 has not study the feasibility of split dataset/model parameter to multiple UEs. |
| OTA Solution 2 | Challenges | - Same challenges as OTA solution 1a, if OTA solution 2 via CP - No benefit over non-OTA solution, as dataset/model parameter needs to transmit to CN, then transmit back to gNB. Relaying dataset/model parameter via gNB to UE then back to UE training entity is not desireable. - Unclear how to guarantee E2E reliability across multiple hops - A risk of proprietary information exposure if gNB and CN are not from the same NW vendor |
|  | Potential suitable scenario | - Feasibility analysis of OTA solution 2 via CP is the same as OTA solution 1a. - OTA solution 2 and its feasibility is required to be evaluated by RAN3 and SA2. |
| OTA Solution 3 | Challenges | - Same challenges as OTA solution 1a, if OTA solution 2 via CP - No benefit over non-OTA solution, as dataset/model parameter needs to transmit to OAM, then transmit back to gNB. - Unclear how to guarantee E2E reliability across multiple hops - A risk of proprietary information exposure that OAM may share to a second NW vendor |
|  | Potential suitable scenario | - Feasibility analysis of OTA solution 3 via CP is the same as OTA solution 1a. - OTA solution 3 and its feasibility is required to be evaluated by RAN3 and SA5. |

From RAN2 perspective, further analysis can focus on Solution 1a since the challenges are shared by all these three solutions. The detailed challenges are as follows:

- Challenge 1: Not feasible to have RRC buffer >200Mbyte for UE in 5G

- Challenge 2: Significant specification impact:

  - Other segmentation beyond RRC layer requires a new SRB protocol stack to perform segmentation, including functions such as handling segmentation, retransmission, etc

  - UE selection

- Challenge 3: Challenges to support E2E reliability, considering dataset/model parameter transfer is shared by different gNB/vendors during UE mobility and different RRC state transition

- Challenge 4: Uu overhead for data collection from UE and dataset/model parameter transfer to UE

- Challenge 5: Overloading CP with large datasets would disrupt core control message transmission (e.g. service degradation, reliability, etc)

For challenge 1, the key issue is the maximum size. In the LS, the typical size of selected options is indicated as follows:

- For Option 4-1, sharing {target CSI, CSI feedback} dataset, the typical size of dataset is 225 MB.

- For Option 3a-1 without target CSI: sharing encoder parameter, the typical size of model parameter is 11.6 MB.

- For Option 3a-1 with target CSI: sharing encoder parameters along with {target CSI} dataset, assuming N2=N1, the typical size of dataset and model parameter is 161.6 MB.

To address challenge 1, one solution is taking the model parameter sharing as a higher priority, since the typical size of the model parameter is 11.6 MB and the minimum size can be 36 kB.

- RAN2 to prioritize the feasibility study of OTA solutions for the model parameter sharing.

For challenge 2, the maximum size of a PDCP SDU is 9k bytes. Assuming the dataset/parameter is shared with one single UE, the segment number of model parameter sharing is 1289.

For the existing downlink CP-based message, the maximum size of one message can be extended with segmentation:

- RRC segmentation is realized with a sequence number and end marker, allowing a maximum of 5 segments for downlink. Therefore, the largest downlink message that RRC can send cannot exceed 45k (9k*5).

- LPP uses a segmentation method with end marker only and has no limit on the number of segments. The LPP layer is responsible for reassembling segments.

Since the current supported segment number of RRC message is far less than the required segment number, it is not recommended to consider increasing the RRC segment number to support CP-based dataset/parameter sharing.

On the other hand, the LPP-like segmentation can be considered, i.e., based on the end marker.

- LPP-like segmentation can be considered, i.e., based on end marker, to support CP-based model parameter sharing.

For UE selection for dataset/model parameter transfer, it is worth noting that in some scenarios there may be some security or user consent issues that may need to be considered. For example, in case the dataset for training a two-sided model is typically composed of data samples collected from different UEs/vendors, sharing dataset/model parameter would mean sharing data samples/model implementation from one UE/vendor to other UEs/vendors. Whether there are security issues and whether an additional mechanism of user consent is needed should be referred to SA3.

- LS to SA3 to study whether there is security issue of sharing dataset/model parameter from one UE/vendor to other UEs/vendors and whether an additional user consent mechanism is needed.

The issues of challenge 3 can be addressed by NW implementation in Rel-19, e.g., the NW can select the UE in the center of the cell (UE with good RRM measurement results) to share the model parameter. Thus no need to consider the mobility issue. Besides, the RRC state of UE is controlled by the gNB. If the model parameter sharing is ongoing, then the gNB can keep the UE in RRC_CONNECTED rather than release it in the RRC_INACTIVE/IDLE.

- No need to consider the reliability issue of model parameter sharing in Rel-19, i.e., rely on NW implementation to ensure the UE can acquire the parameter within the cell in RRC_CONNECTED state.

To relieve the impact of NW-side data collection on existing signaling, it was agreed that the data collection report will not be transmitted over SRB1 and will utilize another SRB with lower priority. The approach can be reused to address challenges 4 and 5, i.e., model parameter sharing is not transferred over SRB1 and can reuse the SRB for NW-side data collection.

- Model parameter sharing is not transferred over SRB1 and can reuse the SRB with low priority for NW-side data collection.

Based on the above analysis, we propose:

- Conclude that model parameter sharing from gNB to UE via CP is feasible from the RAN2 perspective.

- Conclusion

This contribution concludes with the following:

- RAN2 to prioritize the feasibility study of OTA solutions for the model parameter sharing.

- LPP-like segmentation can be considered, i.e., based on end marker, to support CP-based model parameter sharing.

- LS to SA3 to study whether there is security issue of sharing dataset/model parameter from one UE/vendor to other UEs/vendors and whether an additional user consent mechanism is needed.

- No need to consider the reliability issue of model parameter sharing in Rel-19, i.e., rely on NW implementation to ensure the UE can acquire the parameter within the cell in RRC_CONNECTED state.

- Model parameter sharing is not transferred over SRB1 and can reuse the SRB with low priority for NW-side data collection.

- Conclude that model parameter sharing from gNB to UE via CP is feasible from the RAN2 perspective.

- Reference

- R1-2410922	LS on signalling feasibility of dataset and parameter sharing

- R2-2501939	Report of [POST129][029][AI Phy] Model transfer (Xiaomi/Ericsson)