3GPP TSG-RAN WG2 Meeting#129bis	R2-2501812

Wuhan, China, 7th – 11th Apr. 2025

Source:	vivo

Title:	Discussion on A-IoT Paging

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

RAN2#129 meeting has made some progress in terms of A-IoT paging [1]. In this contribution, with the guidance of issues left with FFS, we investigate A-IoT paging message from the following perspectives:

- Reader visibility of paging identifier based on the post meeting email;

- Format;

- Detailed device behaviour on receiving a paging message.

- Discussion

  - Reader visibility of paging identifier

RAN2 has assumed that paging identifier is transparent to A-IoT MAC layer [1].

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2. - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer.

The email discussion [2] investigated that the visibility issue is in essence about how reader associate the assigned AS ID maintained at reader, to the further command service from CN.

Company views are diverse on whether the paging identifier should be ‘visible’ to MAC layer. Some companies also raised that question was not very clear (i.e., what does being ‘visible’ mean?). Given the comments are diverse, Rapporteur does not intend to make any specific proposal on the ‘visibility’ of the paging identifier to the MAC layer. This can be handled based on further progress on the discussion of paging message MAC PDU formats. However, multiple companies raised how the reader knows the association between the paging identifier (presumably provided by the CN to the reader) and the AS-ID (presumably used over the air for at least the inventory+command use case.) Discuss whether and how the reader knows the association between the paging identifier (presumably used by the CN in the request to the reader) and the AS-ID (presumably used over the air for command delivery to device in inventory+command use case.)

In our understanding, the reason why such issue exists lies in how the inventory procedure is triggered and performed for a command service. However, since paging identifier can be in the form of “filtering criteria” to filter a number of devices in one paging message with a number of AS ID assigned for each paged device, it cannot help this problem even if reader is aware of the paging identifier. If the reader is exposed with device ID in MSG3, this problem can somehow be solved within RAN interaction. However, there is no way for such a NAS layer information of device ID to be exposed to the reader for security consideration.

- The paging identifier can be in the form of filtering criteria to filter one or multiple devices, which cannot be associated to a specific AS ID assigned for one device for the further command service.

- The reader needs to know the association between the assigned AS ID and the upcoming command request towards a specific device, where the reader being informed of the paging identifier does not solve the issue.

From the basic procedure for the command service in TS 23.369 [3] as attached in Annex, it is noticeable that the command service from AF can trigger both inventory and command procedures from AIOTF towards device sequentially. And each of these procedures is allocated with a correlation ID. Besides, there is an EN as “FFS how to handle the scenarios that the Inventory Response received for Command Request sent for many individual devices are in parallel”. This means that they would study how to associate the inventory response and command request likely at the AIOTF side.

The need from RAN2 side is that the reader needs to associate the assigned AS ID and the upcoming command for a specific device without being informed of the upper layer device ID. In our understanding, there can be some ways to solve the issue, like NW interface may introduce some additional information transfer to ensure that the reader is informed of the command is towards a specific device without the leak of device ID to the reader.

However, most importantly, we have to admit that this issue is related to SA2/RAN3 design on the interface between the AIOTF and the reader. In this sense, if we firstly confirm the requirement to solve the issue, then we may trigger the discussion in other WG(s) on the possible solutions for such issue.

- RAN2 to send LS to inform SA2 and RAN3 about the requirement at the reader to associate between the assigned AS ID and the upcoming command request toward a specific device. Wait for their solution(s) on such issue.

  - Format

Drawing forth our design on A-IoT paging message format, here cites some related agreements.

- At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). - Specify message types and contents. As starting point consider the following MAC message types. R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) R2D MAC PDU (MSG2) D2R MAC PDU (MSG3 and data) R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements. - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field.

Based on the content of the paging message and aligning with the overall design of A-IoT MAC PDU, we came out the possible format of paging R2D MAC PDU in Figure 2.2-1. Note that the number of occupied bits in each field and whether Reserved field is needed are for example and to be further discussed.

- The Header of paging message consists of the following field:

- The message type field indicates the type of this message (i.e., the format of the MAC PDU) is for A-IoT paging.

- The TI field is the transaction ID.

- The Q field is to indicate the number of access slots in one paging round. Details are up to RAN1 and a technically-reasonable number of devices taking part in a round of A-IoT paging.

- The RA resource allocation field is the dedicated or common resources for MSG1 transmission. Details are up to RAN1. If RA resource allocation needs to be with flexible length, there will be an extra LI field in front of such field.

- The SDU of paging message: Paging Identifier field to be delivered to A-IoT NAS layer.

In some cases, the reader does not include paging identifier in the paging message to page all the devices under its coverage, which is demonstrated in Figure 2.2-2.

Figure 2.2-1: A-IoT MAC PDU format for A-IoT paging message with the paging identifier

Figure 2.2-2: A-IoT MAC PDU format for A-IoT paging message without the paging identifier

- The possible MAC PDU format of A-IoT paging message contains:

- Message type to indicate paging;

- Transaction ID;

- MSG1 resource allocation information;

  - The number of slots for the paging round (e.g., Q value in RFID);

  - Resource allocation for the paging round, details up to RAN1;

The exact form (whether the two parts can be combined) is FFS

- Paging identifier (as an optionally-present upper layer container).

As for the R2D trigger message, firstly, we would like a clarification on the functionality of it. This message is used for timing calibration, i.e., when receiving this message, the device who has received the initial paging message and obtained the randomizing Q value, should consider the next access slot is coming, and should have the locally-maintained slot counter minus 1.

- RAN2 to confirm the R2D trigger message is purely for timing calibration in slotted-aloha CBRA scheme, i.e., at reception of this message, the device has the locally-maintained slot counter minus 1.

Recurring Way-1, In RFID, QueryRep is modelled as an individual message with a different codepoint in message type from paging message. For Way-2, reusing paging message for saving codepoint of message type, R2D trigger can be achieved as the access slot boundary by omitting some redundant part of the full paging message but only left with necessary transaction ID. Here we demonstrate the characteristics for these two ways in Table 2.2-1.

Table 2.2-1: Comparison of two R2D trigger formats

|  | Way-1 | Way-2 |
| --- | --- | --- |
| Characteristic 1 | Device identifies the content by message type. | Device identifies the content via the bitmap indication of the succeeding field. |
| Characteristic 2 | Extra codepoint occupancy in message type. | For the sake of a uniform of MAC PDU format corresponding to a message type, introduction of the bitmap indication of the flexibility combination of the succeeding fields in paging message. |

Based on the above analysis, we slightly prefer Way-1 for R2D trigger message format. This is a more concise design for a simple function as for the slot boundary, letting the functionality partition of paging message and R2D trigger message more precise.

- Adopt Way-1 (i.e., introducing an explicit message type for R2D trigger) for the format of R2D trigger message.

  - Device behavior upon receiving a paging message

In consideration to how device avoids duplicate responses for paging messages with same transaction ID, it is only agreed that device can determine whether to skip sending the response to the paging message. We would like to further talk about the detailed criterion on how the device makes the determination, which should be captured as device behavior. We identify the following aspects for device to determine whether to respond to a paging message.

First and foremost, as a common sense, the device should ascertain that whether it the paging message is of the same transaction ID of the last received one, where transaction ID is employed to identify the corresponding service, and thus allow UE to not respond to the duplicate paging for the same service.

Secondly, the device should check whether it is to be paged in the current paging. This is purely done by the A-IoT NAS layer of the device to confirm the paging identifier identify the device itself is paged.

Thirdly, the device should tell whether it has been successfully responded to the last paging message of the same transaction ID before. Based on the progress discussed in Random access, the re-access can be triggered for contention resolution failure as a baseline scheme, and an optional success/failure indication about MSG3/the following D2R data transmission.

In this understanding, we believe the criteria for the device to tell whether it has been successfully responded to a paging message is the reversed condition for re-access. In this understanding, from the device perspective, the respond to the current paging is determined by 1) whether this is the initial paging message for the corresponding transaction ID; 2) whether it is to be paged for the initial paging message; 3) if this is the subsequent paging message (i.e., the device has received another paging message before corresponding to the same service as the current one), whether the random access is successful for that received paging message.

In sum, entering the latter phase of WI, the device behavior upon receiving the A-IoT paging message (and R2D trigger message) should be considered and specified.

- The A-IoT device responses to a currently-received paging message, if:

- it is paged by the message; and the message is an initial paging message;

- it is paged by the message; the message is a subsequent paging message; and the last access procedure is failure.

- The A-IoT device identifies the currently-received paging message is:

- an initial paging message, if the message includes a transaction ID different from the one in the last-received paging message.

- a subsequent paging message, if the message includes a transaction ID same as the one in the last-received paging message.

- Conclusion

On a basis of above analysis, we make the following proposals regarding A-IoT paging:

Reader visibility of paging identifier based on the post meeting email

- The paging identifier can be in the form of filtering criteria to filter one or multiple devices, which cannot be associated to a specific AS ID assigned for one device for the further command service.

- The reader needs to know the association between the assigned AS ID and the upcoming command request towards a specific device, where the reader being informed of the paging identifier does not solve the issue.

- RAN2 to send LS to inform SA2 and RAN3 about the requirement at the reader to associate between the assigned AS ID and the upcoming command request toward a specific device. Wait for their solution(s) on such issue.

Format

- The possible MAC PDU format of A-IoT paging message contains:

- Message type to indicate paging;

- Transaction ID;

- MSG1 resource allocation information;

  - The number of slots for the paging round (e.g., Q value in RFID);

  - Resource allocation for the paging round, details up to RAN1;

The exact form (whether the two parts can be combined) is FFS

- Paging identifier (as an optionally-present upper layer container).

- RAN2 to confirm the R2D trigger message is purely for timing calibration in slotted-aloha CBRA scheme, i.e., at reception of this message, the device has the locally-maintained slot counter minus 1.

- Adopt Way-1 (i.e., introducing an explicit message type for R2D trigger) for the format of R2D trigger message.

Detailed device behavior on receiving a paging message

- The A-IoT device responses to a currently-received paging message, if:

- it is paged by the message; and the message is an initial paging message;

- it is paged by the message; the message is a subsequent paging message; and the last access procedure is failure.

- The A-IoT device identifies the currently-received paging message is:

- an initial paging message, if the message includes a transaction ID different from the one in the last-received paging message.

- a subsequent paging message, if the message includes a transaction ID same as the one in the last-received paging message.

- Reference

- Report of 3GPP TSG RAN WG2 meeting#129

- R2-2502211	Email discussion report: [POST129][035][AIoT] Paging	Qualcomm

- TS 23.369		Architecture support of Ambient power-enabled Internet of Things

- Annex

6.2.3	Procedure for Command

Editor’s Note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor’s Note:	Alignment is required for how to document and describe Direct Path and Indirect Path options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

Figure 6.2.3-1: Command Procedure

<skipped the irrelevant part>

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT Command request with an appropriate cause code. The AIOTF performs AIoT RAN and optionally AIoT RAN Reader selection as specified in clause 5.3. If no AIoT RAN or AIoT RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

Editor’s Note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

5.	AIOTF sends the Naiotf_AIoT_ Command Response message (accept or reject result for the AIoT Command service operation request) to the NEF.

6.	NEF sends the Nnef_AIoT_ Command Response message (accept or reject result for the AIoT Read/Write/PermanentDisable service operation request) to the AF.

Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed.

Editor's note:	It is FFS whether the AIoT RAN performs aggregation for Inventory Report during the Command procedure.

Editor's note:	It is FFS how to handle the scenarios that the Inventory Response received for Command Request sent for many individual devices are in parallel.

Editor's note:	Further clarification and enhancement on step 7 to step 11 of procedure for Inventory specified in clause 6.2.2 is FFS.

For Direct path as shown in clause 4.2.2.2, step 12a is performed.

12a.	AIOTF sends Command Request message (correlation identifier, NAS Command Request) to the AIoT RAN. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

For Indirect path via AMF as shown in clause 4.2.2.3, step 12b and 12c are performed.

12b.	The AIOTF sends Namf_AIoT_MessageDelivery (AIoT RAN ID, Command Request message (correlation identifier, NAS Command Request)) to AMF. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request..

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

12c.	The AMF sends the Command Request message (correlation identifier, NAS Command Request)) to AIoT RAN.

<skipped the irrelevant part>