3GPP TSG-RAN WG2 Meeting #129bis   	       	R2-2501813

Wuhan, China, 7th – 11th, April 2025

Source:	vivo

Title:	Random Access Procedure for A-IoT Device

Agenda Item:	8.2.3

Document for:	Discussion and Decision

# Introduction

In the last RAN2 meeting, random access procedure of A-IoT device was discussed and some agreements were achieved as follows [1]:

Agreements

- For Rel-19, only 3-step CBRA is supported for A-IoT

- We will specify both CBRA and CFRA.

- Re-use the subsequent paging message to trigger re-access.  There is no need to differentiate msg1 resource for initial access vs re-access.

Agreements

- NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback.

- RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

In this contribution, we continue to discuss A-IoT random access procedure, which includes failure handling and Msg format for CBRA and CFRA.

# Discussion

## 2.1 3-step CBRA

Failure handling

In the following figures, we give our understanding of 3-step CBRA procedures and examples of some failure cases.

- Basic CBRA with 3-step RA type

| - Failure case detected by device | - Failure case detected by device |
| --- | --- |
| - Failure case indicated by reader | - Failure case indicated by reader |

In 3-step CBRA, there are two kinds of failure: contention resolution failure and transmission failure. Failure of Msg1 and Msg2 are contention resolution failure. These two steps are by default considered failure if the expected ACK confirmation is not received timely. For instance, once a device transmits its Msg1 with RN16, it could receive a common NACK of Msg1 if the reader fails to decode Msg1 correctly because of poor link quality or collision or nothing detected. From device perspective, both common NACK of Msg1 and failure to receive its RN16 within a specific time window or a predetermined time, e.g., until next R2D trigger/access slot starts, signify a failure in contention resolution failure, which can either be identified by the device itself or indicated by reader.

Certainly, upon determination of contention resolution failure, the device should maintain continuous R2D monitoring, as the reader may re-try Msg2 transmission again immediately or re-trigger a new paging round at any moment.

- Failure of contention resolution is determined on the device side when:

- The device does not receive its RN16 same as Msg1 timely, e.g. until next R2D trigger/access slot start (baseline), or;

- Receiving common NACK of Msg1.

Once the device successfully receives Msg2 with confirmation of its own RN16, which means contention resolution is successful, all subsequent D2R transmissions will be considered successful unless the reader explicitly indicates otherwise. If the reader is unable to receive Msg3, Msg5, or any other D2R messages, it might send an explicit NACK with a device specific RN16/AS ID to indicate a failure. This new MAC NACK will trigger device re-access. Alternatively, the reader may transmit the R2D message again to trigger the corresponding D2R message re-attempt, which is transparent to the device, and it can be considered as a new message transmission and left to reader implementation.

- Introduce a MAC NACK PDU with device’s RN16/AS ID for D2R transmission (i.e., including Msg3, Msg5 and other D2R message) failure indication and to trigger the device re-access.

A recovery procedure could be initiated if Msg2 or Msg3 reception is unsuccessful. If the reader is unable to detect the expected Msg3 correctly in the Msg3 resource location, the reader may interpret this as a failure in receiving either Msg2 or Msg3 reception failure. In addition to NACK indication and re-access triggering, the reader has the option to retry Msg2 transmission to recover Msg2 or Msg3 reception failures. The corresponding device behaviours might involve the device transmitting Msg3 upon receiving Msg2 again. The content of new Msg2 may be reorganized, for instance, by only including information of the failed device.

- The reader can retry a new Msg2 transmission that only includes the failed device’s RN16 to recover from Msg2 or Msg3 reception failure.

D2R MAC PDU format

For Msg1 format of the 3-step CBRA, the most essential content is the field of 16-bit RN16. Whether the Message type field for Msg1 is needed or not can be analysed further. Firstly, the resource occasion of Msg1 is exactly determined, and there is no uncertainty. The Message type field in Msg1 is unnecessary for indicating and differentiating it from other D2R messages. The only impact of including the Message type field in Msg1 could be to maintain a consistent D2R format. Nonetheless, considering overhead and efficiency, it is preferable to maintain lean design. We prefer to include RN16 field only in CBRA Msg1.

Figure 1 Format of Msg1 for 3-step CBRA

- The content of Msg1 for 3-step CBRA includes 16-bit RN16 field only.

For Msg3 format of 3-step CBRA, the most essential content is the field of the upper layer payload. The upper layer payload could be Device ID (e.g., EPC ID) for Msg3 or command reply for Msg5 and other D2R messages. Due to the variable size of the upper layer payload, a length field is also needed to indicate either the whole/total size or the segment size of the upper layer payload, especially if RAN1 concludes that the TB size of the scheduled D2R resource cannot always match the MAC PDU size, meaning that padding may be necessary. The length field can either be 7 or 8 bits long, as the upper layer payload’s maximum size is approximately 1000 bits, which need 7-bit length to indicate in bytes and 8 bits is preferred for the purpose of byte alignment. Furthermore, the decision of whether D2R MAC PDU should include Message type field depends on the need to distinguish between Msg3 and other D2R messages explicitly by format indicator or procedure timeline. In our understanding, reader can judge the PDU type by the procedure timeline. Hence, message type field is not necessary. Furthermore, if scheduling D2R resource is larger than the Msg3/D2R PDU size, padding may be needed and put in the end of PDU without any indicator.

Figure 2 MAC PDU format of Msg3

- The content of Msg3 for 3-step CBRA may include the following fields:

        - Upper layer payload: variable size;

        - Length: 8 bits;

        - Padding: if any, in the end of PDU without indicator.

R2D MAC PDU format

For the Msg2 format of the 3-step CBRA, the most essential content is the confirmation field of 16-bit RN16. Furthermore, it is up to reader implementation to decide if a new AS ID should be assigned to the device in Msg2. An optional AS ID field might also be required. For R2D messages, a Message type field is mandatory, for instance, to distinguish between Paging message (Msg0), Msg2 and other R2D messages. The size of the Message type can be 4 bits for all R2D message types and any future extension. If Msg2 multiplexing is supported based on the final decision, there could be at least two types of the Msg2, e.g., for a single device and for multiple devices. The different code point of Message type field or a separate bit may be used to distinguish these two Msg2 types.

Figure 3 Msg2 format for single device

Figure 4 Msg2 multiplexing format for 4 ROs case

- The content of Msg2 for 3-step CBRA may include the following fields:

        - Message type: 4 bits for R2D messages differentiation;

        - A or Ax: presence indicator for the corresponding AS_ID field, e.g., 0 means not presence and 1 means presence. Ax is only for multiplexing case;

        - Nx: presence indicator for the corresponding RN_16 field, e.g., 0 means not presence and 1 means presence. The order of Nx is the same as the one of Msg1 ROs/resources. Nx is only for multiplexing case;

        - RN_16: 16-bit number and same as Msg1;

        - AS_ID: 16-bit number and optionally present, i.e. allocated by reader if necessary;

        - Potential Msg3 resource field: up to RAN1 progress.

There might be two kinds of NACK indications for the R2D direction, namely a common NACK for Msg1 or a specific NACK for an individual device. These NACK indications should have separate code point(s) in the Msg type field to distinguish them from other R2D messages. A specific NACK for an individual device should also include the RN16 or AS_ID of that device. Different code points of Message type field or a separate bit can be used to distinguish between these two NACK formats.

Figure 5 MAC PDU format for common NACK

Figure 6 MAC PDU format of NACK for single device

- The content of NACK indication for 3-step CBRA may include the following fields:

        - Message type: 4 bits;

        - RN_16 or AS_ID: 16 bits and only for specific NACK indication to single device.

## 2.2 CFRA

Failure handling

In the following figures, we give our understanding of basic CFRA procedure and some special cases.

| - Basic CFRA with inventory only | - CFRA with subsequent command |
| --- | --- |

In a basic CFRA procedure, there's no need for failure or success indication of Msg1 because initiating a new CFRA procedure is more preferred when the previous Msg1 is not successfully received by the reader. The failure indication of CFRA Msg1 has an unclear function and ambiguity regarding the subsequent behaviors of the device. A new CFRA trigger is more beneficial for re-attempt and potentially with updated resource or parameters allocation, which increase the probability of success in the subsequent retry. Furthermore, the device must consistently reply with Msg1 for duplicated CFRA paging/Msg0 since the previous one remains unconfirmed in inventory-only case, i.e., AS ID is not allocated.

- ACK or NACK indication for CFRA Msg1 is redundant and not necessary.

- Failure indication for CFRA Msg1 is not supported since the device may receive a new CFRA Msg0 triggering when the previous Msg1 is not received successfully by reader.

- Device always responds Msg1 for duplicated CFRA paging/Msg0 since the previous one is not confirmed successfully in inventory only case, i.e. AS ID is not allocated.

D2R MAC PDU format

For Msg1 of CFRA, the most essential content is the field of upper layer payload, e.g. EPC ID. Based on similar reasons with Msg1 of CBRA, Msg1 resource of CFRA is dedicated and exactly determined. Hence, the field of Msg type is not needed. Considering the variable length attribute of EPC ID, length field may be needed and 8 bits is proposed.

Figure 7 Msg1 format for CFRA

- The content of Msg1 for CFRA may include the following fields:

        - Upper layer payload: variable size;

        - Length: 8 bits;

        - Padding: if any, in the end of PDU without indicator.

Other D2R messages may reuse the same format with CBRA (segmentation is not considered in this document and our segmentation solution can be found in another document [2]), e.g. as shown in Figure 2.

R2D MAC PDU format

For the Msg2 format of CFRA, we prefer option 4 of AS ID allocation for CFRA, meaning the AS ID is assigned to the device in Msg2 with the initial R2D command and no RN16 in Msg1, which is the most efficient. If there is a need to confirm the device ID in Msg2, we prefer a truncated ID from the upper layer payload in Msg1, e.g., the 16-bit LSB of the upper layer payload field in Msg1, which is a little similar to the contention resolution ID in Msg4 for legacy Uu 4-step RACH. AS ID is allocated by the reader in command case. The length of the upper layer payload may not be necessary since the R2D transport block is ended by the post-amble design, which is up to the RAN1 conclusion.

Figure 8 Msg2 format for CFRA

- The content of Msg2 for CFRA may include the following fields:

        - Message type: 4 bits;

        - Truncated ID in Msg1: the 16-bit LSB of upper layer payload field in Msg1 if device needs identification in Msg2;

        - AS ID: 16 bits and allocated by reader for command case;

        - Upper Layer Payload: variable size.

# Conclusion

In this contribution, we discuss further details about CBRA and CFRA and have the following observations:

- ACK or NACK indication for CFRA Msg1 is redundant and not necessary.

Based the above discussions and observations, we have the following proposals:

Failure handling for 3-step CBRA

- Failure of contention resolution is determined on the device side when:

- The device does not receive its RN16 same as Msg1 timely, e.g. until next R2D trigger/access slot start (baseline), or;

- Receiving common NACK of Msg1.

- Introduce a MAC NACK PDU with device’s RN16/AS ID for D2R transmission (i.e., including Msg3, Msg5 and other D2R message) failure indication and to trigger the device re-access.

- The reader can retry a new Msg2 transmission that only includes the failed device’s RN16 to recover from Msg2 or Msg3 reception failure.

D2R MAC PDU format for 3-step CBRA

- The content of Msg1 for 3-step CBRA includes 16-bit RN16 field only.

- The content of Msg3 for 3-step CBRA may include the following fields:

- Upper layer payload: variable size;

- Length: 8 bits;

- Padding: if any, in the end of PDU without indicator.

R2D MAC PDU format for 3-step CBRA

- The content of Msg2 for 3-step CBRA may include the following fields:

- Message type: 4 bits for R2D messages differentiation;

- A or Ax: presence indicator for the corresponding AS_ID field, e.g., 0 means not presence and 1 means presence. Ax is only for multiplexing case;

- Nx: presence indicator for the corresponding RN_16 field, e.g., 0 means not presence and 1 means presence. The order of Nx is the same as the one of Msg1 ROs/resources. Nx is only for multiplexing case;

- RN_16: 16-bit number and same as Msg1;

- AS_ID: 16-bit number and optionally present, i.e. allocated by reader if necessary;

- Potential Msg3 resource field: up to RAN1 progress.

- The content of NACK indication for 3-step CBRA may include the following fields:

- Message type: 4 bits;

- RN_16 or AS_ID: 16 bits and only for specific NACK indication to single device.

Failure handling for CFRA

- Failure indication for CFRA Msg1 is not supported since the device may receive a new CFRA Msg0 triggering when the previous Msg1 is not received successfully by reader.

- Device always responds Msg1 for duplicated CFRA paging/Msg0 since the previous one is not confirmed successfully in inventory only case, i.e. AS ID is not allocated.

D2R MAC PDU format for CFRA

- The content of Msg1 for CFRA may include the following fields:

- Upper layer payload: variable size;

- Length: 8 bits;

- Padding: if any, in the end of PDU without indicator.

R2D MAC PDU format for CFRA

- The content of Msg2 for CFRA may include the following fields:

- Message type: 4 bits;

- Truncated ID in Msg1: the 16-bit LSB of upper layer payload field in Msg1 if device needs identification in Msg2;

- AS ID: 16 bits and allocated by reader for command case;

- Upper Layer Payload: variable size.

- Reference

- RAN2#129, Chairman report;

- R2-2501814, AIoT Data Transmission, vivo, RAN2#129bis;