3GPP TSG-RAN WG2 Meeting #129bis	R2-2501824

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.3.2.2

Source:	OPPO

Title:	Discussion on model generalization of RRM measurement prediction

Document for:	Discussion, Decision

Introduction

In RAN2#129, the generalization parameters of cell configurations have been agreed. FR1 is the primary focus of this study with two sets of configurations as in the below table.

| Parameter | Configuration #1 | Configuration #2 |
| --- | --- | --- |
| Deployment scenario | UMi | UMa |
| ISD | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

In addition, we take 30km/h as UE speed for FR1. This paper will study the impact of different cell configurations on generalization.

Discussion

Generalization of FR1 temporal domain case B

In this section, we studied the generalization performance of FR1 temporal domain case B with sub case 3 with non-sliding filtering. The table below compares the performance of baseline and generalization models in terms of average L3 cell-level RSRP difference across the two different cell configurations (CCs). As suggested by the rapporteur, 30km/h for FR1 is evaluated.

Table 1: performance of baseline and generalization models in FR1 temporal domain case B

|  | Baseline |  | GC#1 |  | GC#2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Training dataset | CC#1 | CC#2 | CC#1 | CC#2 | CC#1 + CC#2 |  |
| Testing dataset | CC#1 | CC#2 | CC#2 | CC#1 | CC#1 | CC#2 |
| Training data size | 20K |  |  |  | 40K (20K + 20K) |  |
| Testing data size | 3K |  |  |  | 3K |  |
| Average L3 cell-level RSRP difference (dB) | 1.01 | 0.74 | 0.78 | 1.02 | 0.98 | 0.75 |

CC#1 appears to present a more challenging scenario, as the prediction errors are higher compared to CC#2. Specifically, when using CC#1 as the testing dataset, the results show an approximate degradation of 0.25 dB compared to those obtained with CC#2.

From Table 1, it is evident that when tested on datasets with the same cell configuration, the performance of the baseline, GC#1, and GC#2 models are comparable. For instance, when tested on CC#2, the average L3 cell-level RSRP differences are 0.74 dB for the baseline, 0.78 dB for GC#1, and 0.75 dB for GC#2. These results indicate that all models perform similarly well under consistent testing conditions.

Observation 1: For intra-frequency temporal domain case B in FR1, the generalization models perform well across different cell configurations

Observation 2: For intra-frequency temporal domain case B in FR1, the generalization performance of GC#1 and GC#2 are comparable when cell configuration of the testing dataset is the same.

These results suggest that the generalized models can effectively handle variations in cell configurations, providing robust performance across multiple scenarios.

Conclusion

In this contribution, we have shared our simulation results on model generalization of RRM measurement prediction and have the following observations:

Observation 1: For intra-frequency temporal domain case B in FR1, the generalization models perform well across different cell configurations

Observation 2: For intra-frequency temporal domain case B in FR1, the generalization performance of GC#1 and GC#2 are comparable when cell configuration of the testing dataset is the same.