3GPP TSG-RAN WG2 Meeting #129bis	R2-2501825

Wuhan, China, April. 7th– 11th, 2025

Agenda Item:	8.3.5

Source:	OPPO

Title:	Discussion on LCM issues of AI mobility

Document for:	Discussion, Decision

Introduction

The evaluation on AI mobility use case comes to end slowly and it is time to discuss the potential spec impact of those use cases. In this paper, two high level issues are focused:

1, The potential scope to be discussed in the coming 3 meetings for study on spec impact

2, What is potential spec impact especially on LCM procedures

Scope

Which Use cases?

From the very beginning of the study, HOF is down prioritized i.e. basically nothing about HOF prediction is studied. So, HOF should be dropped for spec impact study.

RAN2 studied simulation assumptions for RLF case, but the evaluation is postponed twice. Eventually RAN2 decide not to study it anymore in the rest RAN2 meetings during study phase. Without evaluation it is difficult to be motivated to study spec impact also.

During RAN#107 there is exercise to figure out rough working scope for Rel20 normative work. In the last summary [1], the scope for AI mobility in slide 11 show no HOF and RLF will be specified. Instead RRM measurement prediction and measurement event prediction are listed for normative work. In RAN2 these two use cases are evaluated sufficiently, and relevant observations are captured in the TR.

Proposal 1: RAN2 study potential spec impact only on RRM measurement prediction and Measurement event prediction

Proposal 2: HOF and RLF prediction are dropped for spec impact study

UE sided model vs network sided model

In the SID scope, both UE sided model and network sided model are covered. While in the past 5 meetings, the evaluation doesn’t differentiate between UE sided model and network sided model. However, the spec impact for UE sided model and network sided model is obviously different.

For better comparison between UE sided model and network sided, we need assume that the performance of UE sided model and network sided model is comparable for the exact same scenario. We believe this is reasonable considering the input to the model is L1 or L3 measurement results only. Some other input parameter e.g. UE location is also considered by some company for UE sided model. But it is very difficult to evaluate the additional impact since they are up to UE’s implementation. On the other hand, network could have some knowledge e.g. topology of the network etc. Due to the same reason, we need put them aside at this stage.

Proposal 3: RAN2 assume that the performance of UE sided model and network sided model is comparable for the same scenario for the comparison between UE sided model and network sided model

If this basic assumption is agreeable, the justification is needed for network sided model against UE sided model or vice versa.

.

Figure 2.2-1 UE sided model				Figure 2.2-2 Network sided model

As illustrated in Figure 2.2-1 and 2.2-2, regardless UE sided or network sided model, what UE does is exactly the same. And the difference between UE sided model and network sided model is mainly signalling overhead over Uu interface, which could be reflected in following table:

| Model type | Content | How often |
| --- | --- | --- |
| UE sided model | L3 measurement results | Legacy periodical measurement report could be baseline |
| Network sided model | L1 or L3 measurement results | Depends on model’s requirement e.g. whether sliding filtering approach is adopted or not. |

Table 2.2-1 RRM measurement prediction

| Model type | Content | How often |
| --- | --- | --- |
| UE sided model | L3 measurement event | When event is triggered or predicted |
| Network sided model | L1 or L3 measurement results | Depends on model’s requirement e.g. whether sliding filtering approach is adopted or not. |

Table 2.2-2 Measurement event prediction

In Table 2.2-2 the signalling overhead is obviously very high for network sided model for measurement event prediction compared to UE sided model. Furthermore, the SID also makes it clear that measurement event prediction is a use case for UE sided model only. On the other hand, network can still derive measurement event based on the measurement report from UE by deploying AI/ML algorithm. For example, UE report historical and/or future measurement results to network periodically and network can check whether a measurement event could be triggered or not in future based on intra-frequency temporal domain case A. But that should be network’s implementation and hence out of scope.

Proposal 4: Measurement event prediction is only supported for UE sided model.

For RRM measurement prediction we need further investigate details. For UE sided model, UE can report the actual/predicted measurement results via periodic measurement report or event triggered periodic measurement report. For network sided model, to performance inference, the only way for UE to provide input measurement results is to report them periodically. It means UE sided model has signalling overhead advantage at least for event triggered periodic report procedure. In addition, RRM measurement prediction is basis for measurement event prediction, at least for indirect methodology, which will be supported anyway by UE sided model.

Proposal 5: RRM measurement prediction is supported by UE sided model

For periodic measurement report procedure there is difference between UE sided model and network sided model. For analysis purpose for intra-frequency temporal domain case A, it is assumed measurement period is 400ms and sampling period is 80ms. The OW and PW could be both 800ms. Table 2.2-3 show there is no difference between UE sided model and network sided model if the measurement report contains cell level measurement results. This is mainly because how often the output of model is produced is aligned with input. But if L1 beam level measurement results are reported for network sided model, then obviously network sided model bears much more signalling overhead because there could be more than one beams in one cell to be reported.

| Model type | Sliding approach | Non-sliding approach |
| --- | --- | --- |
| UE sided model | One MR (L3) per 80ms | One MR(L3) per 400ms |
| Network sided model | One MR (L1 or L3) per 80ms | One MR (L1 or L3) per 400ms |

Table 2.2-3 Temporal domain case A

For intra-frequency temporal domain case B, it is assumed measurement period is 200ms and sampling period is 40ms. MRRT could be 50%. Table 2.2.4 shows network sided model can save 50% signalling overhead. For UE sided model, UE need report both predicted and/or actual measurement results while for network sided model only partial (50% in this example) measurement result. This is true by assuming L3 measurement result is reported for network sided model. But If L1 beam level measurement results need be reported, then this advantage will quickly disappear because it is likely that more than one beams need be reported.

| Model type | Sliding approach | Non-sliding approach |
| --- | --- | --- |
| UE sided model | One MR(L3) per 40ms | One MR(L3) per 200ms |
| Network sided model | One MR (L1 or L3) per 80ms | One MR (L1 or L3) per 400ms |

Table 2.2-4 temporal domain case B

For frequency domain prediction, same assumption for temporal domain case B is taken.

| Model type | Sliding approach | Non-sliding approach |
| --- | --- | --- |
| UE sided model | Two MR(L3) per 40ms | Two MR(L3) per 200ms |
| Network sided model | One MR (L1 or L3) per 40ms | One MR (L1 or L3) per 200ms |

Table 2.2-5 frequency domain prediction

Again, if L3 measurement result is reported, network sided model demands 50% less signalling overhead due to the fact that measurement results of only one frequency carrier need be reported for network while UE need report two frequency carriers. But if L1 beam level measurement results are reported, then UE sided model is obviously better for the same reason as for temporal domain case A and case B.

Observation 1: the signalling overhead is much higher for network sided model if L1 beam level measurement result is reported to gNB compared to UE sided model

Observation 2: if L3 measurement result is reported, here is outcome of comparison between UE sided model and network sided model:

| Scenario | conclusion |
| --- | --- |
| Intra-frequency temporal domain case A | same signalling overhead |
| Intra-frequency temporal domain case B | Less signalling overhead for Network side model |
| Frequency domain prediction | 50% signalling overhead for Network side model |

Table 2.2-6

For network sided model it is clear only sub use case 2 demands UE to report L3 measurement result since the input is L3 measurement result while sub case 1 and 3 demands L1 measurement results. Based on analysis above we have following proposal:

Proposal 6: Only RRM sub case 2 is supported for network sided model

Which sub use case?

For RRM prediction there are 3 sub use cases [2] i.e.:

3 sub-use cases are considered for cell-level RRM measurement prediction:

Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

RAN2 concluded that for temporal domain case A, sub-case 2 outperform sub case1 and 2. But for temporal domain case B and frequency domain, there is no consensus at all. It means 3 sub cases should be all studied at least for temporal domain case B and frequency domain prediction.

Proposal 7: For UE sided model, all 3 sub cases should be studied

Which domain?

For RRM measurement prediction, following scenarios are identified with different priority:

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Table 5.2.1-1 in TR [12]

The simulation exercise focus on scenario 2,3,4 i.e. scenarios with high priority. So far evaluation on RRM measurement prediction shows that AI outperforms non-AI for all those 3 scenarios with different extend.

Observation 3: temporal domain case A, case B and frequency domain outperform non-AI

The performance gain in system level for temporal domain case A shows HO failure rate is reduced compared to legacy L3 HO procedure. For temporal domain case B, RAN2 conclude that minor or even no HO performance degradation is observed. For frequency domain prediction, RAN2 also conclude that F1 score is good without SLS result. It can be expected that the HO performance based on frequency domain prediction could be also acceptable.

Observation 4: System level performance is good in general for temporal domain case A, temporal domain case B and frequency domain.

Scenario 6 i.e. spatial domain prediction serves the same study goal with scenario 2. So, if the additional spec impact due to scenario 6 is marginal, it can be also considered. However, we think scenario 2 should be studied first and RAN2 can check whether scenario 6 need be supported in later stage.

Plus, temporal domain and frequency domain can be supported for both UE sided model and network sided model. But spatial domain should not be supported for network sided model because sub-case 2 is not applicable for spatial domain.

Proposal 8: To study following RRM measurement prediction for both UE sided model and network sided model:

Scenario 2: Intra-frequency temporal domain case B

Scenario 3: Inter-frequency domain prediction

Scenario 4: Intra-frequency temporal domain case A

Proposal 9: Spatial domain can be studied for UE sided model after intra-frequency temporal domain case B is concluded.

L3 beam level prediction

So far RAN2 focus on cell level measurement prediction during evaluation exercise. Only few companies provide simulation results for L3 beam level measurement prediction. Due to lack of evaluation, it is not well justified to study this case. On the other hand, it can be expected the spec impact for L3 beam level measurement prediction could be similar to cell level measurement prediction. RAN2 should focus on study on cell level measurement prediction. Once the study is concluded, RAN2 can further check whether the additional spec impact is marginal and decide whether they should be specified in normative work also.

Proposal 10: RAN2 focus on study of cell level RRM measurement prediction

Proposal 11: L3 beam level measurement prediction can be considered after the study on cell level RRM measurement prediction is concluded

Direct vs indirect prediction

During evaluation of measurement event prediction, both direct and indirect prediction methodology are discussed and captured. It seems indirect prediction is more popular since majority company provide simulation result based on indirect prediction. On the other hand, RAN2 also concluded that F1 score is good for direct prediction.

If we interpret that this is internal methodology for UE to predict measurement event, there should be no difference between direct and indirect prediction because from network perspective UE need transfer predicted measurement event in one measurement report message. From this regard, it looks like UE’s implementation. On the other hand, the model between direct and indirect is totally different. For example, indirect prediction can adopt to different measurement events very well because derivation of measurement event is based on predicted and/or actual measurement result. But it is not the case for direct prediction. It means a UE capable indirect prediction may not be able to do direct prediction.

Our view is that RAN2 can proceed with study on spec impact by assuming indirect prediction at first. Once RAN2 comes to conclusion, it can be checked whether anything additional is needed for direct prediction. Then at that point of time, we can see whether they need be differentiated or not. If yes, what else is needed.

Proposal 12: RAN2 study spec impact by assuming indirect prediction at first.

Proposal 13: RAN2 come to check direct prediction after conclusion is achieved for indirect prediction.

Proposal 14: By taking all the observations and proposals into account, here is summary of study scope:

|  | UE sided model | Network sided model |
| --- | --- | --- |
| Measurement event prediction | Yes, focus on indirect | No |
| Temporal domain case A | Yes, all sub cases | Yes, Only sub-case2 |
| Temporal domain case B | Yes , all sub cases | Yes , Only sub-case2 |
| Frequency domain | Yes , all sub cases | Yes , Only sub-case2 |
| Spatial domain | Pending | No |
| L3 beam level measurement prediction | Pending | No |

Table 2.5-1 scope of spec impact study

LCM issues

Functionality based LCM

The definition of term “LCM” in TR 38.843[4] mixes the functionality and model among LCM procedures e.g. “Functionality/model selection, activation, deactivation, switching, and fallback operation”. During Rel19 RAN1 and RAN2 discussion, it was concluded that sort of LCM is based on functionality i.e. the granularity for management by the system is per functionality instead of specific model(s). For AI Mobility, we don’t find any reason why we should not follow this principle i.e. the LCM for AI Mobility should be also functionality-based LCM.

Proposal 15: To adopt functionality-based LCM for AI mobility

RAN2 agreed some general statement about functionality as following in RAN2#127:

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

Table 3.1-1

These definitions are general enough to be applicable also for AI mobility functionalities apart from the yellow part.

Proposal 16: The definition (green part) in table 3.1-1 is applicable for AI mobility.

In the LS to RAN1 [5] RAN2 is wondering what could be supported functionalities, applicable functionalities and activated functionalities for beam management in terms of granularity. The reason for RAN2 to check RAN1 is mainly because BM (beam management use case) is physical layer procedure, for which RAN1 is of expertized. For AI mobility, obvious RAN2 need define further granularity.

Proposal 17: RAN2 need further discuss the detail granularity of AI mobility functionality

In the RAN2 LS [5], the LCM signalling procedure is well summarized with following figure

Figure 3.1-1[5]

In the rest of paper, those 6 steps will be discussed one by one. Two important procedures are not indicated in the Figure, namely data transfer and model delivery. These issues are either controversial or related to on-going two-sided model for BM use case in RAN side SI [7] or WI [8]. In addition, these two issues are part of Rel20 SA2 SI and hence likely will be specified only in Rel20, if possible. So, in AI mobility SI, we can drop them for now and wait for the further progress of either RAN side WI or SI and/or SA2 SI.

Proposal 18: In AI mobility SI, no discuss is needed on dataset transfer and model delivery issue until some progress made in AI/ML WI/SI and/or SA2 SI.

In the rest part of the paper, the agreements captured in running stage 2 CR[6] in post email discussion [POST129][024][AI PHY] Stage 2 running CR (vivo) are taken as reference for comparison. And in order to avoid duplicated discussion between AI/ML SI[7] and WI[8] and AI mobility SI, RAN2 focus on concluded aspects first to find out where is the gap. For any FFS left from SI[7] or WI[8], this SI should do nothing but wait for the further progress in general unless this SI specific issue need be discussed.

Proposal 19: RAN2 focus on concluded aspects by AI/ML WI [8] and SI [7] and discuss potential gaps and solutions

Proposal 20: For any FFS left from SI[7] or WI[8], this SI should do nothing but wait for the further progress in general unless this SI specific issue need be discussed

Note in the rest of the paper Section 3.2 is for both network sided model and UE sided model, section 3.3~3.6 focus on UE sided model only and section 3.7 focus on network sided model specific issues.

Data collection

In the running CR, here is agreed text [6]:

X.Y.2.2	Data Collection for Model Training

For NW-side model, model training, data collection can be initiated by OAM (via immediate MDT) or by gNB. The following enhancements are introduced for training data collection over air interface:

-	UE can be configured by gNB to log and report L1 measurement results.

Editor's note: FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support.

-	Both periodic and radio condition-based event-triggered data logging are supported. UE can use L3 measurement event-triggered to determine whether to perform data logging. The UE stores the logged data at the AS layer buffer. When the memory reserved for storing logged data becomes full, the UE stops measurement and logging for data collection purposes, and UE indicates the data availability to the network.

Editor's note: FFS if other event(s) for data logging are supported.

Editor's note: FFS when UE reports the indication, e.g., before and/or after buffer become full.

Editor's note:FFS other availability indication trigger condition

-	To reduce UE power consumption, when low power state is detected, the UE can indicate the low power state to the network. Upon reception of the low power status indication, the network should de-configure UE to release the data collection configuration.

NOTE 1:	How the UE determines low power state is up to UE implementation.

-	The UE can indicate the availability of logged data to the network. Upon reception of the availability indication, the network can trigger the UE information procedure, as specified in TS38.331[12]. During handover the UE can retain the logged data and the UE can indicate the availability of logged data to network.

Editor's note: FFS how to handle idle/inactive and RLF cases and whether we have a unified handling.

For UE-side model, the network can configure whether UE is allowed to initiate a request about data collection configuration (e.g., start/stop). The network can also provide UE with data collection configuration or release the data collection configuration (at any point in time), with or without UE request.

Editor's note: FFS whether an indication from UE to network is needed when UE cannot perform data collection based on received configuration.

Table 3.2-1 stage 2 text proposal on data collection for BM use case

In the table 3.2-1, the green part is kind of general principle for data collection for either UE-sided model or network-sided model. Since AI mobility will also support both UE sided model and network sided model, these green parts can be also applicable for AI mobility.

Proposal 21: The green highlighted part text proposal in table 3.2-1 is applicable for AI mobility data collection.

The yellow part is related to detail configuration from gNB can configure for data collection. Since AI mobility SI will cover both L1 and/or L3 measurement result for mobility purpose rather than L1 measurement result for beam management, obviously this part is not applicable.

Proposal 22: RAN2 need discuss how gNB configure data collection for network sided model and UE sided model for AI mobility functionalities. Detail is FFS.

Even same framework as BM will be followed, the logged and reported content for AI mobility functionality is different, which should be discussed in this SI.

Proposal 23: RAN2 need discuss what UE logs locally and reports to gNB by following gNB’s configuration. and Detail is FFS.

UE capability

There is not much progress made on UE capability aspect since for BM use case this is RAN1’s expertise. There is one agreement made in RAN2#125bis:

Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

Table 3.3-1 TP on UE capability for BM use case

This general statement is also applicable for AI mobility functionality.

Proposal 24: The green text in table 3.3-1 is applicable for AI mobility functionality

Then in order to deliver UE capability, RAN2 need define what is the detail AI mobility functionality.

In the endorsed TR 38.744 [2], two study goals are captured:

The use cases in this study focus on RRC_CONNECTED mode and cover RRM measurement prediction, measurement event prediction and RLF/HOF prediction for PCell change procedure in standalone NR scenario. The study of the use cases is driven mainly by two study goals. The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements. The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption)

Table 3.3-2 study goals of AI mobility SI

The 1st goal is achieved by RRM measurement prediction by saving some of the measurement efforts. The 2nd goal is achieved by predicting measurement event or measurement results in advance. So obviously these are two different functionalities because they serve different purposes. The question is whether finer granularity is necessary. UE can save measurement in temporal, spatial or frequency domain prediction. For temporal domain and spatial domain, they are very close in the sense they serve the same purpose but in different way. It seems not so necessary to separate them further. But frequency domain can also help relax measurement gap configuration and hence improve user throughput indirectly. Because of this frequency domain prediction should be a separate functionality.

For measurement event prediction, regardless direct and indirect prediction methodology, this should be separate functionality compared from RRM measurement prediction. For indirect prediction obvious its prerequisite is RRM measurement prediction in future window. For direct prediction there is no such limitation. Those detail can be studied further once RAN2 comes to high level conclusion.

Proposal 25: To support following functionalities for LCM discussion:

Functionality 1: Intra-frequency RRM measurement reduction

Functionality 2: RRM measurement prediction in frequency domain

Functionality 3: Intra-frequency RRM measurement prediction in future window

Functionality 4: Measurement event prediction*

* Relationship between this and RRM measurement prediction is FFS.

Applicability procedure

The running CR [6] also capture the agreements on functionality applicability. For AI mobility the CR is examined for more detail aspects i.e. inference configuration, consistency between UE and network, applicability signalling procedure and activation/deactivation.

Inference configuration

The relevant text in running CR is:

The network can provide inference configuration associated with NW-side additional conditions to UE via CSI report configuration.

Obviously, this is not applicable for AI mobility. For RRM measurement prediction and measurement event prediction, UE is still performing a measurement task but with some enhancement. For temporal domain case A, UE perform the measurement exactly as legacy procedure. On top of that some measurement results in future window can be predicted. For temporal domain case B, UE skipped partial measurement in temporal domain by predicting some of them. But when UE perform actual measurement, there is no difference is foreseen compared to legacy procedure. For frequency domain prediction, the measurement on f1 (carrier to be actually measured) is exactly the same as legacy measurement procedure. On top of that measurement result of f2 (carrier to be predicted) can be produced with help of AI algorithm. Having said that we can conclude that inference configuration for RRM measurement should follow existing measurement configuration framework with potential new parameters. The measurement event prediction is more or less built on top of RRM measurement, so same conclusion is applied.

Proposal 26: Legacy RRM measurement configuration framework is reused as baseline for inference configuration for both RRM measurement prediction and measurement event prediction. FFS what is enhancement to enable AI algorithm in UE side.

Additional condition of network

There is an associated ID from gNB as additional condition of network to check functionality applicability. Our understanding is that this is related to the generalization study i.e. two models need be associated with an associated ID if they can’t be generalized with each other.

So far for AI mobility there is minor problem for UE speeds. There is generalization issue for frequency domain prediction, but simulation results also show that it can be resolved by training with mixed data set. It means so far there is no such consistency issue between UE and network. Nevertheless, the generalization is still ongoing for two set of cell configurations for either FR1 or FR2. RAN2 can come back to this issue once the evaluation is completed.

Proposal 27: Once generalization on cell configuration is completed, RAN2 can come back to additional condition issue once again.

Applicability signalling procedure

The relevant TP for BM is here:

UE can indicate its supported functionalities to the Network via UE capability information. For UE-side model, UE can decide the applicable AI/ML functionalities based on e.g., NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in the UE.

The network can provide inference configuration associated with NW-side additional conditions to UE via CSI report configuration. And the UE can explicitly report its initial functionality applicability in RRCReconfigurationComplete message. When the network has configured applicability reporting via UEAssistanceInformation message for a functionality, the and applicability of the functionality changes, the UE can explicitly report updated functionality applicability in UEAssistanceInformation message.

…

During handover, the target gNB can provide UE with NW-side additional conditions and/or the inference configuration related to the target gNB via handover command. Then the UE can report its applicable/non-applicable AI/ML functionalities to the target gNB after the handover completion.

Table 3.4.3-1 TP on applicability procedure of BM use case

TP in table 3.4.3-1 is about applicability procedure for BM use case. The green text is sort of general principle and hence they can be applied for mobility also. In the yellow part, as indicated in section 3.4.2 it is FFS whether any additional condition is needed for AI mobility, and they can be left open before generalization study is concluded. As for the way to configure inference for AI mobility use case, it is proposed in section 3.4.1 that updated measurement configuration should be taken as baseline.

Proposal 28: The green text in table 3.4.3-1 is applicable for AI mobility functionalities

There is big debate for BM use case whether full configuration or partial configuration should be configured in step 3 as illustrated in Figure 3.1-1. Option A i.e. full configuration in step 3 is agreed at RAN2#129 meeting while in this meeting further discussion on option B i.e. partial configuration in step 3 will continue.

Proposal 29: Whether full or partial configuration is needed in step 3 in Figure 3.1-1 can be discussed after progress is made for generalization study.

Activation and deactivation

The TP for activation and deactivation for BM is here:

When provided with periodic CSI configuration consistent with reported UE capabilities in CSI report configuration, upon reporting the applicable functionalities, the UE autonomously activates the applicable AI/ML functionalities. When provided with semi-persistent CSI and/or aperiodic CSI configuration, upon reporting the applicable AI/ML functionalities, applicable AI/ML functionality activation follows the CSI measurement and reporting, i.e., semi-persistent reporting can be activated by MAC CE/DCI and aperiodic CSI reporting can be activated by DCI.

When an activated AI/ML functionality becomes inapplicable, the UE does not autonomously deactivate it. Upon reception of UE indication of the functionality becoming non-applicable, the network should deactivate or release this activated functionality.

Table 3.4.4-1 TP for (de)activation for BM use case

The idea of the green part is that periodic CSI configuration is always configured by RRC message and UE applies it immediately just like other RRC parameters. Hence if the inference is enabled by periodic CSI configuration, relevant functionality should be activated immediately.

If RAN2 can agree that updated measurement configuration is the baseline for inference configuration, the same principle can be applied. In addition, it is possible that more measurement tasks are linked to the same functionality, so detail should be further studied.

Proposal 30: AI mobility functionality is activated when full inference configuration via RRC message is received. Detail is FFS.

The 2nd paragraph addresses the issue how does UE react when an activated functionality becomes inapplicable. For BM use case, RAN2 agreed that UE doesn’t autonomously deactivate it. Instead, UE should report the situation to gNB quickly so that network can take further action. If the UE is close to gNB, handshake between UE and gNB is not a big deal. But if UE is close to cell edge, then there could be negative impact on mobility. So this should be studied further.

Proposal 31: FFS on UE’s behaviour when AI mobility functionality is not applicable anymore

Inference operation

The inference operation for UE sided model refer to the report procedure once inference activated and on-going i.e. to report measurement results or measurement event after inferencing to gNB.

In general, we think legacy measurement report framework can be taken as baseline for discussion i.e. it is not necessary to introduce brand new report message/procedure to do the job. But it is possible some enhancement is needed to e.g. improve the Uu signaling efficiency.

Proposal 32: Legacy measurement report framework is taken as baseline for UE to report measurement result or measurement event predicted by AI mobility functionality. FFS on enhancement detail.

For RRM measurement prediction on cell level result, UE should report only L3 measurement result. For sub-use case 1, although the outcome of the model is L1 beam level measurement result. But UE should still report L3 result to network since the L3 result instead of L1 result is wanted by network. Plus, reporting L1 beam level measurement results could be a big burden for Uu signalling since most likely there are multiple L1 beams need be reported.

Proposal 33: For RRM measurement prediction, only L3 measurement results are reported.

For measurement event report, the handover model captured in the TR is evaluation purpose. Then 1st question is which one should be specified? For HO model option 1 in the TR (i.e. option 2 in previous meeting’s context) how to report measurement event? For handover model option 2 in TR (i.e. option 3 in previous meeting’s context), UE need report to gNB when entry condition is met. The simulation results [9] also show there is also early handover risk. Then the question is that whether UE is allowed to wait a bit longer to transmit such message? For temporal domain case B, what could be additional assistance information apart from legacy measurement event information?

Proposal 34: To report measurement event, some open issues should be discussed:

- Which handover model should be specified for temporal domain case A?

- For HO model option 1 or 2 in the TR, how to report predicted measurement event?

- For HO model option2 in the TR, is UE allowed to wait a bit longer after entry condition is met to avoid early HO risk?

- For HO model option 3 (temporal domain case B or frequency domain prediction), how to report predicted measurement event?

Monitoring procedure

Here is the TP on monitoring for BM:

X.Y.2.4	Monitoring and Management

For UE-side model, the network can initiate performance monitoring and can make management decisions based on the performance monitoring results.

Table 3.6-1 TP on monitoring for BM use case

For UE sided model the green text is general enough to be applicable.

For RRM measurement prediction, in general monitoring procedure could have two options w.r.t. who calculate the metrics:

Option 1: UE provided label information i.e. ground truth measurement result and predicted measurement result so that gNB can calculate prediction accuracy

Option 2: UE calculate prediction accuracy based on ground truth measurement result and predicted measurement result and provide the outcome to gNB

Because predicted measurement result will be anyway reported to the network, so the additional signalling overhead for monitoring procedure is the ground truth measurement result. In option 2 UE need provide calculated metric instead of ground truth measurement result. Th prediction accuracy is kind of statistics information i.e. the metric should be calculated within preconfigured duration and a bunch of ground truth measurement results are needed for such calculation. So, from signalling overhead perspective option 2 is better.

Proposal 35: For RRM measurement prediction, network initiate metrics calculation in UE side

For measurement event prediction, the first issue is what metric need be monitored? For indirect prediction methodology, prediction accuracy can already reflect the performance of the model. For direct prediction methodology, intermediate KPI i.e. F1 score and/or its components can reflect performance of the model more or less. But it is bit tricky for UE to calculate F1 score. During evaluation the ground truth is gathered via legacy HO procedure in advance. In field deployment UE may not be able to know anything about ground truth. For example, for temporal domain case A, handover may be triggered based on predicted measurement event before a real measurement event is triggered. For temporal domain case B and frequency domain prediction a good model is expected to predict a measurement event closely around a ground truth measurement event. Considering there is duration between reporting measurement event and handover execution, it is possible for UE to get ground truth measurement event.

Observation 5: For measurement event prediction based on temporal domain case A, it is possible that ground truth measurement event will never happen during monitoring.

Observation 6: For measurement event based on temporal domain case B and inter-frequency prediction, good model can expect ground truth measurement event occurring around predicted measurement event

One alternative for monitoring F1 score is to monitor system level performance e.g. HO failure rate. It means UE need collect handover history across serving cells. The detail can be studied by RAN2.

Proposal 36: For measurement event monitoring, RAN2 need discuss whether intermediate metric e.g. F1 score or system level KPI e.g. HO failure rate should be monitored

Proposal 37: If system level KPI is monitored, monitoring KPI across serving cells need be studied

Network sided model specific issue

For LCM procedures in the Figure 3.1-1, for network sided model pretty much majority of them can be left to gNB implementation, apart from following aspect:

- Data collection (already covered by section 3.2)

- Inference related procedure

- Monitoring procedure

As indicated in section 2.2 network sided model need UE to provide measurement result as input to model. If only RRM sub case 2 need be studied UE need report L3 measurement results to network. there is possibility that legacy procedure may work well. But there is also possibility that some enhancement is needed. Our view is that we can take legacy measurement report procedure as baseline and further check whether any enhancement is needed.

Proposal 38: Legacy measurement report procedure is baseline for UE to provide measurement result for inference operation in network sided model. FFS on enhancement.

For monitoring procedure, here is the TP on BM use case:

X.Y.2.4	Monitoring and Management

For NW-side model, the gNB is responsible for performance monitoring. UE will not be involved in management decisions (e.g., selection, (de)activation, switching etc.) of gNB-side model, except for being configured to provide the required measurement/data. Additionally, the UE will not be informed about any gNB-side management decision.

Table 3.7-1 TP on monitoring for BM use case

The green text is general principle for network sided model and could be applicable also for AI mobility functionality.

Proposal 39: The green text in table 3.7-1 is applicable for AI mobility network sided model

For RRM measurement prediction to monitor model’s performance what network need is label information i.e. ground truth L3 measurement result. In general, there are two steps are needed:

- Step 1: network configure UE when and how to report ground truth measurement result

- Step 2: UE report ground truth measurement result

Step 1 will be triggered only when needed. And UE need know some assistant information e.g. UE need know the time duration and target frequency/cell etc.

In step 2 since ground truth measurement result is L3 measurement result, technically legacy measurement report message can be taken as baseline. But more information may be needed. For example, for frequency domain prediction the timing relationship of the measurement result between f1 (carrier to be measured) and f2 (carrier to be predicted) should be clear so that gNB can link the input and output measurement result together.

Proposal 40: gNB need configure UE with some assistant information for UE to provide ground truth measurement result. Detail is FFS.

Proposal 41: Legacy measurement report message can be used as baseline to report ground truth measurement results. FFS further enhancement is needed.

Conclusion

We have following observations and proposals:

Observation 1: The signalling overhead is much higher for network sided model if L1 beam level measurement result is reported to gNB compared to UE sided model

Observation 2: if L3 measurement result is reported, here is outcome of comparison between UE sided model and network sided model:

| Scenario | conclusion |
| --- | --- |
| Intra-frequency temporal domain case A | same signalling overhead |
| Intra-frequency temporal domain case B | Less signalling overhead for Network side model |
| Frequency domain prediction | 50% signalling overhead for Network side model |

Observation 3: temporal domain case A, case B and frequency domain outperform non-AI

Observation 4: System level performance is good in general for temporal domain case A, temporal domain case B and frequency domain.

Observation 5: For measurement event prediction based on temporal domain case A, it is possible that ground truth measurement event will never happen during monitoring.

Observation 6: For measurement event based on temporal domain case B and inter-frequency prediction, good model can expect ground truth measurement event occurring around predicted measurement event

Proposal 1: RAN2 study potential spec impact only on RRM measurement prediction and Measurement event prediction

Proposal 2: HOF and RLF prediction are dropped for spec impact study

Proposal 3: RAN2 assume that the performance of UE sided model and network sided model is comparable for the same scenario for the comparison between UE sided model and network sided model

Proposal 4: Measurement event prediction is only supported for UE sided model.

Proposal 5: RRM measurement prediction is supported by UE sided model

Proposal 6: Only RRM sub case 2 is supported for network sided model

Proposal 7: For UE sided model, all 3 sub cases should be studied

Proposal 8: To study following RRM measurement prediction for both UE sided model and network sided model:

- Scenario 2: Intra-frequency temporal domain case B

- Scenario 3: Inter-frequency domain prediction

- Scenario 4: Intra-frequency temporal domain case A

Proposal 9: Spatial domain can be studied for UE sided model after intra-frequency temporal domain case B is concluded.

Proposal 10: RAN2 focus on study of cell level RRM measurement prediction

Proposal 11: L3 beam level measurement prediction can be considered after the study on cell level RRM measurement prediction is concluded

Proposal 12: RAN2 study spec impact by assuming indirect prediction at first.

Proposal 13: RAN2 come to check direct prediction after conclusion is achieved for indirect prediction.

Proposal 14: By taking all the observations and proposals into account, here is summary of study scope:

|  | UE sided model | Network sided model |
| --- | --- | --- |
| Measurement event prediction | Yes, focus on indirect | No |
| Temporal domain case A | Yes, all sub cases | Yes, Only sub-case2 |
| Temporal domain case B | Yes , all sub cases | Yes , Only sub-case2 |
| Frequency domain | Yes , all sub cases | Yes , Only sub-case2 |
| Spatial domain | Pending | No |
| L3 beam level measurement prediction | Pending | No |

Proposal 15: To adopt functionality-based LCM for AI mobility

Proposal 16: The definition (green part) in table 3.1-1 is applicable for AI mobility.

Proposal 17: RAN2 need further discuss the detail granularity of AI mobility functionality

Proposal 18: In AI mobility SI, no discuss is needed on dataset transfer and model delivery issue until some progress made in AI/ML WI/SI and/or SA2 SI.

Proposal 19: RAN2 focus on concluded aspects by AI/ML WI [8] and SI [7] and discuss potential gaps and solutions

Proposal 20: For any FFS left from SI[7] or WI[8], this SI should do nothing but wait for the further progress in general unless this SI specific issue need be discussed

Proposal 21: The green highlighted part text proposal in table 3.2-1 is applicable for AI mobility data collection.

Proposal 22: RAN2 need discuss how gNB configure data collection for network sided model and UE sided model for AI mobility functionalities. Detail is FFS.

Proposal 23: RAN2 need discuss what UE logs locally and reports to gNB by following gNB’s configuration. and Detail is FFS.

Proposal 24: The green text in table 3.3-1 is applicable for AI mobility functionality

Proposal 25: To support following functionalities for LCM discussion:

- Functionality 1: Intra-frequency RRM measurement reduction

- Functionality 2: RRM measurement prediction in frequency domain

- Functionality 3: Intra-frequency RRM measurement prediction in future window

- Functionality 4: Measurement event prediction*

* Relationship between this and RRM measurement prediction is FFS.

Proposal 26: Legacy RRM measurement configuration framework is reused as baseline for inference configuration for both RRM measurement prediction and measurement event prediction. FFS what is enhancement to enable AI algorithm in UE side.

Proposal 27: Once generalization on cell configuration is completed, RAN2 can come back to additional condition issue once again.

Proposal 28: The green text in table 3.4.3-1 is applicable for AI mobility functionalities

Proposal 29: Whether full or partial configuration is needed in step 3 in Figure 3.1-1 can be discussed after progress is made for generalization study.

Proposal 30: AI mobility functionality is activated when full inference configuration via RRC message is received. Detail is FFS.

Proposal 31: FFS on UE’s behaviour when AI mobility functionality is not applicable anymore

Proposal 32: Legacy measurement report framework is taken as baseline for UE to report measurement result or measurement event predicted by AI mobility functionality. FFS on enhancement detail.

Proposal 33: For RRM measurement prediction, only L3 measurement results are reported.

Proposal 34: To report measurement event, some open issues should be discussed:

- Which handover model should be specified for temporal domain case A?

- For HO model option 1 or 2 in the TR, how to report predicted measurement event?

- For HO model option2 in the TR, is UE allowed to wait a bit longer after entry condition is met to avoid early HO risk?

- For HO model option 3 (temporal domain case B or frequency domain prediction), how to report predicted measurement event?

Proposal 35: For RRM measurement prediction, network initiate metrics calculation in UE side

Proposal 36: For measurement event monitoring, RAN2 need discuss whether intermediate metric e.g. F1 score or system level KPI e.g. HO failure rate should be monitored

Proposal 37: If system level KPI is monitored, monitoring KPI across serving cells need be studied

Proposal 38: Legacy measurement report procedure is baseline for UE to provide measurement result for inference operation in network sided model. FFS on enhancement.

Proposal 39: The green text in table 3.7-1 is applicable for AI mobility network sided model

Proposal 40: gNB need configure UE with some assistant information for UE to provide ground truth measurement result. Detail is FFS.

Proposal 41: Legacy measurement report message can be used as baseline to report ground truth measurement results. FFS further enhancement is needed.

Reference

[1] RP-250812 Summary for RAN Release 20 5G-Adv, RAN Vice Chairs, RAN1, RAN2, RAN3, RAN4 Chairs

[2] R2-2500287 Text proposal on TR 38.744

[3] RP-242393 Revised SID on AIML for mobility in NR.docx, OPPO

[4] TR 38.843 v18.0.0

[5] R2-2407848 LS on applicable functionality reporting.docx, RAN2

[6] R2-2501806 Introduction of AI/ML for NR Air interface feature in TS38.300

[7] RP‑250308 Revised SID for Study on AI/ML for NR air interface phase 2, Qualcomm

[8] RP‑250792 Revised WID on AI/ML for NR air interface

[9] R2-2500640	Simulation results for measurement event prediction and system level performance	Samsung