3GPP RAN WG2 Meeting #129bis                                                     	R2-2501830

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.2.4

Source:	HONOR

Title:	Discussion on Data Transmission for Ambient IoT

Document for:	Discussion and Decision

1. Introduction

In the RAN#106 meeting, a new Work Item for Ambient IoT(A-IoT) has been approved in RAN plenary [1] and RAN2 has made the below agreements regarding A-IoT data transmission and other general aspects in RAN2#129[2]:

AS ID assignment

Agreements - For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. - From device perspective, it is only required to use one AS ID. - CFRA is not supported for group ID - RAN2 assumes, AS ID is needed for CFRA at least for inventory + command procedure - For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: - Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. - Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2 - Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2

Segmentation

- Agreements on segmentation

- To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size.   If not possible, FFS if the 1 bit is sufficient.

- Segment retransmission is supported.

- For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

- R2D segmentation is not supported for R19 A-IoT.

Procedures

- Agreements

- From RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”.

MAC PDU format

Agreements on MAC PDU format design - Aim to design simple MAC PDU format design - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages - At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). - FFS whether for D2R we need message type field, any length and need for padding - Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

In this contribution, we provide our considerations on the aspects of data transmission functionality and other remaining general aspects, such as AS ID handling, segmentation and MAC PDU format.

2 Discussion

2.1 AS ID Assignment

Gains of AS ID could be observed and it has been agreed to be applied for CFRA at least for “inventory and command” scenario. AS ID applying for the first command is considered as an important factor on the AS ID assignment.

For “inventory” case, MSG3 will be transmitted to the reader and the size of MSG3 could be more than 100bits which may be segmented. Similarly, AS ID applying for MSG3 could bring the same gains as applying for command.

On the other hand, the failure handling of MSG3 is under discussion and one possible solution is introducing a R2D message for NACK. Compared with device ID, AS ID could be much better to identify the device for failure handling.

The AS ID size may be 16bits which could support more than 60000 devices and it is enough to assign the ID to devices triggered in one inventory procedure, no matter there is following command or not.

Proposal 1: AS ID could be applied for Inventory only case.

To enjoy more benefits, the device should acquire the AS ID at the beginning of the data transmission procedure. MSG2 is the best option to assign the AS ID applying for the first command. Meanwhile, AS ID could be reused random ID included in MSG1 and it is naturally to indicate the reusing of random ID in MSG2 since MSG2 will echo MSG1 with the random ID.

Proposal 2: For CBRA, AS ID should be assigned via MSG2.

Inventory procedure will always start with a paging message and the AS ID could be assigned to the device when it is triggered by the paging message. After completing the procedure, it is not necessary to store the AS ID for future service since there will be paging message to start the new procedure and AS ID can be assigned again if needed.

Observation 1: AS ID could be assigned to the device after triggered by a paging message and could be re-assigned if triggered again.

Paging round will star with paging message and end with the other paging message and there may be several slots in one paging round. Considering the characteristic of A-IoT devices, the reader should send the R2D message with command as soon as possible upon receiving the MSG3 to avoid unnecessary failure caused by energy depletion. Therefore, the whole procedure should be completed within brief period, e.g. one paging slot or paging round. If the command message cannot reach the device in time, the reader should re-trigger the device via subsequent paging and new AS ID could be assigned to the triggered device.

Observation 2: Considering energy consumption of A-IoT device, it is beneficial and feasible that the device could receive the command within a brief period, e.g. in the same paging slot or paging round with the slot/round sending MSG3. There is no need to apply the AS ID in the other round.

The device will release the AS ID upon power off and the reader cannot aware the AS ID invalidity status without device charging information. In this case, the AS ID will be released from device side, while not in the reader side. If the AS ID is valid across several paging rounds, the reader may schedule the device with the AS ID in the other round and it will be considered as service failure as there is no response. Therefore, it will be better to restrict the AS ID will be valid in one paging round and the AS ID should be re-assigned in the new round to avoid the failure case.

Observation 3: Assignment of AS ID after triggered by paging could avoid inconsistent understanding of AS ID validity between reader and device.

It is possible that the device may missing any R2D messages, including paging or new message for AS ID release. From device side, the message missing may lead to AS ID conflict with the other devices, i.e. multiple devices with one AS ID. The conflict issue could be resolved in upper layer because the upper layer packet carries the identity information of the target device. For example, device A will interpret the R2D data in upper layer and detect it is not a message for itself or decryption error and discard the message without feedback. Device B will construct the D2R message with upper layer feedback and transmission with the allocated resource.

Observation 4: Missing of paging message may lead to AS ID conflict and it could be resolved in upper layer because the upper layer packet carries the identity information of the target device.

Proposal 3: AS ID is valid in one paging round, i.e. the AS ID will be released upon receiving the other paging.

2.2 Segmentation

It has been agreed that the CN node could not always provide the estimated expected D2R message size to AIoT RAN node depicted as below [3].

Observation 5: The CN node will not always provide the estimated expected D2R message size.

Generally, the expected D2R message size is an important factor for the resource allocation. Without the size information, the reader may allocate more resources for a small D2R message which will lead to resource wastage, and unnecessary service failure of other A-IoT devices queued for data transmission, e.g. energy depleted. Or the reader may allocate less resource for the device with large D2R data and the feedback will be divided into several segments with multiple-times transmission. This may increase the probability of service failure of the device, e.g. loss of packets, energy depleted.

Observation 6: Without D2R message size information, the reader can’t allocate resource accurately which may increase the A-IoT service failure probability.

A 1-bit indication is considered to indicate whether there is more data or not. However, the reader could only know whether there is D2R data left for transmission and still does not know how to allocated resource without D2R message size information.

Observation 7: A 1-bit indication is not enough for reader to make appropriate scheduling decision if the reader could not know the estimated expected D2R message size.

Proposal 4: The device should be able to report the information about expected D2R message size.

Device could get the accurate upper layer packet size after receiving the “Inventory” or “Command” from CN. There could be two options for the device to report the D2R message size information.

Option1: Report accurate data packet size

Considering the upper layer data packet size will always be less than 1000 bits, the expected D2R message size could occupy nearly 10bits (units: bit) or 7bits (units: byte). It is beneficial to report only once to reduce D2R message overhead, e.g. reporting with the first segment. The reporting size could be the whole message size including the segment in this D2R MAC PDU or the remaining message size left for transmission.

However, this will increase device complexity as the device should decide whether to report the size information and select corresponding MAC PDU formats.

Option2: Report the level of packet size

In option2, the device could indicate the D2R packet size within a packet size range which is quite similar with BSR scheme in NR. The bits occupied depends on how the range is partitioned. For the message with maximum 1000bits, 2bits or 3bits index for the packet size level is enough to indicate at most 4 or 8 levels.

As the overhead is relatively low, the device could report the indication in every D2R message and it will be set to “zero” if there is no data left for further transmission, which could also work for unsegment packet. This means unified D2R MAC PDU format and is quite simple for device.

Proposal 5.1: It is proposed RAN2 to consider the following two options for expected D2R message size information reporting:

- Option1: Report accurate data packet size, e.g with the first segment;

- Option2: Report the level of packet size.

Proposal 5.2: The indication of packet size range should be included in every D2R message including upper layer data.

In last meeting, it has been agreed to support segment retransmission with an offset in MAC layer. For the first segment or unsegmented packet lost case, it could also work with the offset sets to “zero”. In such scenarios, the device can uniformly handle retransmissions of the initial R2D message and subsequent R2D message for segment scheduling, with simplifying implementation.

Proposal 6: The offset (i.e. number of bits successfully received) in the MAC layer should also be used for unsegmented packet re-transmission.

The reader could allocate the resource depends on D2R message size (i.e., upper layer data), target coverage, device availability, etc. For MSG3 carrying the device ID, the length of the device ID is not fixed and the reader may not be able to allocate enough resource for device ID transmission. Therefore, segmentation of MSG3 is necessary if the TBS is less than device ID size.

Proposal 7: It is proposed to support segmentation of MSG3.

The segmentation re-transmission scheme is introduced to avoid resource wastage, energy consumption and unnecessary re-triggering of the A-IoT service. It is beneficial to apply to MSG3 transmission when segmentation happens.

Proposal 8: The offset (i.e. number of bits successfully received) in the MAC layer should be applied to re-transmit segments of MSG3.

2.3 MAC PDU Formats

2.3.1 MAC PDU Formats for R2D Data transmission

It has been agreed that the AS ID has benefits for scheduling and could be applied at least for “Inventory and command”. When AS ID is assigned, it should be included in the R2D MAC PDU to indicate the target device.

Observation 8: The AS ID should be included in the R2D MAC PDU if it is assigned.

The device needs to construct D2R MAC PDU based on the scheduling information from the reader. RAN1 is discussing the content of scheduling information and how to send this information to devices, e.g. via L1 or L2. If it should be sent via L2, the R2D MAC PDU will set the scheduling information field accordingly.

Observation 9: The scheduling information for D2R transmission may be included in R2D MAC PDU which depends on RAN1 conclusion.

As stated in chapter 2.2, it is feasible and beneficial to introduce offset (i.e. number of bits successfully received) in the MAC layer for unsegmented packet re-transmission to achieve low device complexity with consistent R2D MAC PDU format.

Observation 10: It is feasible and beneficial to apply the offset (i.e. number of bits successfully received) in the MAC layer for unsegmented packet retransmission.

The initial R2D message will include the upper layer data carried in the R2D data field, e.g. Command. When segmentation happens, the reader needs to scheduling the device for following segments transmission. There could be two options for the scheduling.

One option is scheduling the device via re-transmitting the initial message including R2D data field for upper layer data. However, this may result in unnecessary resource wastage for transmitting the upper layer data several times.

The other option is introducing a new message type, i.e. R2D MAC PDU for scheduling only, to schedule the device. Comparing the new type with initial R2D MAC PDU, their contents are nearly the same except without upper layer data.

Considering simple MAC PDU format design, it is preferable to define only one message type for the R2D MAC PDU for data transmission. And the optionally presence of R2D data field could avoid unnecessary upper layer data transmission. The R2D data will be present in initial R2D message and not present when the reader schedules the device for subsequent segment transmission.

Proposal 9: The R2D data field is an optionally field in the R2D MAC PDU for data transmission. And it should be not present if the R2D MAC PDU is used for scheduling the device for the subsequent segment transmission.

Based on the above observations and proposals, it is proposed that:

Proposal 10: Except MT and Length field, the R2D MAC PDU other than Paging and Msg2 includes:

- AS ID;

- Scheduling information (Depends on RAN1);

- Offset (i.e. number of bits successfully received);

- R2D data (Optional).

During RA procedure, there could be lots of devices triggered and MSG2 supports multiplexing of information for multiple devices. And then, the reader may also need to send command(s) to multiple devices if it received multiple MSG3. It is possible that the R2D message multiplexes information for multiple devices to trigger the D2R transmission simultaneously.  This could reduce the number of R2D message and scheduling devices with lower delay.

Except the initial R2D message, the following R2D message will include scheduling information for subsequent segment. Similar with MSG2, there could be multiple device waiting for scheduling information for segment transmission. It is efficient to multiplex the scheduling information of multiple devices. Therefore, it is proposed that the R2D message for scheduling should support multiplexing of multiple devices.

Proposal 11: The R2D message should be possible to multiplex information for multiple devices.

2.3.2 MAC PDU Formats for D2R Data transmission

Sensing and positioning are also target scenarios of A-IoT which could be discussed in the further version. And more D2R MAC PDU formats will be introduced for different scenarios. Considering extensibility, the Message Type should be included in the D2R MAC PDU.

Proposal 12: Message Type should be included in the D2R MAC PDU for further extension.

The device may be allocated more resources than it needs to transmit the whole upper layer data because the reader could not always know how many bits waiting for transmission. In this case, padding should be considered if the actual message size could not match the allocated resource. And the Length field is necessary to indicate the end of the D2R data or beginning of padding.

Proposal 13: Length should be included in the D2R MAC PDU.

Based on the above observations and proposals, it is proposed that:

Proposal 14: The D2R MAC PDU for MSG3 and data includes:

- Message Type;

- Length;

- MSG size information (OP or MP depends on how to report the information);

- D2R data.

3 Conclusions

In this contribution, we analyze the data transmission procedure for A-IoT device and made the following observations and proposals:

Observations

Observation 1: AS ID could be assigned to the device after triggered by a paging message and could be re-assigned if triggered again.

Observation 2: Considering energy consumption of A-IoT device, it is beneficial and feasible that the device could receive the command within brief period, e.g. in the same paging slot or paging round with the slot/round sending MSG3. There is no need to apply the AS ID in the other round.

Observation 3: Assignment of AS ID after triggered by paging could avoid inconsistent understanding of AS ID validity between reader and device.

Observation 4: Missing of paging message may lead to AS ID conflict and it could be resolved in upper layer because the upper layer packet carries the identity information of the target device.

Observation 5: The CN node will not always provide the estimated expected D2R message size.

Observation 6: Without D2R message size information, the reader can’t allocate resource accurately which may increase the A-IoT service failure probability.

Observation 7: A 1-bit indication is not enough for reader to make appropriate scheduling decision if the reader could not know the estimated expected D2R message size.

Observation 8: The AS ID should be included in the R2D MAC PDU if it is assigned.

Observation 9: The scheduling information for D2R transmission may be included in R2D MAC PDU which depends on RAN1 conclusion.

Observation 10: It is feasible and beneficial to apply the offset (i.e. number of bits successfully received) in the MAC layer for unsegmented packet retransmission.

Proposals

- AS ID Assignment

Proposal 1: AS ID could be applied for Inventory only case.

Proposal 2: For CBRA, AS ID should be assigned via MSG2.

Proposal 3: AS ID is valid in one paging round, i.e. the AS ID will be released upon receiving the other paging.

- Segmentation

Proposal 4: The device should be able to report the information about expected D2R message size.

Proposal 5.1: It is proposed RAN2 to consider the following two options for expected D2R message size information reporting:

-	Option1: Report accurate data packet size, e.g with the first segment;

-	Option2: Report the level of packet size.

Proposal 5.2: The indication of packet size range should be included in every D2R message including upper layer data.

Proposal 6: The offset (i.e. number of bits successfully received) in the MAC layer should also be used for unsegmented packet re-transmission.

Proposal 7: It is proposed to support segmentation of MSG3.

Proposal 8: The offset (i.e. number of bits successfully received) in the MAC layer should be applied to re-transmit segments of MSG3.

- MAC PDU formats

Proposal 9: The R2D data field is an optionally field in the R2D MAC PDU for data transmission. And it should be not present if the R2D MAC PDU is used for scheduling the device for the subsequent segment’s transmission.

Proposal 10: Except MT and Length field, the R2D MAC PDU other than Paging and Msg2 includes:

-	AS ID;

-	Scheduling information (Depends on RAN1);

-	Offset (i.e. number of bits successfully received);

-	R2D data (Optional).

Proposal 11: The R2D message should be possible to multiplex information for multiple devices.

Proposal 12: Message Type should be included in the D2R MAC PDU for further extension.

Proposal 13: Length should be included in the D2R MAC PDU.

Proposal 14: The D2R MAC PDU for MSG3 and data includes:

-	Message Type;

-	Length;

-	MSG size information (OP or MP depends on how to report the information);

-	D2R data.

4. References

[1] RP-243326

[2] RAN2 #129 meeting report

[3] TS 23.369 v0.2.0 Architecture support for Ambient power-enabled Internet of Things