3GPP RAN WG2 Meeting #129bis	R2-2501831

# Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.4.2

Source:	HONOR

Title:	LP-WUS in RRC_IDLE/INACTIVE

Document for:	Discussion and Decision

1. Introduction

In RAN#103 meeting, the following objectives of Rel-19 LP-WUS work item were agreed [1]:

- For IDLE/INACTIVE modes (RAN1, RAN2, RAN3, RAN4) - Specify procedure and configuration of LP-WUS indicating paging triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point.

The following agreements on LP-WUS monitoring conditions and subgrouping were made [2] in RAN2#129 meeting:

On separate band issue RAN2 understands that UE can report which band(s) is supported by LP-WUR to NW. RAN2 understands that any potential overload issues could be addressed by current mechanism in spec. Send LS to RAN1 and RAN4 to inform the agreements. On sub-grouping For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - UE_ID is related to 5G-S-TMSI, - N is the number of total paging frames in one DRX cycle, - Ns is the number of the PO for a PF, - Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively. On SA2 LS (R2-2500050/S2-2412876) RAN2 sends reply LS to SA2/RAN3/CT1, and CC RAN1 and RAN4 to confirm the “LP-WUS” terminology. Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.

In this contribution, we focus on the discussion of the following issues for LP-WUS in RRC_IDLE/INACTIVE.

- Configuration

- LP-WUS and emergency PDU session

- UE paging probability information

- Monitoring condition of LP-WUS

- LP-WUS capability

2. Discussion

2.1 Configuration

RAN1 defines two types of LP-WUR, e.g., OFDM-based LP-WUR and OOK-based LP-WUR. Some companies argue that gNB may need to enable both types of LP-WUS in one cell or configure the type of LP-WUS to adapt different types of LP-WUR. In our view, it is not necessary.

RAN1 believes that gNB only send OFDM-based LP-WUS. OOK-based UE could receive LP-WUS by the way of envelope detection, while OFDM-based UE could receive LP-WUS by the way of early identification. So, gNB does not need to configure the type of LP-WUS or enable both types of LP-WUS.

Proposal 1: gNB does not need to configure the type of LP-WUS or enable both types of LP-WUS.

2.2. LP-WUS and emergency PDU session

RAN2 agreed to FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

In our view, whether UE disables UE-ID based subgrouping when it has an emergency PDU session should depend on the additional paging latency.

For PEI, the additional paging latency can be determined by parameter pei-FrameOffset-r17, which is 0~160ms. The additional paging latency introduced by PEI is deemed tolerable. For LP-WUS, RAN1 has defined three candidate values for the wake-up delay that UE support, which are {[80ms], [500ms] and [900ms]}. Namely, the additional paging latency introduced by usage of LP-WUS are {[80ms], [500ms] and [900ms]}. Compared to additional paging latency introduced by PEI, at least the additional paging latency of 80ms should be deemed tolerable. The other two paging latency are much larger than those introduced by PEI, which are unacceptable.

In our view, when UE satisfies entry conditions and has an emergency PDU session, it can determine whether to disable UE-ID based subgrouping based on UE wake-up delay.

Proposal 2：UE can determine whether to disable UE-ID based subgrouping based on the wake-up delay it supports when it has an emergency PDU session.

2.3. UE paging probability information

Following agreement has been made in RAN2 127bis meeting.

For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion.

In Rel-17 PEI, it is specified that the UE may include its UE paging probability information in the Requested PEIPS assistance information IE if the UE has set the NR-PSSI bit to "NR paging subgrouping supported" in the 5GMM capability IE. The granularity of the paging probability is 5%.

TS 24.501

Table 9.11.3.80.1: PEIPS assistance information element

(omit)

Bit

5 4 3 2 1                        UE paging probability information value

0 0 0 0 0                                          p00

0 0 0 0 1                                          p05

0 0 0 1 0                                          p10

0 0 0 1 1                                          p15

0 0 1 0 0                                          p20

0 0 1 0 1                                          p25

0 0 1 1 0                                          p30

0 0 1 1 1                                          p35

0 1 0 0 0                                          p40

0 1 0 0 1                                          p45

0 1 0 1 0                                          p50

0 1 0 1 1                                          p55

0 1 1 0 0                                          p60

0 1 1 0 1                                          p65

0 1 1 1 0                                          p70

0 1 1 1 1                                          p75

1 0 0 0 0                                          p80

1 0 0 0 1                                          p85

1 0 0 1 0                                          p90

1 0 0 1 1                                          p95

1 0 1 0 0                                          p100

On the other hand, paging probability is usually less than 5% (e.g., 3% or 1% or <%1) in RAN1 simulation for LP-WUS power consumption. The reason is little power saving gain and even negative power saving gain may be introduced by LP-WUS if the UE paging probability is high due to the extremely high MR transition energy (15000uint ~ 40000uint) [3].

Obviously, the granularity of paging probability defined for PEI is not suitable for LP-WUS. RAN2 consider more finer granularity of paging probability (e.g., 1% or <1%) for reporting to CN, and then inform CT1 our conclusion.

Proposal 3: RAN2 consider more finer granularity of paging probability (e.g. 1% or <1%) for LP-WUS for reporting to CN.

2.4. Monitoring condition of LP-WUS

Frequent switching between LP-WUR and MR may happen when UE moves back and forth at the edge of LP-WUS coverage. As MR transition energy is huge (15000uint ~ 40000uint) [3], ping-pong between LP-WUR and MR may have a severe impact on UE power consumption. RAN2 needs to consider how to avoid ping-pong, e.g., add a hysteresis timer in entry condition.

Proposal 4: RAN2 discuss how to avoid ping-pong between LP-WUR and MR at the edge of LP-WUS coverage, e.g., add a hysteresis timer in entry condition.

The following agreement has been achieved in RAN1 118bis meeting.

- Confirm the following working assumption for iDRX: For each UE, the periodicity of LO is the same as its iDRX cycle

According to the working assumption, the following issue has been observed which is showed in the below figure.

For example, the wake-up delay supported by UE is 800ms, the I-DRX cycle configured by gNB is 640ms. When a wake-up indication received within LO1, UE would miss corresponding PO1 after it wakes up. It does not make sense to monitor LP-WUS in this scenario. It is proposed that if the wake-up delay supported by UE is larger than I-DRX cycle, UE should not monitor LP-WUS.

Proposal 5: If the wake-up delay supported by UE is larger than I-DRX cycle, UE should not monitor LP-WUS.

2.5. LP-WUS capability

RAN1 has agreed to report UE wake-up delay to gNB. In our view, UE wake-up delay can also be used to indicate that UE supports LP-WUS feature and UE_ID based subgrouping. Besides, due to UE LP-WUR may not support all the bands that MR supports, another capability that LP-WUS UE should report is the band lists that LP-WUR supports. These two capabilities can be reported to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message. Based on these two capabilities, gNB can determine whether to send LP-WUS to UE, as well as the time domain location of the LP-WUS.

Proposal 6: UE reports wake-up delay and band lists that LP-WUR supprts to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message.

Some companies believe that LP-WUR type should be reported to gNB so that gNB can send different LP-WUS type based on UE LP-WUR type. According to the analysis in Section 2.1, both OOK-based LP-WUR and OFDM-based LP-WUR can receive both type of LP-WUS correctly. It is not necessary to report LP-WUR type for paging.

Proposal 7: UE does not report LP-WUR type in UE capability message.

3. Conclusions

In this contribution, we discussed Configuration, LP-WUS and emergency PDU session, UE paging probability information, Monitoring condition of LP-WUS and LP-WUS capability. The proposals are summarised as below.

Proposal 1: gNB does not need to configure the type of LP-WUS or enable both types of LP-WUS.

Proposal 2：UE can determine whether to disable UE-ID based subgrouping based on the wake-up delay it supports when it has an emergency PDU session.

Proposal 3: RAN2 consider more finer granularity of paging probability (e.g. 1% or <1%) for LP-WUS for reporting to CN.

Proposal 4: RAN2 discuss how to avoid ping-pong between LP-WUR and MR at the edge of LP-WUS coverage, e.g., add a hysteresis timer in entry condition.

Proposal 5: If the wake-up delay supported by UE is larger than I-DRX cycle, UE should not monitor LP-WUS.

Proposal 6: UE reports wake-up delay and band lists that LP-WUR supprts to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message.

Proposal 7: UE does not report LP-WUR type in UE capability message.

4. References

[1] RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] Report of 3GPP TSG RAN WG2 meeting #129

[3] TR 38.869 Study on low-power wake up signal and receiver for NR