3GPP RAN WG2 Meeting #129bis                                                                                 R2-2501834

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.5.3

Source:	HONOR

Title:	Discussion on on-demand SIB1

Document for:	Discussion and Decision

- Introduction

In RAN2#129, the following aspects were agreed to be specified in the on-demand SIB1 work item [1]:

Agreements on OD-SIB1

1. There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

2. Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/INACTIVE UEs.

3. The UE follows the legacy validity principle of stored SIB.

4. For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

5. SIB-x can be cell specific or area specific, as legacy.

6. Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

In this contribution, we will analyse remaining issues of OD-SIB1 procedure, including contents of WUS configuration, and triggering conditions of UL-WUS.

- Discussion

2.1 Legacy UE bars the OD-SIB1 Cell

RAN2 has agreed to use K_SSB signaling in MIB to indicate legacy UE to bar OD-SIB1 cell without introducing other barring mechanisms in last meeting [1].

Agreement

There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

In RAN2#128, it is also agreed that the K_SSB value of NES cell will be set as K_SSB < 24 for FR1 or K_SSB < 12 for FR2 when the SIB1 of the NES cell is broadcasting [2].

Agreement

NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

However, legacy UE will interpret K_SSB < 24 for FR1 or K_SSB < 12 for FR2 as an indicator of CD-SSB and consider the cell as a non-NES cell (cell A).

Observation 1: Legacy UE will consider the cell as legacy cell if the K_SSB value is K_SSB < 24 for FR1 or K_SSB < 12 for FR2 and try to camp on the cell.

Since the SIB1 in the NES cell can only be broadcast within a specific time window, once this window expires, the cell will return to a NES cell with OD-SIB1. In this case, the legacy UEs may have to reselect another cell, as they are unable to initiate OD-SIB1 to obtain the SIB1 of the current serving cell.

To avoid continuous cell reselection, it is suggested that indication in MIB, e.g. cellbarred, could be adopted to prohibit legacy UE access when the NES cell is broadcasting SIB1 for OD-SIB1 request.  In the meantime, NES UEs should ignore this indication and access such NES cells normally.

Proposal 1: If SIB1 is broadcasting upon OD-SIB1 request, the NW should configure cellbarred as barred to prohibit legacy UEs access, while NES UEs ignores the cellbarred indicator.

2.2 Triggering conditions of UL-WUS

In RAN2#127bis meeting, RAN2 agreed that NES UEs should check the SIB1 broadcast status of a NES cell before they initiate OD-SIB1 procedure [3].

Agreement

If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Based on the above two agreements in 2.1, it can be inferred that when the NES cell is not broadcasting SIB1, the K_SSB value will be set as K_SSB > 23 for FR1 or K_SSB > 11 for FR2. This is also consistent with the conclusion of RAN1#118bis meeting [4].

Agreement For further work of on-demand SIB1 in idle/inactive mode, it is assumed that: - SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster. - UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A.

In a word, NES UEs can confirm the SIB1 broadcast status of NES cell through K_SSB value directly.

Therefore, the conditions for NES UEs to trigger UL-WUS can be clearly defined as follows: the NES UE should have a valid WUS configuration for the NES cell and confirm that the K_SSB value of the cell is K_SSB > 23 for FR1 or K_SSB > 11 for FR2.

Proposal 2: NES UE with valid SIB1 request configuration of a NES cell, confirming that the NES cell’s SSB contains K_SSB > 23 for FR1 and K_SSB > 11 for FR2, will trigger UL-WUS for OD-SIB1.

2.3 SI change procedure in NES Cell

In legacy spec, the network broadcasts the updated SIB1 in the next modification period (MP) after notifying UEs about changes in system information (SI). If no UEs are present during this broadcast, it results in wasted resources and energy. Therefore, to further achieve the goal of NES, the network will not actively broadcast the updated SIB1 after sending the SI change indication. Instead, the UE should trigger WUS transmission to request SIB1.

Proposal 3: After the NW sends the SI change indication in the NES cell, it will not actively broadcast the updated SIB1 unless a NES UE sends an OD-SIB1 request.

2.4 OD-OSI procedure in NES Cell

The UE should acquire the SIB1 for si-RequestResources/si-BroadcastStatus before sending the request for on-demand system information. If SIB1 is provided on-demand, the UE needs to trigger a Random Access (RA) process for SIB1 and then the other RA for the invalid OSI. This may result in frequent SIB1 requests, leading to significant network energy consumption. It is beneficial if the UE could trigger the OSI request without re-acquiring SIB1 to check the broadcasting status.

Proposal 4: In NES cell, the UE could trigger the request for OSI without re-acquiring SIB1 to check OSI broadcasting status.

To avoid incorrect mapping between si-RequestResources and SI message, network should configure the si-RequestResources for specific SIB/SI message instead of explicitly mapping based on broadcasting status to avoid incorrect MSG1-based request.  Alternatively, the network should not enable the MSG1-based OSI, which depends on network implementation.

Proposal 5: For OD-OSI in NES cell, the mapping between si-RequestResources and SI message should not be changed with broadcasting status of every OSI.

2.5 Contents of WUS configuration

In the NES cell, multiple UEs may initiate RA process in the same RACH Occasion (RO) for various reasons, such as initial access, OD-SIB1, and OD-OSI. In legacy spec, these UEs will receive the same MAC PDU for RAR which includes one or more MAC subPDUs constructed according to the data format depicted below [5]:

TS 38.321

MAC PDU (Random Access Response)

A MAC PDU consists of one or more MAC subPDUs and optionally padding. Each MAC subPDU consists one of the following:

-	a MAC subheader with Backoff Indicator only;

-	a MAC subheader with RAPID only (i.e. acknowledgment for SI request);

-	a MAC subheader with RAPID and MAC RAR.

A MAC subheader with Backoff Indicator consists of five header fields E/T/R/R/BI as described in Figure 6.1.5-1. A MAC subPDU with Backoff Indicator only is placed at the beginning of the MAC PDU, if included. 'MAC subPDU(s) with RAPID only' and 'MAC subPDU(s) with RAPID and MAC RAR' can be placed anywhere between MAC subPDU with Backoff Indicator only (if any) and padding (if any).

A MAC subheader with RAPID consists of three header fields E/T/RAPID as described in Figure 6.1.5-2.

Padding is placed at the end of the MAC PDU if present. Presence and length of padding is implicit based on TB size, size of MAC subPDU(s).

Therefore, UEs need to decode the MAC subPDUs one by one to confirm whether each RAR corresponds to their own request, based on the RAPID carried in the MAC subheader. If the RAPID is different from the preamble used by the UE to initiate RA, the UE needs to determine whether to skip 1 byte or 8 bytes to locate the next MAC subPDU, according to the parameter totalNumberOfRA-Preambles [6] in SIB1.

However, in the NES cell, when NES UEs try to receive OD-SIB1 RAR, they cannot obtain the totalNumberOfRA-Preambles through SIB1. In the discussion of the WUS configuration content by RAN1, considering the limited size of the WUS configuration, they also prefer not to provide totalNumberOfRA-Preambles in the WUS configuration. Therefore, when NES UEs decode the MAC PDU for RAR, they cannot determine the number of bytes that need to be skipped as mentioned above. This may cause them to fail to receive OD-SIB1 RAR.

To avoid this problem, OD-SIB1 RAR should not be transmitted together with other types of RAR in the same MAC PDU. A feasible method is to configure a separate RO or assign a common RNTI for OD-SIB1 reception in the WUS configuration.

Proposal 6: Configure a separate RO or assign a common RNTI for OD-SIB1 reception in the WUS configuration.

In addition, SIB1 also contains some necessary information for cell access, such as PLMN ID, cellbarred indication for UEs with different features, etc. The information will instruct the UE to confirm whether the current cell allows access and whether it needs to perform a cell reselection.

In NES cells, NES UEs that are barred by the networks must initiate the OD-SIB1 procedure to obtain SIB1 before confirming their access prohibition. For these UEs, the OD-SIB1 procedure is unnecessary. To prevent these UEs from attempting the OD-SIB1 procedure, the WUS configuration should provide the necessary cell access information. This allows NES UEs to directly confirm access permissions, reducing unnecessary energy consumption and resource waste while improving UE cell reselection efficiency.

Proposal 7: WUS configuration should provide some necessary cell access information, such as PLMN ID, cellbarred indication for UEs with different features, etc.

Besides, in RAN1#120 meeting, they have agreed that WUS configuration provides searchSpaceZero and controlResourceSetZero for on-demand SIB1 [7].

Agreement

For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster.

However, in RAN2’s opinion, K_SSB value of NES cell will be set as K_SSB < 24 for FR1 or K_SSB < 12 for FR2 when the SIB1 of the NES cell is broadcasting. At this time, the PDCCH-ConfigSIB1 in MIB will provide searchSpaceZero and controlResourceSetZero for SIB1 transmission as legacy.

Observation 2: If the K_SSB value could be K_SSB < 24 for FR1 or K_SSB < 12 for FR2 in NES cell, the PDCCH-ConfigSIB1 in MIB provides searchSpaceZero and controlResourceSetZero for SIB1 as legacy.

In this case, NES UEs can acquire searchSpaceZero and controlResourceSetZero through PDCCH-ConfigSIB1 in MIB. Namely, as long as the NES cell is broadcasting SIB1, the network will simultaneously update the K_SSB value and PDCCH-ConfigSIB1 in MIB to indicate OD-SIB1 reception. Conversely, if the NES cell is not broadcasting SIB1, the UE does not need to obtain the reception parameters for OD-SIB1. Therefore, considering the limited size of WUS configuration, it is suggested that UL WUS configuration could provide searchSpaceZero and controlResourceSetZero for OD-SIB1 optionally. If these parameters are not included in WUS configuration, the UE should obtain them from PDCCH-ConfigSIB1 in MIB.

Proposal 8: WUS configuration could provide searchSpaceZero and controlResourceSetZero for OD-SIB1 optionally.

Proposal 9: The UE should obtain parameters for OD-SIB1 acquisition, i.e. searchSpaceZero and controlResourceSetZero from PDCCH-ConfigSIB1 in MIB if they are not included in WUS configuration.

- Conclusions

In this contribution, we discussed several detailed on-demand SIB1 issues, and made the following observations and proposals:

Observation 1: Legacy UE will consider the cell as legacy cell if the K_SSB value is K_SSB < 24 for FR1 or K_SSB < 12 for FR2 and try to camp on the cell.

Proposal 1: If SIB1 is broadcasting upon OD-SIB1 request, the NW should configure cellbarred as barred to prohibit legacy UEs access, while NES UEs ignores the cellbarred indicator.

Proposal 2: NES UE with valid SIB1 request configuration of a NES cell, confirming that the NES cell’s SSB contains K_SSB > 23 for FR1 and K_SSB > 11 for FR2, will trigger UL-WUS for OD-SIB1.

Proposal 3: After the NW sends the SI change indication in the NES cell, it will not actively broadcast the updated SIB1 unless a NES UE sends an OD-SIB1 request.

Proposal 4: In NES cell, the UE could trigger the request for OSI without re-acquiring SIB1 to check OSI broadcasting status.

Proposal 5: For OD-OSI in NES cell, the mapping between si-RequestResources and SI message should not be changed with broadcasting status of every OSI.

Proposal 6: Configure a separate RO or assign a common RNTI for OD-SIB1 reception in the WUS configuration.

Proposal 7: WUS configuration should provide some necessary cell access information in the SIB1 of the NES cell, such as PLMN ID, cellbarred indication for UEs with different features, etc.

Observation 2: If the K_SSB value could be K_SSB < 24 for FR1 or K_SSB < 12 for FR2 in NES cell, the PDCCH-ConfigSIB1 in MIB provides searchSpaceZero and controlResourceSetZero for SIB1 as legacy.

Proposal 8: WUS configuration could provide searchSpaceZero and controlResourceSetZero for OD-SIB1 optionally.

Proposal 9: The UE should obtain parameters for OD-SIB1 acquisition, i.e. searchSpaceZero and controlResourceSetZero from PDCCH-ConfigSIB1 in MIB if they are not included in WUS configuration.

- References

- Draft_RAN2_129_Meeting_Report_v3

- R2-2500002 RAN2#128 Meeting Report	MCC report Late

- R2-2409502 RAN2#127bis Meeting Report MCC report

- R1-2409341 Report of RAN1#118bis meeting ETSI MCC

- TS 38.321 V18.3.0, Medium Access Control (MAC) protocol specification, 2024-09

- TS 38.331 V18.3.0, Radio Resource Control (RRC) protocol specification, 2024-09

- Draft_Minutes_report_RAN1#120_v010