3GPP TSG-RAN WG2 Meeting #129bis	R2-2501837

Wuhan, China, April 7th - 11th, 2025

Agenda Item:	8.6.4

Source:	HONOR

Title: 	Discussion conditional LTM

Document for:	Discussion and decision

1	Introduction

For the conditional LTM, UE evaluates conditions for triggering LTM and execute cell switch upon satisfying the event. RAN2#129 meeting come to several agreements as following.

Agreements on C-LTM:

1.	From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

2.	The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

3.	For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

4.	When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

5.	For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

6.	For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

7.	Do not support DG-based first UL transmission for RACH-less conditional LTM.

8.	For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

9.	For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

10.	Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

In this contribution, we discuss the general concept of the conditional LTM like execution trigger events and the early UL synchronization. Then we propose some solutions/way-forwards accordingly.

- Discussion

Early sync for CLTM

For Rel-18 LTM, the TA is carried on the LTM switch command to trigger the execution. Since the execution for the conditional LTM is triggered by UE when the L1 measurement results meet the pre-configured condition, the TA needs to be sent to the UE ahead to enable the RACH-less switch. RAN2 has agreed to introduce a new MAC CE to convey the TA of candidate cells. For this new MAC CE, at least the TA value and the information of the candidate cell should be sent to the UE. In addition, other related information like the TAT, TAG ID could also be considered to provide up-to-date configuration/information of this TA.

Proposal 1: RAN2 to discuss which info is needed in the new MAC CE for the early sync: TA value, configuration ID and TAG ID, TAT.

With the early UL synchronization designed in Rel-18, UE could perform the RACH-less LTM on the triggered candidate. To avoid signalling and power overhead resulted from frequent TA acquisition, TA could be maintained in the UE after pre-acquisition before the execution and UE could use the corresponding TA when executing the switch to the candidate if the TA of this candidate is still valid. Furthermore, whether the former received TA could be reused after the cell switch is another question and needs to be aligned between the UE and NW. If the TA could be maintained, the validity of the TA could be monitored based on the legacy mechanism like TAT and DL RSRP threshold.

Proposal 2: At the UE side, TA validity could also be monitored based on the legacy DL RSRP solution and/or the new timer for candidate’s TA.

During previous discussion, RAN2 already confirmed that the event triggered L1 measurement is designed for LTM early synchronization. This enables that the early synchronization could be obtained earlier than the cell switch by performing synchronization upon the satisfaction of an easily satisfied condition and executing the cell switch upon a stricter condition. The TA of a candidate cell is sent from the serving cell before the execution of the CLTM could enable the RACH-less LTM. However, the L1 measurement is performed by UE and sent to the NW by MR to provide the information to trigger the TA acquisition for the candidate cell with one/several good enough beams which may cause long latency. To increase the possibility of the RACH less, it is beneficial if the UE detects the TA is not valid and directly report the explicit information to the NW.

Proposal 3: UE could request the TA acquisition to the NW when it detects the TA of one candidate is invalid/needed.

The variation of the TA mainly caused by the distance between the UE and NW. Thus, the TA may be still valid even the serving cell is changed. The UL sync to the candidate causes more latency and signalling burden which should be avoid as long as the TA is valid. For the target cell, the UE could continue the communication with the current TA if it is still valid which reduces the interruption caused by cell switch.

Proposal 4: RAN2 to confirm that the received CLTM TAs of other candidate cells is not released at UE side upon CLTM execution.

Proposal 5: The received CLTM TAs of the target cell is still valid within the running TAT at UE side after CLTM execution.

In Rel-18, the UE would perform the LTM execution upon the LTM command MAC CE. The TA is applied upon the reception of the MAC CE without the monitoring of TAT. UE starts the TAT from the target upon accessing the cell and re-start the TAT when obtain a new TA. But in Rel-19, TA was received before the execution and monitored by the TAT configured by the serving cell upon the reception of the new MAC CE for TA delivery. It would be risky if the UE just re-start the TAT with the timer configured by the target without TA re- acquisition since the TA was received some time ago.

Proposal 6: The UE keeps running the TAT corresponding the received TAs of the target cell at UE side upon CLTM execution.

3	Conclusions

In this contribution, we discussed the conditional LTM for Rel-19 mobility WI. Based on the discussion, the following observation and proposals are concluded:

Proposal 1: RAN2 to discuss which info is needed in the new MAC CE for the early sync: TA value, configuration ID and TAG ID, TAT.

Proposal 2: At the UE side, TA validity could also be monitored based on the legacy DL RSRP solution and/or the new timer for candidate’s TA.

Proposal 3: UE could request the TA acquisition to the NW when it detects the TA of one candidate is invalid/needed.

Proposal 4: RAN2 to confirm that the received CLTM TAs of other candidate cells is not released at UE side upon CLTM execution.

Proposal 5: The received CLTM TAs of the target cell is still valid within the running TAT at UE side after CLTM execution.

Proposal 6: The UE keeps running the TAT corresponding the received TAs of the target cell at UE side upon CLTM execution.

4	References