3GPP TSG-RAN WG2 Meeting #129bis	R2-2501843

Wuhan, China, April 7th - 11th, 2025

Agenda Item:	8.8.4

Source:	HONOR

Title: 	Discussions on the support of broadcast service

Document for:	Discussion and decision

1	Introduction

During the previous meetings for Rel-19 NTN, RAN2 clarified use cases of the broadcast service in NTN system and made some basic agreements during RAN2#129 meeting as follows.

Agreements 1. In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list 2. It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH) 3. Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates” (SIB6 is not segmented)

In this contribution, we would further discuss how to provide the broadcast service via NTN based on the previous agreements.

- Discussion

Considering the size of the new SIB, if the UE already obtained the list of some intended service areas which are overlapped with some cells, it could save it to the variable part and use it based on the information provided in the MCCH when moved in another cell. In this way, UE is not required to re-obtain the new SIB before it establish/release the MRB if there is no update (reuse the legacy systemInformationAreaID of the SIB is not changed) when it still within the area in the obtained area information. Furthermore, in the current specification, the UE could skip the MCCH acquisition based on the cell/tracking area list in the USD (User Service Description) when the UE is outside the MBS service area of the MBS broadcast service. The MCCH skip is reasonable since the UE out of the intended service area could not receive the MBS service even the cell could provide the service. Similar operation could be performed to avoid MCCH

Proposal 1：The index of the intended service areas is unified assigned across cells.

Although the issue is more severe in the NTN scenario, the intended area is defined since the NTN cell may be too big to provide MBS service within the proper area. However, since the TN cell is related to the deployment of the gNB and the intended service area is based on the policy provided by the service provider. This means the border of the intended service area may be not aligned with the border of the TN cell. Thus, TN UEs could also utilize the info to understand whether it can obtain the specific MBS service based on its location.

Proposal 2: The MBS intended service area mechanism should not be restricted to neither NTN cells nor NTN-capable UEs.

The MBS Interest Indication (MII) is introduced to provide the information about the receiving/interested frequencies/services and the priority related to the MBS. The information provided in the MII may be used to assist the NW to determine the MBS related configuration or the RB management. As shown in the previous local MRB service, the UE could enjoy the service as long as it could receive the data since the service is only provided to all UEs in a geographical area. However, for the intended service area, the UE able to receive the broadcast may be located out of the intended area. Thus, assistance information about the UE’s ISA could be reported along with the MII to assist the NW.

Proposal 3: The MII mechanism could be reused to report the ISA ID of the UE.

In the current specification, UE could avoid acquiring MCCH for the local broad cast service as shown in the below. Based on the assumption that the ISA may be quite static, it is also beneficial that the UE use the intended service area list related to one specific broadcast service in the USD to avoid acquiring the MCCH when UE is outside the MBS service area of this MBS broadcast service.

Proposal 4: UE could use the intended service area list related to one specific broadcast service in the USD to avoid acquiring the MCCH when UE is outside the MBS service area of this MBS broadcast service.

3	Conclusions

In this contribution, we discussed the issues related to intended service area in NTN. Based on the discussion, the following proposals are concluded:

Proposal 1：The index of the intended service areas is unified assigned across cells.

Proposal 2: The MBS intended service area mechanism should not be restricted to neither NTN cells nor NTN-capable UEs.

Proposal 3: The MII mechanism could be reused to report the ISA ID of the UE.

Proposal 4: UE could use the intended service area list related to one specific broadcast service in the USD to avoid acquiring the MCCH when UE is outside the MBS service area of this MBS broadcast service.

4	References

- 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".