3GPP TSG-RAN WG2 Meeting #129bis	R2-2501846

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.2.2

Source: 	CATT

Title: 	Discussion on Paging for Ambient IoT

Document for: 	Discussion and Decision

Introduction

Based on the progress of RAN2#129 [1] and the discussion of post email [2] and [3], this contribution further discusses the AIoT paging mechanisms, including parallel service requests, subsequent paging mechanism and paging message contents.

Discussion

2.1	Multiple paging received in parallel

Post email [1] discusses several scenarios of device receiving multiple paging messages from different/same reader(s) for different/same service request(s). For these proposed scenarios, the analysis is given thereinafter.

- Device receives paging messages from the same reader for the same service request;

- Device receives paging messages from the same reader for different service requests;

- Device receives paging messages from different readers for the same service request;

- Device receives paging messages from different readers for different service requests.

Scenario a) is a normal case called by subsequent paging and will be discussed in section 2.2; Scenario b) can be excluded based on the agreement achieved in RAN2#129, which states that parallel service requests from the same reader is not supported. It is commonly understood that Scenario b) can be avoided by reader implementation.

Proposal 1: It is the responsibility of the reader implementation to prevent a device from receiving paging messages for different service requests from the same reader at the same time. RAN2 will not continue to discuss improvements for this specific case.

Scenario c) and scenario d) arise from overlapping coverage among readers. Based on comments from companies [1], the rapporteur summarizes the following proposals, which assume by default that the device in the overlap area could be able to decode R2D messages from multiple readers in parallel (then further discussion on device behaviour under these scenarios is possible) correctly. However, we should review the assumption before further proceed.

Proposal 1: Discuss and agree on the device behavior if it gets a new service request while one procedure is still ongoing (i.e. not completed or failed yet): (a) ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure, or (b) terminate the ongoing procedure and respond to the latest request. (a/b/ffs = 17/8/4) Proposal 2: Discuss and decide how a device determines a procedure is ‘ongoing’ or not.

Take the figure provided by a company as an example [1], where a device is located in the overlap area of two readers. These two readers initiate either different or same service requests based on CN request. The R2D messages (including paging message and all other subsequent R2D messages) from these two readers are interlaced in time domain.

Figure 1. Example of parallel services procedure [1]

Since FDM and CDM are not supported for R2D in Rel-19 [4], R2D messages from the two readers will highly likely cause mutual interfere without pre-coordination in the time domain. Under this situation, we are suspicious of the possibility on the device successfully decoding the R2D messages.

#### 6.1.1.8 R2D multiplexing For R2D, time-domain multiplexing is the baseline. Code-domain multiplexing is not considered for device 1/2a/2b. Frequency-domain multiplexing is not considered for the devices with an RF-ED receiver (see Clause 5). For device 2b with IF-ED or ZIF receivers, the study considered the following technical aspects:

Observation 1: Since FDM and CDM are not supported for R2D in Rel-19, R2D messages from the two readers will highly likely cause mutual interfere without pre-coordination in the time domain. This interference may cause the device to fail to decode the R2D messages.

Therefore, it is recommended that RAN1 first evaluates this scenario before RAN2 discusses details on the device behaviour. If RAN1 determines that decoding failure is probable, time-domain resource coordination among readers should be considered, along with RAN3 involvement. Hence, it is proposed:

Proposal 2: Send an LS to RAN1 (CC RAN3) to evaluate the possibility of the successfully decoding parallel service requests/R2D messages from multiple readers, considering the potential R2D interference in overlapping areas.

2.2	How to support AIoT paging procedure with subsequent paging messages

Note that this section primarily focuses on the Scenario a) in section 2.1. If the interference issue in Scenario c) and d) is tolerable (i.e., does not cause serious device decoding failure), the solution given in this section can also be taken as baseline for further discussion on these scenarios.

During online discussion of RAN2#129 [1] and the post email [2], we discussed some details regarding the subsequent paging solution based on the transaction ID, including the length of transaction ID and which entity generates the transaction ID. Further, the device behaviour is discussed thereinafter.

Figure 2. AIoT paging procedure with subsequent paging messages

Referencing TR [4], the transaction ID is used by the device to determine whether to skip the response to A-IoT paging message or not. That is, once the device has successfully responded to the service request with a transaction ID included in a paging message, it should not respond to the subsequent paging messages containing the same ID value. It is worth noting that the reader may reuse the same transaction ID for the following service requests at non-overlapping time (e.g., reuse the ID value after the current service is finished). For a certain service request, the “skip response” operation for this transaction ID should be only performed during this service procedure to avoid missing the following service requests from CN.

Proposal 3: Once the device has successfully responded to a service indicated by a transaction ID within the paging message, it shall not respond to subsequent paging messages with the same transaction ID before the end of the entire paging procedure for this service. Otherwise, it shall respond to subsequent paging messages (if received).

- Determination of successful respond

Regarding how the device determines the successful respond to an ongoing service, we think an explicit R2D message for this purpose is necessary. Specifically, once the device has successfully responded the inventory/command, the reader sends a “successfully completed indication” to the device. This solution is also proposed by companies in the context of the “validity of AS ID” in the post email [3], for the device to determine when to release the stored AS ID (refer to option 4b or option 6). Given that introducing an explicit indication is simple and straightforward, and can address both issues (i.e., determination of successful respond and release of AS ID), it is proposed:

Proposal 4: Introduce an R2D message to enable the device to determine whether it has successfully responded to the current service request.

- Storage of necessary state information

During the paging procedure with subsequent paging messages, the device needs to store some status information for the ongoing service request, e.g., the transaction ID and whether it has successfully responded to the service request or not. RAN1 has agreed to support the following two types of memory [5].

- Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage.

Considering the status information is used in short-term and changes frequently, it is more suitable to store the information in the temporary memory.

Proposal 5: Before the end of the ongoing paging procedure, the device needs to maintain some status information for the service request in its temporary memory (e.g., the transaction ID and whether it has successfully responded or not, etc.).

- When to release the stored status information

The maintained status information should be released when the whole procedure of the ongoing AIoT paging procedure is completed, allowing the device to response to the following service request that reuses the same transaction ID. One potential approach is relying on the device power-off: when the device is out of power, all temporarily stored information in the temporary memory, including the status information for subsequent paging procedure, is erased [5]. While the power-off mechanism is workable and is supported by default, relying solely on this mechanism is not beneficial for device power consumption and may cause the device to miss possible paging messages when it is out of power. On this basis, several candidate solutions on releasing the stored status information are summarized as follows:

- Option 1: The device receives paging message with a different transaction ID.

- Option 2: A timer is used by device to determine end of the paging procedure, the length of timer could be predefined in spec (e.g., 10s) or configured by reader in the R2D message.

- Option 3: The device is explicitly indicated by the reader with an “end” indication (e.g., to be a one bit indication) in the last paging message of a paging procedure.

Among the three options, Option 1 is simplest but this solution has limited applicability. The time interval between two service requests from readers is unpredictable, and if too long, device will retain status information until power loss, which is impractical. For option 2 and option 3, though requiring more spec impacts, benefit device power saving, ensure stable device status and prevent paging misses. We therefore favour Options 2 and 3.

Proposal 6: Regarding the conditions for device to release the stored status information for the ongoing paging procedure with subsequent paging messages, RAN2 further discusses which of the following options to adopt:

- The device uses a timer to determine the end, where the timer length can be predefined in the specification or configured by reader in R2D message.

- Explicitly indicated by the reader with an “end” indication in the last paging message of the paging procedure.

2.3	Paging message content and format

2.3.1		Paging identifier

Note that the term “paging identifier” used in this contribution refers to the ID used to address one single device or group ID used to address a group of devices.

- Whether the paging identifier is visible to AIoT MAC

According to the TR 23700-13 [6], from SA2’s perspective, the procedures/messages supported by AIoT Device NAS layer do not include the inventory request, i.e., the paging message does not contain AIoT Device NAS layer data so far.

- AIoT Device NAS protocol is supported between the AIoT Device and the AIOTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

Observation 2: So far, SA2 assumes no AIoT NAS data is contained in the inventory request/ AIoT paging message.

Also, referring to the LS from SA3 [7] and agreed CR [8], SA3 will introduce a temporary ID for security use. Note that, based on the yellow highlight text below, the temporary ID is per device, and the design for a group ID is not in the current SA3 scope. Whether the group ID over the AIoT interface has security issue is still pending on SA3. We understand the temporary ID is primarily used to page single UE over the AIoT interface.

The following aspects and principles are agreed for the conclusion on KI#3 - A mechanism to protect AIoT device ID based on the use of temporary ID shall be supported. - Mechanism shall allow unambiguous identification of the AIoT device - A mechanism to re-synchronize de-synchronized temporary IDs shall be supported. Editor’s Note: Additional conclusions on solution are FFS

Observation 3: SA3 agreed to introduce temporary ID to protect device ID. RAN2 understands the temporary ID is used to page single UE over the AIoT radio interface, group ID is not in the scope so far.

On this basis, we understand the temporary ID share the similar functionality as the 5G-S-TMSI in NR, and should be transmitted over NG and AIoT interface as cleartext IE, i.e., visible to AIoT MAC layer. However, the final decision depends on SA2 and SA3, i.e., specifically on the format in which AIoTF sends the paging identifier to reader over the NG message. If the AIoTF sends the paging identifier to reader in format of AIoT NAS data, the reader will forward it to the device in format of AIoT NAS data; else if the AIoTF sends the paging identifier to reader as clear text IE in NG message, the reader will forward it to device as cleartext IE in AIoT MAC message.

Therefore, it is recommended that RAN2 sends LS to SA2 and SA3 to confirm our assumption above.

Proposal 7: RAN2 understands the reader pages a single device via the temporary ID over the AIoT radio interface, and the temporary ID is not in the A-IoT NAS data format, i.e., visible to A-IoT MAC layer. Send an LS to SA2 and SA3 for confirmation.

- Whether to support sub-group paging solution

During the online discussion of RAN2#129, the FFS “if there is really a need for visibility in the MAC layer” was left due to the potential sub-paging operation by the A-IoT MAC layer. The proponent wants to enable the reader to further sub-group the target devices when the AIoTF addresses all the devices or a group of devices. As analyzed above, the discussion on the visibility of paging identifier pertains to single device paging, whereas the sub-group paging solution targets paging a group of devices or all the devices. The progress made by SA2 and SA3 cannot be used as arguments to support sub-group paging solutions.

Additionally, we are uncertain how the reader sub-groups the devices for the cases of paging all devices and paging a group of devices with group ID. Even if the reader could do that, we are concern on the additional complexity to the device, as the device needs to support matching mechanism for sub-group paging.

Observation 4: When addressing all devices or a group of devices (even though the group ID is visible to the MAC layer), the reader is unaware of how to further divide the target devices into sub-groups.

Proposal 8: Do not support sub-group paging solution by the reader.

2.3.2		Access configuration

In last meeting, we reached the following agreements.

- Agreements - the A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field

We understand that the second agreement refers to use R2D transmission to determine the start timing point of certain access occasion(s) rather than providing the RACH configurations for the device. Since configuring msg1 resource in the paging message is already supported, it seems unnecessary to support configuring RACH configurations in the subsequent R2D messages. We only identifies timing requirement for the R2D transmission in the second agreement, and using L1 signalling to realize that is simpler. We propose sending LS to RAN1 to ask whether the R2D transmission, used to determine the timing of access occasion, can be realized by L1 signalling.

Proposal 9: Clarify that the R2D transmission, which determines the Msg1 resource(s) is used to determine the start time point of certain access occasions rather than configuring random access parameters.

Proposal 10: Send an LS to RAN1 to ask whether the R2D transmission which determines the Msg1 resource(s) can be realized by L1 signalling.

Conclusion

In this contribution, paging functionality for A-IoT is discussed with giving corresponding observations and proposals:

Multiple service requests received in parallel

Proposal 1: It is the responsibility of the reader implementation to prevent a device from receiving paging messages for different service requests from the same reader at the same time. RAN2 will not continue to discuss improvements for this specific case.

Observation 1: Since FDM and CDM are not supported for R2D in Rel-19, R2D messages from the two readers will highly likely cause mutual interfere without pre-coordination in the time domain. This interference may cause the device to fail to decode the R2D messages.

Proposal 2: Send an LS to RAN1 (CC RAN3) to evaluate the possibility of the successfully decoding parallel service requests/R2D messages from multiple readers, considering the potential R2D interference in overlapping areas.

Subsequent paging

Proposal 3: Once the device has successfully responded to a service indicated by a transaction ID within the paging message, it shall not respond to subsequent paging messages with the same transaction ID before the end of the entire paging procedure for this service. Otherwise, it shall respond to subsequent paging messages (if received).

Proposal 4: Introduce an R2D message to enable the device to determine whether it has successfully responded to the current service request.

Proposal 5: Before the end of the ongoing paging procedure, the device needs to maintain some status information for the service request in its temporary memory (e.g., the transaction ID and whether it has successfully responded or not, etc.).

Proposal 6: Regarding the conditions for device to release the stored status information for the ongoing paging procedure with subsequent paging messages, RAN2 further discusses which of the following options to adopt:

- The device uses a timer to determine the end, where the timer length can be predefined in the specification or configured by reader in R2D message.

- Explicitly indicated by the reader with an “end” indication in the last paging message of the paging procedure.

Paging message content and format

Observation 2: So far, SA2 assumes no AIoT NAS data is contained in the inventory request/ AIoT paging message.

Observation 3: SA3 agreed to introduce temporary ID to protect device ID. RAN2 understands the temporary ID is used to page single UE over the AIoT radio interface, group ID is not in the scope so far.

Proposal 7: RAN2 understands the reader pages a single device via the temporary ID over the AIoT radio interface, and the temporary ID is not in the A-IoT NAS data format, i.e., visible to A-IoT MAC layer. Send an LS to SA2 and SA3 for confirmation.

Observation 4: When addressing all devices or a group of devices (even though the group ID is visible to the MAC layer), the reader is unaware of how to further divide the target devices into sub-groups.

Proposal 8: Do not support sub-group paging solution by the reader.

Proposal 9: Clarify that the R2D transmission, which determines the Msg1 resource(s) is used to determine the start time point of certain access occasions rather than configuring random access parameters.

Proposal 10: Send an LS to RAN1 to ask whether the R2D transmission which determines the Msg1 resource(s) can be realized by L1 signalling.

Reference

- RAN2#129 Chair’s notes.

- Report of [POST129][035][AIoT] Paging

- Report of [POST129][036][AIoT] AS ID

- 3GPP TR 38.769 Study on solutions for ambient IoT (Internet of Things).

- RAN1#116 Chair’s notes.

- 3GPP TR 23.700-13: Study on Architecture support of Ambient power-enabled Internet of Things.

- R2-2501502	Reply LS on security aspects of Ambient IoT

- S3-251048	Conclusion on KI#3