3GPP TSG-RAN WG2 Meeting #129bis                                                              R2-2501848

Wuhan, China, April. 7th – 11th, 2025

Source:	CATT

Title:	Discussion on data transmission for A-IoT

Agenda Item:	8.2.4

Document for:	Discussion

1.	Introduction

In this paper, we give our views on the potential issues for A-IoT data transmission according to the WID in [1] and the progress at RAN2#129. The following aspects are covered in our discussion,

- Segmentation for D2R

- MAC PDU/signaling general format

- Data (re)transmission for failure handling

Agreement has been reached at RAN2#129 meeting as below:

Agreement Aim to design simple MAC PDU format design Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). FFS whether for D2R we need message type field, any length and need for padding Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

The Section 2 gives the detailed discussion, and followed by Section 3 that summarizes this paper.

2.	Discussion

2.1 Segmentation

2.1.1 One-bit Indication for segmentation

According to the RAN2#129 agreements, RAN2 will further discuss if the 1 bit is sufficient for the segmentation functionality, if CN does not provide the expected D2R message size to the reader.

Agreement

To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size.   If not possible, FFS if the 1 bit is sufficient.

We can find the final conclusion in SA2 after the SI is closed. There is editor’s note: How the AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) without input from the AF is FFS.

## 5.4 Assistance information provided to AIoT RAN node The AIOTF provides the assistance information to the AIoT RAN together with the service operation requests. The AIOTF determines the assistance information provided to the AIoT RAN based on the information received from the AF. The Assistance information is used by the AIoT RAN for performing service operations, e.g. radio resource allocation. The following assistance information may be provided: - AIoT service type (e.g. Inventory, Command); - Optionally, approximate number of AIoT devices based on AF request; - Optionally, approximate D2R message size based on AF request. Editor's note: Other assistance information may be added later if necessary. Editor's note: How the AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) without input from the AF is FFS.

Let’s assume that two bits of indication are required including Segment or Not, not the Last or the last. According to the table value, it is found that ‘the end of data’ can be represented by notTheLast with ‘0’, and more data can be represented by notTheLast with ‘1’, no matter whatever it is segment or not. So it seems 1 bit indication of notTheLast is sufficient even without the message size indication from CN to gNB.

| Segment or not | notTheLast | Comments |
| --- | --- | --- |
| 1(seg) | 0 | The end of data |
| 1(seg) | 1 | More data |
| 0(not seg) | 0 | The end of data |
| 0(not seg) | 1 | Won’t happen |

Proposal 1: 1-bit indication (e.g. notTheLast) is sufficient to support the segmentation functionality, even if CN cannot provide an estimated expected D2R message size to gNB.

2.1.2 Retransmission of the unsegmented packet

There is FFS issue on the retransmission of the unsegmented packet at RAN2#129 as below:

FFS This implies that unsegmented packet can also be retransmitted.

Below please find the procedures of retransmitting segmented and unsegmented packet. In unsegmented packet case, gNB can set the offset (e.g. 0) indicating retransmission of this packet. So it seems that the packet without segmentation also can be retransmitted within the same mechanism.

Proposal 2: RAN2 to confirm that packets without segmentation can also be retransmitted using the same mechanism as segmented retransmissions, based on the 1-bit ‘notTheLast’ indication from device to reader and the offset indication from reader to device.

2.1.3 Segment and retransmission of MSG3

There is FFS issue on if segment and retransmission applies to msg3 at RAN2#129:

- For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

MSG3 is for piggybacking the EPC ID according to the previous conclusion. However it still may happen that the TB size of MSG3 is not large enough to transmit the whole EPC ID (e.g. 128bits). So the mechanism that reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start) can work on retransmission of MSG3.

Observation 1: Segment and retransmission can be applied to MSG3 which piggybacks the stored data (e.g., EPC ID) in device.

On the other hand, the reader can control not to support the segmentation of MSG3 since SA2 intended to define a fixed size of EPC ID after SI discussion. So it’s up to reader to provide the sufficient resource for MSG3 transmission. .

Observation 2: Segment and retransmission of MSG3 is not mandatory for device.

Proposal 3: The reader can control no segmentation of MSG3 by implementation.

2.2 MAC PDU design

2.2.1 R2D MAC PDU format

RAN2 has discussed the R2D MAC PDU format in general and reached the agreement on R2D PDU at RAN2#129 meeting:

Agreement

•	At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s).

	R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF))

	R2D MAC PDU (MSG2)

	R2D MAC PDU (R2D data)

MAC header in R2D message

There are a few potential message types of R2D and the potential contents in these MSGs according to the previous agreement and discussion which is summarized in below table:

| R2D Message Type | Identified by | Contents | Use case |
| --- | --- | --- | --- |
| Paging | 000 | Paging ID, Transaction ID, resources for MSG1, | Paging message is sent when reader receives the service request from CN |
| MSG2 | 001 | RN16, AS ID assignment, resources for MSG3, | MSG2 is sent when the MSG1 is received in CBRA. |
| Only CE | 010 | Control info of segmentation (Offset) indication, scheduling resources designed by RAN1, service end indication (FFS). | Based on channel conditions and coverage, data and control CE(s) can be transmitted individually or combined into a single message for sending. |
| Only SDU | 011 | Command from CN |  |
| CE+ SDU | 100 | Scheduling resources designed by RAN1 and Command from CN |  |

It seems 3 bits are required to indicate what the R2D message type is.

The target AS ID is also required in the MAC header to be distinguished from other devices in the R2D message, except for paging and MSG2 message.

Proposal 4: 3-bits is sufficient for message type field in the MAC header of R2D message.

Proposal 5: AS ID field is required in the MAC header of R2D message, except for paging and MSG2 message.

MAC CE design in R2D message

The information can be included in one MAC CE, including Control info of segmentation (Offset), and scheduling resources (e.g. UL Grant) designed by RAN1. So we proposed:

Proposal 6: The MAC CE in R2D PDU consists of:

- The control info of segmentation (fixed size); and/or

- Scheduling resources (e.g. UL Grant) designed by RAN1 (variable size); or

- Service end indication (option 6 in validity of AS ID discussion as FFS)

FFS others for multicast R2D message

Agreement

2.	Support multiplexing of information for multiple devices in R2D message for msg2.  FFS others for multicast messages

RAN2 has agreed multiplexing of information for multiple devices in R2D message for MSG2. But for other R2D messages, i.e. after MSG2, there is no strong motivation to support multiplexing of any message after MSG2 (for Control info, Data, or Data and CE) for multiple devices.

Proposal 7: There is no others R2D message that needs multiplexing of information for multiple devices except for MSG2.

Format design of MSG2

RN16, AS ID assignment and resources for MSG3 will be included in MSG2. So these contexts will be multiplexed since multiplexing of information for multiple devices is required in R2D message for MSG2.

For example, 4 devices are multiplexing in MSG2,

| Number of Multiplexing of devices | Multiplexing of RN16 |  |  |  |  | Multiplexing of AS ID assignment | Multiplexing of new AS ID |  | Multiplexing of UL grants |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00100(4 devices Up to 32) | 16bits for device#1 | 16bits for device#2 | 16bits for device#3 | 16bits for device#4 |  | 0110 | 8bits for device#2 | 8bits for device#3 | Device#1 | Device#2 | Device#3 | Device#4 |

Number of Multiplexing of devices: ‘100’ means there are 4 multiplexing devices in this MSG2.

Multiplexing of AS ID assignment: Each bit represents one device in this field. ‘0110’ means device 2 and 3 are assigned with new AS ID, while device 1 and 4 are not.

Multiplexing of new AS ID: First AS ID is for device2, and the second is for device3.

Multiplexing of UL grants: All of the Multiplexing of devices are assigned with UL grants.

So we proposed:

Proposal 8: RAN2 to consider the MSG2 format with the following information as baseline:

- The number of multiplexing devices (fixed size, e.g. 5bits);

- Multiplexing of RN 16 (variable size);

- Multiplexing of AS ID assignment (fixed size, e.g. each bit represents one device);

- Multiplexing of new AS ID (e.g. 8bits per device) (variable size);

- Multiplexing of Scheduling resources designed by RAN1 (variable size).

2.2.2 D2R MAC PDU format

RAN2 has discussed the D2R MAC PDU format in general and reached the agreement on D2R PDU at RAN2#129 meeting:

Agreement

- FFS whether for D2R we need message type field, any length and need for padding

Specify message types and contents.  As starting point consider the following MAC message types.

    - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not)

    - D2R MAC PDU (MSG3 and data)

    - Other message types are FFS.  The message types may evolve based on functionality agreements.

We are going to discuss the need of message type field, length and padding field in D2R PDU.

MAC header of D2R PDU

There are following types of D2R PDU in inventory and inventory + command cases:

| R2D Message Type | Contents |
| --- | --- |
| MSG1 | RN16 |
| Data | EPC ID (MSG3), Data following MSG3 |

The message type of MSG1 is not required since the reader is aware of receiving MSG1 according to the detection of the random access occasions. That means MSG1 can be designed as 16-bit fixed size without additional information.

But the length of MSG3 and the following messages are variable because the corresponding size of D2R data message is controlled by the reader. So the length of D2R data PDU is required and padding will be filled in the D2R data PDU to match the TB size.

Proposal 9: MSG1 and D2R data message can be designed separately with no message type field in the D2R message.

Proposal 10: The format design of MSG1 requires only a 16-bit field for RN16.

Proposal 11: A length field is needed in the D2R data PDU. Padding will be added to the D2R data PDU if the data length does not match the TB size.

3. Conclusion

Based on the analysis in the Section 2, the conclusions of this paper are summarized as follows:

Observation 1: Segment and retransmission can be applied to MSG3 which piggybacks the stored data (e.g., EPC ID) in device.

Observation 2: Segment and retransmission of MSG3 is not mandatory for device.

One-bit Indication for segmentation

Proposal 1: 1-bit indication (e.g. notTheLast) is sufficient to support the segmentation functionality, even if CN cannot provide an estimated expected D2R message size to gNB.

Retransmission of the unsegmented packet

Proposal 2: RAN2 to confirm that packets without segmentation can also be retransmitted using the same mechanism as segmented retransmissions, based on the 1-bit ‘notTheLast’ indication from device to reader and the offset indication from reader to device.

Proposal 3: The reader can control no segmentation of MSG3 by implementation.

MAC header in R2D message

Proposal 4: 3-bits is sufficient for message type field in the MAC header of R2D message.

Proposal 5: AS ID field is required in the MAC header of R2D message, except for paging and MSG2 message.

MAC CE design in R2D message

Proposal 6: The MAC CE in R2D PDU consists of:

- The control info of segmentation (fixed size); and/or

- Scheduling resources (e.g. UL Grant) designed by RAN1 (variable size); or

- Service end indication (option 6 in validity of AS ID discussion as FFS)

FFS others for multicast R2D message

Proposal 7: There is no others R2D message that needs multiplexing of information for multiple devices except for MSG2.

Format design of MSG2

Proposal 8: RAN2 to consider the MSG2 format with the following information as baseline:

- The number of multiplexing devices (fixed size, e.g. 5bits);

- Multiplexing of RN 16 (variable size);

- Multiplexing of AS ID assignment (fixed size, e.g. each bit represents one device);

- Multiplexing of new AS ID (e.g. 8bits per device) (variable size);

- Multiplexing of Scheduling resources designed by RAN1 (variable size).

D2R MAC PDU format

Proposal 9: MSG1 and D2R data message can be designed separately with no message type field in the D2R message.

Proposal 10: The format design of MSG1 requires only a 16-bit field for RN16.

Proposal 11: A length field is needed in the D2R data PDU. Padding will be added to the D2R data PDU if the data length does not match the TB size.

4. References

[1] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[2] 3GPP TR 38.769 “Study on solutions for ambient IoT (Internet of Things)” V2.0.1

[3] 3GPP TS 23.xyz “Architecture support for Ambient power-enabled Internet of Things” V0.1.0