3GPP TSG-RAN WG2 Meeting #129bis	R2-2501856

Wuhan, China, 7-11 April 2025

Agenda Item:	8.1.2.3

Source:	NEC

Title:	LCM for Positioning use case

Document for:	Discussion, Decision

# Introduction

In the previous RAN2 meetings, the following agreement was reached for LCM for Positioning use case:

Agreement@RAN2#129

1:	Introduce AI/ML positioning Case 1 as a new positioning method.

2:	Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extension can be discussed while drafting the stage 3 CR.

3:	FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation

4: 	The content of error cause is discussed while drafting stage3 CRs.

5: 	As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list.

6: 	UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality.   FFS if any additional LMF control is needed.

Agreement

For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline.

- 	Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF .

-	Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF.  Configuration details are FFS

- Agreement

- As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause.  FFS if other fallback options are considered

During RAN#107 meeting, the revised WID for Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved with removal of 2nd priority cases. Both case 2a and case 2b were removed from the objective of the WID.

This paper discusses the remaining issues for life cycle management (LCM) for Positioning use case with AIML assistance.

# Discussion

## Model inference

For positioning use case, considering the specification impact will be mainly caused by the UE sided model, RAN2 mainly focus on cases 1 for specification impact analyses, since both case 2a and case 2b were removed from the revised WID for AIML during RAN#107.

For case 1, it is UE-based positioning with UE-sided model, direct AI/ML positioning, it should be the UE that generates the measurement for AI/ML model inference based on the reception of DL PRS transmission and also the assistant information received from LMF. Meanwhile, according to RAN1 agreement, the LMF will indicate the purpose of the assistance data for UE-based positioning for all assistance information from legacy UE-based DL-TDOA other than info #7, and the final decision on the alternatives for providing info #7 is to be decided by RAN1.

RAN2#129 agreed to introduce AI/ML positioning Case 1 as a new positioning method and also agreed to reuse the existing LPP procedures related to Location Information Transfer for providing and requesting the results of the UE sided model inference operation, which include both RequestLocationInformation and ProvideLocationInformation messages.

Within the agreements made by RAN2 at the last meeting, there is an FFS on whether UE autonomous switching between AI/ML and non-AI/ML methods is allowed and also an FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation.

During the email discussion [POST128][026][AIML], there was a discussion on the case where UE can perform both AI/ML and non-AI/ML, but initially UE reports that the applicability condition is not met but later it identifies the applicability conditions are met. Hence, there may be an autonomous switching option for the UE from non-AI/ML to AI/ML positioning method. In our opinion, this switching operation should be subject to network control. The reason, if the applicability conditions were changed in the UE, the UE is expected to report its support of AIML positioning based on the change of the applicability conditions is to the network (i.e., LMF), and then the LMF may be able to instruct the UE to activate such AIML positioning. With this said, the conditional change between AI/ML and non-AI/ML methods has no reason to be supported, since all of these procedures are up to network signalling.

Proposal 1: UE autonomous or conditional switching between AI/ML and non-AI/ML methods is not allowed.

## Performance Monitoring

In RAN1, there were many discussions during RAN1#116bis and RAN1#118 for performance monitoring for AI positioning use case. In summary, the following agreements reached are listed for model performance monitoring for Position use case for UE sided model.

RAN1#116bis Agreement

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label:

- Option A. The target UE side performs monitoring metric calculation.

- Option B. The LMF performs monitoring metric calculation.

RAN1#118 Agreement

Model monitoring related

For case 3a, both gNB and LMF performs monitoring metric calculation are feasible from RAN1 perspective. Further selection left to RAN3’s decision. Method to perform the metric is up to gNB/LMF implementation.

For case 1, RAN1 will not discuss Option A-4(monitoring data transferred from PRU to target UE with specification transparent way).

RAN1#119 Agreement

For model performance monitoring of AI/ML positioning Case 1, support at least:

- Option A. The target UE side performs monitoring metric calculation.

  - The target UE may signal the monitoring outcome to the LMF.

  - FFS: content of monitoring outcome

- FFS: Option B

Meanwhile, there are different options identified for both Option A and Option B for UE sided model. Based on the agreement in RAN1#119, we can see the following Option A operation for the monitoring metric generation and monitoring decision that may impact the air interface

- Target UE performs monitoring metric calculation and signals to the results to LMF, and LMF makes monitoring decision

Even though RAN1 will continue the discussion for the content of the monitoring content and RAN3 may discuss the metric calculation in either gNB or LMF, RAN2 may be able to discuss which type of signalling should be used from the UE to the LMF for transmitting the monitoring outcome, after target UE performs monitoring metric calculation. For example, RAN2 can tentatively discuss if a new signalling is designed for AIML positioning performance monitoring, or just reuse the existing procedures (e.g. Location Information transfer procedure). We would suggest that a new signalling specific for performance monitoring be used between UE and LMF in order to have a clean message structure design.

Proposal-2: A new signalling specific for performance monitoring is used between UE and LMF for the performance data transfer for case 1.

Proposal-3: For model performance monitoring for Position use case for UE sided model, wait for further input from RAN1 on the content of the performance monitoring.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: UE autonomous or conditional switching between AI/ML and non-AI/ML methods is not allowed.

Proposal-2: A new signalling specific for performance monitoring is used between UE and LMF for the performance data transfer for case 1.

Proposal-3: For model performance monitoring for Position use case for UE sided model, wait for further input from RAN1 on the content of the performance monitoring.

# Annex: Agreement made by RAN1#118bis for AI/ML based positioning

Agreement

For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is:

- When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455;

  - FFS: details on how to associate quality indicator to timing information

Conclusion

For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

Note: the UE can be a PRU and/or a Non-PRU UE.

Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Agreement

From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) Channel measurement;

- (Optional) Quality of the channel measurement;

  - FFS: details of the quality

- (Mandatory) Time stamp of the channel measurement.

Agreement

From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) timing information;

- (Optional) Quality of the timing information;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp.

FFS: LOS/NLOS indicator.

Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

Agreement

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference,

- for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other;

- Companies can provide inputs on further enhancements of existing assistance data, including new information

- Note: this does not mean that training and inference phases are mentioned in assistance data.

Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable.

|  | Information | UE-based DL-TdoA | UE-based DL-AoD |
| --- | --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |  |  |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 3 | DL-PRS configuration of candidate NR TRPs |  |  |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |  |  |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |  |  |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |  |  |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |  |  |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 9 | PRS-only TP indication |  |  |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID |  |  |
| 11 | LOS/NLOS indicators |  |  |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |  |  |
| 13 | Validity Area of the Assistance Data |  |  |
| 14 | PRU measurements together with the location information of the PRU |  |  |
| 15 | Data facilitating the integrity results determination of the calculated location |  |  |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |  |  |
| 17 | Expected Angle Assistance information |  |  |
| 18 | PRS priority list |  |  |

# Annex: Agreement made by RAN1#119 for AI/ML based positioning

Agreement

For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows.

  - starting time = first detected path rounded down with timing granularity T.

Note: UE-side measurement is a separate discussion

Agreement

For model performance monitoring of AI/ML positioning Case 1, support at least:

- Option A. The target UE side performs monitoring metric calculation.

  - The target UE may signal the monitoring outcome to the LMF.

  - FFS: content of monitoring outcome

- FFS: Option B

Agreement

For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement,

- The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T.

- The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values.

- The timing information is defined relative to a reference time, same as the path-based measurement.

- The Nt’ selected time domain channel measurement values are expected to be those with the highest power.

- The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T.

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values:

  - Nt'<=24. FFS: Nt' values.

  - Nt = {32, 64, 128}

  - FFS: k

  - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement

- FFS: whether transmit offset from gNB to LMF

Note: measurement by UE is a separate discussion.

Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

Agreement

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following:

- Alternative 1. Info #7 is provided implicitly via associated ID.

  - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly.

  - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 3.  Info #7 is not be provided from LMF to UE.

  - If info #7 is not provided, UE may assume info #7 is consistent between training and inference.

- Alternative 4. Info #7 is provided explicitly from LMF to UE.