3GPP TSG-RAN WG2 Meeting #129bis	R2-2501858

Wuhan, China, 7-11 April 2025

Agenda Item:	8.1.5

Source:	NEC

Title:	Signalling feasibility of dataset and parameter sharing

Document for:	Discussion, Decision

# Introduction

According to RAN1 LS (R1-2410922), RAN2 is expected to evaluate feasibility of the RAN1 identified solutions on two-sided model for CSI compression.

RAN1 has been discussing various ways for alleviating/resolving the issues related to inter-vendor training collaboration to support the use case of CSI compression(two-sided model). In particular, RAN1’s discussion requires the following procedure:

- NW-side shares model parameters and/or dataset to the UE or UE-side via standardized signaling, where the model parameters and/or dataset received at the UE or UE-side go through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., model (re-)training.

RAN1 is considering the following Sub-Options for further study with potential further down-selection.

- Option 4-1: sharing dataset containing N1 samples of {target CSI, CSI feedback} corresponding to the input and output of a nominal encoder, respectively

- Option 3a-1 without target CSI: sharing parameters corresponding to a nominal encoder

- Option 3a-1 with target CSI: sharing parameters corresponding to a nominal encoder, along with dataset containing N2 samples of {target CSI}

According to RAN1 LS in R1-2410922, the size of the dataset and model parameters are as below:

- Option 4-1: sharing {target CSI, CSI feedback} dataset:  around 225MB

- Option 3a-1 without target CSI: sharing encoder parameter: around 11.6 MB

- Option 3a-1 with target CSI: sharing encoder parameters +{target CSI} dataset: 11.6MB+data set

RAN1 concluded that a standardized signaling, if feasible and specified, can be used for parameter and/or dataset exchange to alleviate/resolve the inter-vendor training collaboration complexity.

RAN1 concluded that the standardized signaling may be Over-the-air or other approaches.

The Email discussion of [POST129][029][AI Phy] Model transfer discussed both non-OTA approach and OTA approach for the transmission of model dataset and model parameters. This paper intends to discuss the CP based approach for OTA based transmission.

# Discussion

As captured in TR38.843, Model size >45kBytes is not supported based on existing number of RRC segments.

During the email discussion, the rapporteur suggested that the following specification impact can be considered to achieve OTA solution 1a feasibility from RAN2 point of view:

- Segmentation to support hundreds of MB dataset/model parameter transfer

- Requires service continuity support for SRBs with segmentations during UE mobility

- Xn/NGAP enhancement(s) for dataset/model parameter transfer continuity

- Impact on SRB in DL, e.g., a new SRB with configurable priority, etc.

We share the same views as the companies showing concerns during email discussion ([POST129][029]) regarding the specification cost and standardization efforts for solution 1a based approach (i.e. CP based mechanism). For example, the RRC buffer equipped within UE can not handle such large RRC message for the current implementation. Meanwhile a new design based segmentation at RRC layer or above is subject to tremendous protocol architecture design, which will be not enjoyed by 5G NR UE in general, due to the introduction of the late release of NR. Some companies indicated the possibility to transfer partial model data set or parameters for a particular UE, and then multiple UE may be able to deliver all of the data set or parameters to the UE side AIML model training server in a joint manner. We think in this case, there is huge efforts need to ensure proper UE selection for dataset/parameter transfer from a UE vendor, and it is very hard to ensure the E2E reliability of dataset/model parameter transfer.

With the above analysis, we suggest to not pick CP based mechanism for OTA solution for 5G Rel-19.

Proposal-1: Solution 1a (CP based mechanism) is not adopted for the transmission of AIML dataset or parameters for OTA solution for 5G Rel-19.

Meanwhile, we think that sharing dataset (with a size up to 225MB)or sharing parameters corresponding to a nominal encoder (up to 11.6MB) from the gNB to the UE via data transmission tunnel (UP) may be possible. However traditionally the UP data tunnel is established between the UE and UPF in the 5GC. The gNB may need to send the dataset or parameters to the core network, that is able to distribute the content to the UE via traditional UP tunnel (CN->gNB->UE). In this manner, the sharing dataset or the sharing parameters will be transmitted as pure data, not signalling. It is basically the solution-2 as documented in TR38.843

Proposal-2: the gNB sends the dataset or parameters to the Core Network, and solution-2 is used to distribute the content to the UE via traditional UP tunnel (CN->gNB->UE).

Considering the fact that sharing the data set and the parameter for AIML training from the network may apply to multiple UEs, there may be a possibility to use broadcast or multicast based transmission over the air. In addition, as the transmission of the data set and the parameter for AIML training takes a long time and the delay requirement is very low for such transmission. Hence there may be possible for the UE to receive such data set and the parameter when transiting to RRC Idle or RRC Inactive state.

Proposal-3: the UE may receive the data set and the parameter for AIML training via broadcast or multicast based reception.

Proposal-4: the UE can receive the data set and the parameter for AIML training during RRC Idle or RRC Inactive state.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: Solution 1a (CP based mechanism) is not adopted for the transmission of AIML dataset or parameters for OTA solution for 5G Rel-19.

Proposal-2: the gNB sends the dataset or parameters to the Core Network, and solution-2 is used to  distribute the content to the UE via traditional UP tunnel (CN->gNB->UE).

Proposal-3 the UE may receive the data set and the parameter for AIML training via broadcast or multicast based reception.

Proposal-4: the UE can receive the data set and the parameter for AIML training during RRC Idle or RRC Inactive state.