3GPP TSG-RAN WG2 Meeting #129bis	           R2-2501864

Wuhan, China, April 7th –11th,2025

Agenda item:		8.5.3	On-demand SIB1

Title:	Remaining issues for On-demand SIB1 request and UE behaviour

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

During RAN2 #127bis, the following agreements were made:

OD-SIB1 Validity: 7. A cell for which SIB1 request configuration is available, can periodically broadcast SIB1. 8. If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

During RAN2 #128, the following agreement was made:

NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

During RAN #106, it was agreed that:

RAN confirms that Case 2 also includes the case where a particular Cell A can switch to become a NES Cell.

During RAN #129, it was agreed that:

Agreements on OD-SIB1 - There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1. - Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running: - When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid. - The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition. - The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs. - The UE follows the legacy validity principle of stored SIB. - For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected. - SIB-x can be cell specific or area specific, as legacy. - Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

In this contribution, we discuss further on remaining issues for UE behaviour for on-demand SIB1 acquisition.

# 2. Discussion

2.1	UE behaviour before UL WUS request transmission

During RAN2 #127bis, it was agreed that UE should check if SIB1 is being broadcast before it transmits WUS request.
There are two scenarios when this could happen:

- Scenario 1: the NES cell is broadcasting SIB1 periodically

  - SIB1 may be regularly broadcast by NES cell with the legacy periodicity (e.g. 160ms)

- Scenario 2: the NES cell is transmitting SIB1 only temporarily

  - It may be triggered by the on demand SIB1 request sent to the NES cell from other UE(s).

  - It may be triggered by a regular SIB1 update within the NES cell.

In Scenario 1, the UE will be aware of the broadcast SIB1 from the MIB, so the default behaviour should be to ignore the UL WUS configuration and listen to the broadcast SIB1, as agreed in RAN2 #127-bis.

In Scenario 2, if SIB1 is transmitted only temporarily, it is not clear what condition there is or how long a UE should wait until sending its own WUS request.

We see two options:

- Option 1: The network can temporarily indicate SIB1 transmission using MIB.

- Option 2: UE follows the CORESET0 to monitor SIB1 transmission for a specific duration before requesting OD-SIB1.

- Option -3: It is up to the UE implementation when the UE send the SIB1 request to the NES cell when SIB1 is not available.

In Option 1, the situation is similar to Scenario 1. A new indication might be required to avoid misinterpreting the announced SIB1 broadcast as being periodic, although previous agreements indicate that a legacy UE having a valid SIB1 shall not bar the NES Cell. Then another option is that the broadcast SIB1 itself will then bar legacy UEs. Any new IE added to the MIB would impact the overall performance. We don’t suggest Option-1. While this has RAN2 impacts, this is ultimately up to RAN1 to define such signalling, if deemed necessary.

In Option 2, we assume that the UE tries to receive the SIB1 for a specific duration. RAN2 should discuss the need to define a duration (e.g. configurable) for which the UE monitors SIB1 before attempting UL WUS for OD-SIB1. If the duration is too short, many UEs may unnecessarily trigger UL WUS request. If the duration is too long, there could be a problem where every UE waits for another UE to trigger UL WUS request. Network may configure a timer to the UE, the UE may start the timer and wait for the expiration of the timer before sending the OD-SIB1 request to the NES cell.

In Option 3, It is up to the UE implementation. This may be the simplest solution without having much specification impact.

Proposal 1: The monitoring duration for NES cell’s SIB1 before sending the OD-SIB1 request can be up to the UE implementation.

  - Special “Case 2” where Cell A becomes an NES cell

During RAN #106, it was agreed to include the special scenario where a Cell A can provide UL-WUS configuration for itself before becoming a NES cell.

There are two scenarios:

- Scenario 1: No other cell A, only future NES cell is broadcasting its UL WUS config

  - Then the NW needs to configure at least one new Cell A

- Scenario 2: There was already another Cell A2

  - NES cell may indicate to the other Cell A that is switching to NES mode, however

  - May not be necessary since other Cell A can broadcast WUS config anyway and UEs check if SIB1 is being broadcast before doing OD-SIB1

Configuring a Cell A to a NES cell is up to network implementation and will be possible even in a more typical “Case 2” scenario. If the cell A will be turned into NES cell, the uplink WUS configuration of cell A may needed to be broadcast by another cell A. Hence, cell A may need to share its uplink WUS configuration to that cell A before switching to NES cell mode.

Proposal 2: Configuring a cell from Cell A to NES cell is up to network implementation.

Proposal 3: The coordination between cell A to be switched into NES cell and another cell A is up to RAN3 to define kind of Xn signaling needed for this purpose.

# 3. Conclusion

In this contribution, we have made the following proposals.

Proposal 1: The monitoring duration for NES cell’s SIB1 before sending the OD-SIB1 request can be up to the UE implementation.

Proposal 2: Configuring a cell from Cell A to NES cell is up to network implementation.

Proposal 3: The coordination between cell A to be switched into NES cell and another cell A is up to RAN3 to define kind of Xn signaling needed for this purpose.