3GPP TSG-RAN WG2 Meeting #129bis	                   R2-2501865

Wuhan, China, April 7th –11th,2025

Agenda item:	8.5.4 Adaptation of common signal channel transmissions

Source:	NEC

Title:	PRACH, paging and SSB adaptation for NES

Document for:	Discussion, Decision

# Introduction

Below is mentioned regarding the agreements on RACH adaptation in RAN2#129,

1.	Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.

2a.	RAN2 starts CBRA for RACH adaptation.

2b. 	RAN2 starts 4-step RACH adaptation.

3. 	From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request.

4.	From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.

5a. 	From R2 perspective, RACH adaptation is not modelled as RA feature(s).

5b.	From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

6.	Will follow legacy mechanism regarding how to select RACH resource.

In this contribution, we discuss the latest agreements in RAN2 #129 and how they impact specification work going forward.

# Discussion

## PRACH adaptation from RAN2 point of view

For the adaptation of PRACH, the time-domain based approach was discussed by RAN1. Specifically, the adaptation can be based on the configuration of additional or different PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs.

Traditionally, the UE knows the PRACH time-domain resources (periodicity, sub-frame, slot and OFDM symbol it used) through the PRACH configuration index (i.e., prach-ConfigurationIndex, or by msgA-PRACH-ConfigurationIndex) that is transmitted by SIB1/RRC messages to the UE.

Hence, the first issue for RAN2 is to clarify how the gNB configure such additional or different PRACH resources for NES-capable UEs. To discuss this issue, an assumption should be made if the UE can read the SIB1 message from the serving cell. In the case where SIB1 is subject to on-demand request as discussed in Rel-19 NES WID, the UE may be not able to read this additional or different PRACH resources.

Proposal 1: RAN2 to discuss the configuration of additional or different PRACH resources for NES-capable UEs.

In legacy NR systems, the UE will use RA-RNTI to identify itself during RACH procedure. RA-RNTI is used by UE to receive Random Access Response (RAR) once PRACH is transmitted. Different RACH occasions within a system frame are associated with different RA-RNTI values to ensure that RAR for a given PRACH transmission can be properly addressed to only the UE which has used that given RACH occasion. If RA-RNTI values are same for 2 RACH occasions, then there is a chance of confusion where 2 UEs which have used different RACH occasions act on the same RAR message. To ensure this uniqueness, RA-RNTI is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where,

s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80)

f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8)

ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier)

When additional RACH resources are configured to UE, there may be a chance that RA-RNTI confusion may occur as one legacy RACH resource and an additional RACH resource have get the same RA-RNTI value.

Indeed, RAN1 has not ruled out the possibility of sharing PRACH resources in both time and frequency “FFS: Additional case whether full/partial overlap in both time and frequency is allowed.”

Proposal 2: RAN2 to discuss the possibility of the occurrence of RA-RNTI confusion with additional RACH resources.

Another topic is whether Rel-19 NES-capable UEs should be restricted to their new resources or could also access legacy resources. During high load scenarios with increased demand from Rel-19 UEs, additional PRACH resources are provisioned. NES UEs are allowed to access both legacy and NES-specific PRACH resources under such conditions. Rel-19 UEs initially utilize the NES-specific PRACH resources and only fall back to legacy PRACH resources if needed. This mechanism ensures that legacy UEs remain unaffected.

Proposal 3: NES UEs should prioritize NES-specific resources and fall back to legacy resources only when needed, ensuring no impact on legacy UEs.

## Paging Adaptation

Below is mentioned regarding the agreements on paging adaptation in RAN2#129,

- For N, values smaller than T/32 are not supported.

- The maximum possible value for Ns is 8.

- Introduce a separate PEI configuration.

- Paging adaptations are configured semi-statically and updated via system information update notification.

- A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

Based on the discussion on the last meeting, there was a FFS on on whether we have a common capability for all NES features. Defining separate UE capability information could lead to increasing signalling overhead. Network need to define a feature with sub capabilities defined within that so that the sub features can be included and the complexity of the implementation can also be reduced.

Proposal 4: RAN2 may support introduction of a common UE capability indicator for NES feature in Rel-19 allowing network to enable feature differentiation through sub-feature options.

## SSB Adaptation

Below is mentioned regarding the agreements on SSB adaptation in RAN2#129, 1. RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell. 2. RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. 3. For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

SSB adaptation and OD-SSB both aim to save energy by reducing unnecessary SSB transmissions. Since they are unlikely to be applied together for a UE, a unified approach can simplify signaling.

A single MAC CE can indicate either SSB adaptation or OD-SSB, avoiding redundancy. This aligns with RAN1 progress and reduces implementation complexity.

Proposal 5: RAN2 should adopt a unified MAC CE design for signaling SSB adaptation and OD-SSB to optimize efficiency and streamline standardization.

In Rel-19 NES, SSB adaptation introduces varying SSB periodicities to enable energy-efficient operation, but the static configuration of SMTC may lead to mismatches where the UE misses SSB transmissions during measurements. To address this, we propose introducing multiple pre-configured SMTC configurations, each aligned with a specific SSB periodicity level (e.g., 20 ms, 160 ms, 320 ms). The network can signal the appropriate SMTC index alongside the SSB adaptation configuration, ensuring that the UE always uses an SMTC window that overlaps with expected SSB transmissions. 

Proposal 6: RAN2 may discuss introducing multiple pre-configured SMTCs, each associated with a specific SSB periodicity, with the network signaling the appropriate SMTC index to the UE in order to ensure proper alignment between SMTC windows and adapted SSB transmission periodicity.

# Conclusion and Proposal

We have the following proposals:

Proposal 1: RAN2 to discuss the configuration of additional or different PRACH resources for NES-capable UEs.

Proposal 2: RAN2 to discuss the possibility of the occurrence of RA-RNTI confusion with additional RACH resources.

Proposal 3: NES UEs should prioritize NES-specific resources and fall back to legacy resources only when needed, ensuring no impact on legacy UEs.

Proposal 4: RAN2 may support introduction of a common UE capability indicator for NES feature in Rel-19 allowing network to enable feature differentiation through sub-feature options.

Proposal 5: RAN2 should adopt a unified MAC CE design for signaling SSB adaptation and OD-SSB to optimize efficiency and streamline standardization.

Proposal 6: RAN2 may discuss introducing multiple pre-configured SMTCs, each associated with a specific SSB periodicity, with the network signaling the appropriate SMTC index to the UE in order to ensure proper alignment between SMTC windows and adapted SSB transmission periodicity.

# Reference

- RAN1#116 meeting minutes

- RAN1#119 meeting minutes

- RAN2#129 meeting minutes

- TS38.321

- TS38.304

# Annex

## Agreements during previous RAN1 meetings on PRACH adaptation in time domain

At RAN1#116 meeting, the following agreements were reached for this subtopic regarding PRACH adaptation and paging adaptation:

Agreement For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study - Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - For the additional PRACH resources, - Adaptation of PRACH resource periodicity/PRACH occasion - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle - Adaptation based on extending cell DRX operation for PRACH - Concentrating ROs in time domain Other options are not precluded

Agreement For the adaptation mechanisms of PRACH in time-domain - Support at least PRACH adaptation provided by gNB without UE trigger - FFS: PRACH adaptation with UE trigger - Note: UE trigger means UE requests adaptation of PRACH - Study at least the following, - Dynamic signaling and/or semi-static signaling of PRACH adaptation - Adaptation of PRACH transmission according to certain condition - Applicability to idle/inactive and/or connected mode UEs Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 UEs)

At RAN1 #117 meeting, the following was agreed:

For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed Above does not preclude discussion for the case where the configuration for additional PRACH resources

- contains legacy PRACH resources

Observation 1: From RAN1 point of view, only time AND frequency overlap, i.e. sharing, of PRACH resources for NES-capable and legacy UEs has been discarded.

Agreement At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule. - Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes. - Note: This mapping is not impacted by time domain PRACH adaptation - Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Observation 2: From RAN1 point of view, there should be no SSB-RO mapping ambiguity for additional PRACH resources in the case where there is no overlap, either in time or frequency, which is expected legacy behaviour.