3GPP TSG-RAN WG2 Meeting #129	bis					R2-2501868

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.5.4

Source:	Samsung

Title:	Adaptation of common signal transmissions

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss the RAN2 aspects of Paging clustering/bundling and PRACH adaptation in time domain.

# Discussion

## 2.1. Paging clustering/bundling/adaptation

RAN2 has agreed option-b (i.e. Extension of N and Ns) as baseline for R19 NES paging enhancement for paging clustering/bundling.

In option-b POs are clustered at the beginning of DRX cycle by extending the values of N to have increased interval between PFs (e.g. T/32, T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. For example, let’s say 32 POs are needed in a DRX cycle to meet the paging load in a cell. Network can configure 1 PF in DRX cycle and 32 POs per PF.

If cell supports paging clustering/bundling,

- it configures new parameters N’ and Ns’ for indicating number of paging frames and number of POs for UEs supporting paging clustering/bundling.

- Starting PDCCH monitoring occasion number (firstPDCCH-MonitoringOccasionOfPO’) of PO is also configured separately for paging clustering/bundling. Starting PDCCH monitoring occasion number of PO is optional as in legacy.

- PF_Offset’ is also configured separately for paging clustering/bundling for flexible allocation of PF with paging clustering/bundling and legacy PF.

Legacy UEs in the cell will apply legacy parameters N, Ns, PF_Offset and firstPDCCH-MonitoringOccasionOfPO (if configured) whereas NES UEs supporting paging clustering/bundling applies N’, Ns’. PF_Offset’ and firstPDCCH-MonitoringOccasionOfPO’.

Proposal 1: If cell supports paging clustering/bundling/adaptation,

- Starting PDCCH monitoring occasion number (firstPDCCH-MonitoringOccasionOfPO’) of PO is also configured separately for paging clustering/bundling. Starting PDCCH monitoring occasion number of PO is optional as in legacy.

In RRC_CONNECTED state, UE only receives SI change/emergency notifications which are common for all UEs (legacy UEs and UEs supporting paging clustering/bundling). There is no need to support paging clustering/bundling in RRC_CONNECTED. If supported, network will have to additionally transmit SI change/emergency notifications in POs configured by paging clustering/bundling configuration.

Proposal 2: Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED

## 2.2. PRACH adaptation in time domain

## 2.2.1 Configuration aspects

For network energy savings, legacy RACH resources can be configured sparsely and additional RACH resources can be configured dynamically as and when needed. RAN1 has discussed configuration of additional RACH resources in time and frequency domain and agreed the following:

RAN1#118b Agreements

- For configuration of additional PRACH resources, support both of the following:

  - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources

  - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources

RAN1#119 Agreements

- Frequency domain configuration:

  - At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH.

Discussion

Based on the above agreements,  following configuration are allowed for random access configuration/random access resource set

- Separate PRACH configuration index for the additional PRACH resources and legacy RACH resources + msg1-FrequencyStart is same for legacy RACH resources and additional PRACH resources

- Separate PRACH configuration index for the additional PRACH resources and legacy RACH resources + separate msg1-FrequencyStart for the legacy RACH resources and additional PRACH resources

- Same PRACH configuration index for the additional PRACH resources and legacy RACH resources + msg1-FrequencyStart is same for legacy RACH resources and additional PRACH resources

- Same PRACH configuration index for the additional PRACH resources and legacy RACH resources + separate msg1-FrequencyStart for the legacy RACH resources and additional PRACH resources

prach-ConfigurationIndex and msg1-FrequencyStart are signalled in RACH-ConfigGeneric IE. So, it’s straightforward to include additional RACH resources configuration in RACH-ConfigGeneric IE. An example is shown below. An alternate would be to signal a seperate RACH-ConfigCommon for additional RACH resources. However, this is unnecessary overhead as it would lead to duplicate signalling of several random access parameters.

RACH-ConfigGeneric information element

RACH-ConfigGeneric ::=              SEQUENCE {

prach-ConfigurationIndex            INTEGER (0..255),

msg1-FDM                            ENUMERATED {one, two, four, eight},

msg1-FrequencyStart                 INTEGER (0..maxNrofPhysicalResourceBlocks-1),

zeroCorrelationZoneConfig           INTEGER(0..15),

preambleReceivedTargetPower         INTEGER (-202..-60),

preambleTransMax                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200},

powerRampingStep                    ENUMERATED {dB0, dB2, dB4, dB6},

ra-ResponseWindow                   ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80},

...,

[[

prach-ConfigurationPeriodScaling-IAB-r16    ENUMERATED {scf1,scf2,scf4,scf8,scf16,scf32,scf64}              OPTIONAL,   -- Need R

prach-ConfigurationFrameOffset-IAB-r16      INTEGER (0..63)                                                 OPTIONAL,   -- Need R

prach-ConfigurationSOffset-IAB-r16          INTEGER (0..39)                                                 OPTIONAL,   -- Need R

ra-ResponseWindow-v1610                     ENUMERATED { sl60, sl160}                                       OPTIONAL,   -- Need R

prach-ConfigurationIndex-v1610              INTEGER (256..262)                                              OPTIONAL    -- Need R

]],

[[

ra-ResponseWindow-v1700                     ENUMERATED {sl240, sl320, sl640, sl960, sl1280, sl1920, sl2560} OPTIONAL    -- Need R

]],

[[

additionalROConfig			 SEQUENCE {

prach-ConfigurationIndex            INTEGER (0..255), OPTIONAL    -- Need R

msg1-FrequencyStart    INTEGER (0..maxNrofPhysicalResourceBlocks-1), OPTIONAL    -- Need R

// other parameters, if any

} OPTIONAL    -- Need R

]]

}

Proposal 3: Introduce prach-ConfigurationIndex and msg1-FrequencyStart for additional rach resouces in RACH-ConfigGeneric IE.

PRACH adpatation in time domain for NUL or SUL or both

RAN1 is discussing a general design for PRACH adpatation in time domain. There is no difference in random access configuration for SUL and NUL. So there is no reason to exclude PRACH adpatation in time domain for either carrier. Network can configure additional RACH resources in random access resource configuration of SUL or random access resource configuration of NUL or both.

Proposal 4: PRACH adpatation in time domain is supported for both NUL and SUL.

## 2.2.2 RACH adaptation activation in RRC_CONNECTED

RAN1 has agreed that for DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. This DCI is paging DCI and transmitted in PO and addressed to P-RNTI.

Inorder to receive this, UE needs to monitor PO in RRC_CONNECTED state. However, PO is monitored only if paging search space is configured in active DL BWP. In case paging search space is not configured in active DL BWP, UE cannot receive the DCI format 1_0. So other mechanism (such RRC Reconfiguration message or MAC CE or group DCI) for adaptation for additional PRACH resources may be needed.

Proposal 5: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1: if adaptation for additional PRACH resources is supported in active UL BWP, network ensures that paging search space is configured in active DL BWP.

Option 2: Introduce other mechanism (such RRC Reconfiguration message or MAC CE or group DCI) for adaptation for additional PRACH resources in RRC_CONNECTED.

If paging search space is configured in active DL BWP, UE can monitor PO in RRC_CONNECTED for DCI-based adaptation for additional PRACH resources. However, current requirement is that UE monitors any PO at least once every modification period. If adaptation faster than modification period is needed, this will not work. Alternate would be that for DCI-based adaptation for additional PRACH resources in RRC_CONNECTED, UE monitors any PO at least once every defaultPagingCycle similar to emergency notifications.

Proposal 6: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1: For DCI-based adaptation indication for additional PRACH resources in RRC_CONNECTED, UE monitors the indication in any paging occasion at least once every defaultPagingCycle.

Option 2: For DCI-based adaptation for additional PRACH resources in RRC_CONNECTED, UE monitors the indication in any paging occasion at least once every modification period.

UE can be configured with multiple BWPs in RRC_CONNECTED. Multiple BWPs can be configured with additional PRACH resources. The issue is which BWPs additional PRACH resources are activated when the DCI is received. In our view, for DCI-based adaptation for additional PRACH resources in RRC_CONNECTED, DCI activates the additional PRACH resources configured in the active UL BWP. This is a simple approach and does not additional changes to DCI.

Proposal 7: For DCI-based adaptation for additional PRACH resources in RRC_CONNECTED, DCI activates the additional PRACH resources configured in the active UL BWP.

# Conclusion

Proposal 1: If cell supports paging clustering/bundling,

- Starting PDCCH monitoring occasion number (firstPDCCH-MonitoringOccasionOfPO’) of PO is also configured separately for paging clustering/bundling. Starting PDCCH monitoring occasion number of PO is optional as in legacy.

Proposal 2: Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

Proposal 3: Introduce prach-ConfigurationIndex and msg1-FrequencyStart for additional rach resouces in RACH-ConfigGeneric IE.

Proposal 4: PRACH adpatation in time domain is supported for both NUL and SUL.

Proposal 5: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1: if adaptation for additional PRACH resources is supported in active UL BWP, network ensures that  paging search space is configured in active DL BWP.

Option 2: Introduce other mechanism (such RRC Reconfiguration message or MAC CE or group DCI) for adaptation for additional PRACH resources in RRC_CONNECTED.

Proposal 6: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1: For DCI-based adaptation indication for additional PRACH resources in RRC_CONNECTED, UE monitors the indication in any paging occasion at least once every defaultPagingCycle

Option 2: For DCI-based adaptation for additional PRACH resources in RRC_CONNECTED, UE monitors the indication in any paging occasion at least once every modification period

Proposal 7: For DCI-based adaptation for additional PRACH resources in RRC_CONNECTED, DCI activates the additional PRACH resources configured in the active UL BWP.

# References