3GPP TSG-RAN WG2 Meeting #129bis                                                        	R2-2501870

Wuhan, China, 7th – 11th April 2025

Source:	CATT

Title:	Consideration on LCP enhancement

Agenda Item:	8.7.4.1

Document for:	Discussion and Decision

Introduction

In this contribution, we will further discuss the issues on LCP enhancement including Bj issue in resource allocation and intra-UE prioritization for SR, our views are also given in this paper.

Discussion

Bj enhancement

RAN2#129 Agreements on LCP enhancements - No additional PBR is needed for priority adjusted data. - FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

Based on above agreements in RAN2#129 meeting[1], no PRB enhancement will be introduced in LCP procedure, but one FFS discuss on Bj remained for further discussion.

According to legacy procedure as below, the logical channels selected for the UL grant are allocated resources in a decreasing priority order with Bj >0 (step 1), and Bj will be updated (step 2), then logical channels selected are allocated remaining resources in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first (step 3).

TS 35.321 5.4.3.1.3 Allocation of resources Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover. The MAC entity shall, when a new transmission is performed: 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. NOTE 1: The value of Bj can be negative.

Based on legacy procedure, the logical channel with Bj > 0 will be selected in the first round of resource allocation. However, considering that the additional priority of LCH will give priority-adjusted data more opportunity to be scheduled, after using of the last UL grant, it is very possible that there is priority-adjusted data with Bj <=0 before the first round of resource allocation. In order to avoid the priority-adjusted data missing this UL grant and to be obsolete in the first round of resourc allocation, it is better not to consider the limitation of Bj > 0 and make the priority-adjusted data to be transmitted as soon as possible.

If the limitation of Bj > 0 is cancelled for LCH with an upgraded priority in first round of resource allocation, the further issue is how much data for this LCH will be included in the UL grant. In legacy procedure, the default stop-condition is to meet the PBR, in order to simply the procedure and align to agreement no further enhancement to PBR, it is suggested to leave this issue to UE implementation.

Hence, considering the resource allocation procedure, it is proposed:

Proposal 1: When allocating resource for LCH with priority-adjusted data in the first round, it is not restricted by Bj>0.

Proposal 2: It is UE implementation to determine the data amount in the first round of resource allocation for LCH with priority-adjusted data and negative Bj.

Intra-UE prioritization

RAN2#129 Agreements on LCP enhancements - There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. - Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact - FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination

Based on above agreements, the priority of UL grant will apply the additional LCP priority. But it is still FFS whether additional LCP priority is used for SR priority determination.

TS 38.321 5.4.4 Scheduling Request As long as at least one SR is pending, the MAC entity shall for each pending SR: 1> if the MAC entity has no valid PUCCH resource configured for the pending SR; and 1> if there is no ongoing RACH-less LTM cell switch; and 1> if rach-LessHO is not configured: 2> initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR. 1> else, for the SR configuration corresponding to the pending SR: 2> when the MAC entity has an SR transmission occasion on the valid PUCCH resource for SR configured; and 2> if sr-ProhibitTimer is not running at the time of the SR transmission occasion; and 2> if the PUCCH resource for the SR transmission occasion does not overlap with a measurement gap: 3> if the PUCCH resource for the SR transmission occasion does not overlap with any of a UL-SCH resource whose simultaneous transmission with the SR is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, an SL-SCH resource, or an SL-PRS resource; or 3> if the MAC entity is able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource; or 3> if the MAC entity is configured with lch-basedPrioritization, and the PUCCH resource for the SR transmission occasion does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or with the PUSCH duration of a MSGA payload, and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5 overlaps with any other UL-SCH resource(s), and the physical layer can signal the SR on one valid PUCCH resource for SR, and the priority of the logical channel that triggered SR is higher than the priority of the uplink grant(s) for any UL-SCH resource(s) where the uplink grant was not already de-prioritized and its simultaneous transmission with the SR is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCHgroups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, and the priority of the uplink grant is determined as specified in clause 5.4.1; or 3> if both sl-PrioritizationThres and ul-PrioritizationThres are configured and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5 overlaps with any UL-SCH resource(s) carrying a MAC PDU, and the value of the priority of the triggered SR determined as specified in clause 5.22.1.5 is lower than sl-PrioritizationThres and the value of the highest priority of the logical channel(s) in the MAC PDU is higher than or equal to ul-PrioritizationThres and any MAC CE prioritized as described in clause 5.4.3.1.3 is not included in the MAC PDU and the MAC PDU is not prioritized by upper layer according to TS 23.287 [19]; or 3> if an SL-SCH resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource, and either transmission on the SL-SCH resource is not prioritized as described in clause 5.22.1.3.1a or the priority value of the logical channel that triggered SR is lower than ul-PrioritizationThres, if configured; or 3> if an SL-SCH resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource, and the priority of the triggered SR determined as specified in clause 5.22.1.5 is higher than the priority of the MAC PDU determined as specified in clause 5.22.1.3.1a for the SL-SCH resource; or 3> if an SL-PRS resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-PRS resource, and either transmission on the SL-PRS resource is not prioritized as described in clause 5.22.1.3.1a or in the clause 5.22.1.3.5, or the priority value of the logical channel that triggered SR is lower than ul-PrioritizationThres, if configured; or 3> if an SL-PRS resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-PRS resource, and the priority of the triggered SR determined as specified in clause 5.22.1.5 is higher than the priority of the MAC PDU and SL-PRS, if available, determined as specified in clause 5.22.1.3.1a or the SL-PRS resource in clause 5.22.1.3.5: 4> consider the SR transmission as a prioritized SR transmission. 4> consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s), except for the overlapping uplink grant(s) whose simultaneous transmission is allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup; <Skip> 3> else: 4> consider the SR transmission as a de-prioritized SR transmission.

Regarding legacy procedure in above yellow marked part, the priority of SR is determined by the priority of logical channel triggering SR. If a LCH with additional priority triggers the SR, it is better to apply the additional priority for SR, which could give the SR higher pirority to be transmitted for XR data transmission and avoid the obsolete data.

If the triggerred SR is agreed to apply the additional priority of the LCH, one issue is, maybe, there is no LCP procedure for this LCH when SR is triggered, which means that the additional priority of the LCH needs to be specified at SR procedure. Based on above table on SR procedure, it can be seen that the spec impact is limited and it is feasible to introduce the additional priority of the LCH in SR procedure.

Proposal 3: The additional priority of LCH could be considered for the priority of SR in intra-UE prioritization.

Proposal 4: The determination of additional priority of LCH could be added in SR procedure.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: When allocating resource for LCH with priority-adjusted data in the first round, it is not restricted by Bj>0.

Proposal 2: It is UE implementation to determine the data amount in the first round of resource allocation for LCH with priority-adjusted data and negative Bj.

Proposal 3: The additional priority of LCH could be considered for the priority of SR in intra-UE prioritization.

Proposal 4: The determination of additional priority of LCH could be added in SR procedure.

Reference

- RAN2#129 meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei