3GPP TSG-RAN WG2 Meeting #129bis                                                               R2-2501871

Wuhan, China, 7th – 11th April 2025

Source:	CATT

Title:	Consideration on DSR Enhancement

Agenda Item:	8.7.4.2

Document for:	Discussion and Decision

Introduction

In this contribution, we will further discuss and give our views on the issues on DSR enhancement, including the co-existing of new DSR and legacy DSR, the handling of Control PDU and retransmission data, and the issue on non-delay-reporting data in PDU set.

Discussion

The enhanced DSR agreements in RAN2#128 [1] are as follows.

The co-existing of new DSR and legacy DSR

In RAN2#129bis meeting, one FFS issue as follows needs to be further discussed:

RAN2#129bis agreement: FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. 确饿哦e

The legacy DSR is reported per LCG based on the remainingTimeThreshold configured per LCG, hence the new DSR is also configured per LCG. It is possible that network could configure some LCG with legacy DSR including only one remaining time threshold and configure other LCG with new DSR including multiple reporting thresholds, which depends on the network requirement to the delay granularity for each LCG.

For UE side, the LCG(s) configured with legacy remainingTimeThreshold and the LCG(s) configured with the new reporting threshold(s) trigger the delay status report simultaneously, considering the new DSR could cover the report of only one pair of remaining time information and buffer size information for legacy DSR, it is preferable to use the new DSR for all the triggered LCH. In the new DSR, for the LCG configured with remainingTimeThreshold, the introduced indication will be set based on new DSR format, then network could interpret the new DSR with same options and get the delay status information for the LCG without any mistake.

Proposal 1: The UE will use new DSR MAC CE when at least one LCG is configured with one or multiple reporting thresholds.

Handling of Control PDU and retransmission data

In legacy procedure, only one remainingTimeThreshold is used to trigger the DSR reporting. When R18 DSR is triggered, as the Control PDU and retransmission data in PDCP/RLC layer are high priority, so they will be included in delay-critical RLC data volume as follows.

TS 38.323 5.15 Data volume calculation for delay status reporting For the purpose of MAC delay status reporting, the transmitting PDCP entity shall consider the following as delay-critical PDCP data volume: - the delay-critical PDCP SDUs for which no PDCP Data PDUs have been constructed; - the PDCP Data PDUs that contain the delay-critical PDCP SDUs and have not been submitted to lower layers; - the PDCP Control PDUs; - for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13; - for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5. TS 38.322 5.5 Data volume calculation For the purpose of MAC delay status reporting, the UE shall consider the following as delay-critical RLC data volume: - delay-critical RLC SDUs and delay-critical RLC SDU segments that have not yet been included in an RLC data PDU; - RLC data PDUs pending for initial transmission, and containing a delay-critical RLC SDU or a delay-critical RLC SDU segment; - RLC data PDUs that are pending for retransmission (RLC AM). In addition, if a STATUS PDU has been triggered and t-StatusProhibit is not running or has expired, the UE shall estimate the size of the STATUS PDU that will be transmitted in the next transmission opportunity, and consider this as part of RLC data volume for MAC buffer status reporting and as part of delay-critical RLC data volume for MAC delay status reporting.

Now, for Rel-19 DSR, the network could configure multiple reporting thresholds to a LCG, generally, the Control PDU and retransmission data needs to be associated to one reporting threshold. One option is associated it to the shortest reporting threshold included in a specific DSR, another option is to associated it to the shortest configured reporting threshold. Compared with two options, the first option will avoid the further issue on how to define the remaining time if only Control PDU and retransmission data is associated to a reporting threshold for a LCG. However, we are preferable the second option, since it makes the Control PDU and retransmission data always to be considered as the highest priority data volume and the procedure is also simple.

Proposal 2: All PDCP/RLC control PDUs and retransmission data should always be associated to the smallest configured reporting threshold.

For the issue based on above second option, if there is only Control PDU and retransmitted data in a LCH, and the data triggered DSR in another LCH is associated to not-shortest configured reporting threshold, how to set the remaining time associated to the shortest configured reporting threshold in R19 DSR needs to be determined. it can be considered as UE implementation to choose a suitable value less than the shortest configured reporting threshold.

Proposal 3: For cases where the calculated buffer size for the smallest configured reporting threshold only contains PDCP/RLC Control PDUs and/or retransmission data, it is UE implementation to set the remaining time field for this reporting threshold in the DSR MAC CE.

Non-delay-reporting data in PDU set

Based on the agreement, the non-delay-reporting data ahead of delay-reporting data will be included in the R19 DSR. According to current running CR as below, in R19 DSR, the non-delay-reporting data will be associated to one reporting threshold based on the relationship of its COUNT value and the COUNT value of delay-reporting data.

RAN2#129 PostEmail511 PDCP Running CR for R19 XR： Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU that is associated with a COUNT value smaller than the largest COUNT value of the delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold. RAN2#129 PostEmail512 RLC Running CR for R19 XR： Non-delay-reporting RLC SDU: a non-delay-reporting RLC SDU associated with the i:th dsr-ReportingThreshold is a RLC SDU that is associated with a COUNT value smaller than the largest COUNT value of the delay-reporting RLC SDU associated with the i:th dsr-ReportingThreshold.

However, it is possible that the non-delay-reporting data is in PDU set level, i.e., the non-delay-reporting data in the PDU set could be distributed in different location of the buffer. In this case, some of them is associated to one dsr-ReportingThreshold, and the others is associated to another dsr-ReportingThreshold. Considering that these data are not configured with delay-reporting parameters and no delay reporting condition will be met, hence they can be considered as non-delay-critical data and be allowed to distribute in buffer with different dsr-ReportingThresholds, the network could schedule them based on different remaining time information in Rel-19 DSR, which is acceptable and feasible. Thus, to confirm current definition to no-delay-reporting data, for non-delay-reporting data ahead of delay-reporting data, it is not necessary to distinguish whether it is in a PDU set or not, the data volume for non-delay-reporting data in a PDU set ahead of delay-reporting data will be included in the data volume for adjacent delay-reporting data in buffer, not calculated in PDU set level.

Proposal 4: The data volume calculation for non-delay-reporting data ahead of delay-reporting data will not be carried out in the PDU set level.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: The UE will use new DSR MAC CE when at least one LCG is configured with one or multiple reporting thresholds.

Proposal 2: All PDCP/RLC control PDUs and retransmission data should always be associated to the smallest configured reporting threshold.

Proposal 3: For cases where the calculated buffer size for the smallest configured reporting threshold only contains PDCP/RLC Control PDUs and/or retransmission data, it is UE implementation to set the remaining time field for this reporting threshold in the DSR MAC CE.

Proposal 4: The data volume calculation for non-delay-reporting data ahead of delay-reporting data will not be carried out in the PDU set level.

Reference

- RAN2#129 meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei