3GPP TSG-RAN WG2 Meeting #129bis                                                   R2-2501873

Wuhan, China, 7th – 11th April 2025

Source:	CATT

Title:	Discussion on XR Rate Control

Agenda Item:	8.7.6

Document for:	Discussion and Decision

Introduction

In RAN2#129 meeting, regarding to XR rate control, the below agreements were agreed [1]:

Assistance information:

RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2.

We may revisit UAI option based on SA2/RAN3 reply

Rate value signalling:

Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc.

We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed

UE bit rate query:

Working assumption:

    - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE.

    - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy).

Companies to check with their SA4 colleagues whether there are any issues with this

In the current contribution, we further discuss the below three key leftover issues on XR rate control:

- Rate value signalling.

- Per QoS flow vs. Per DRB MAC CE.

- FFS on the bit rate type.

Discussion

Rate value signalling

In RAN2#129 meeting, RAN2 reached the agreement that a new table for XR rate control will be introduced. FFS on the distribution (exponential, linear), codepoints etc. To solve this issue, we want to further share our view from the below two aspects:

- Function: the goal in the current WID is to specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion. That is to say, once the gNB detects UL congestion happens, the gNB can send the Bitrate MAC CE to UE to achieve the reduction of UL bitrate. From technical points of view, the gNB has more comprehensive information about the resource usage of the current cell, the responsibility of gNB is to recommend one lower bitrate to UE to ease the UL congestion. Then leave to gNB implementation, one lower than current UL bitrate value will be sent to UE through the MAC CE. But there are no specification impacts for UE behavior once UE receives the bitrate MAC CE for XR from gNB. From the whole procedure point of view, for the accuracy requirement of the bitrate indication, there is no high requirement. From SA4’s LS reply[4], the bitrate range for XR service varies from 100kbps to 40Mbps, and coarser granularity (10%) may also be sufficient. All in all, we don’t need to chase the 1% granularity in the current release due to the function requirements.

- Overhead: to introduce per-QoS flow granularity in the recommended bitrate MAC CE, the overhead of the new MAC CE will be increased. Since in the RAN AS layer, the QoS flow is invisible after QoS to DRB mapping. To identify specific QoS flow in the new MAC CE, besides the QoS flow ID, the PDU session ID should also be included which means the overhead compared to legacy will increase dramatically. For the detailed distribution of the new table, there are many companies shared the same view that to achieve the 10% quantitation error, the overhead is about 6bits for exponential encoding and 10 bits for piecewise linear encoding. Totally combine the function and overhead, it is preferred to adopt the exponential method for the new introduced table, with the quantization of 10%.

Proposal 1: RAN2 adapts the exponential encoding for the new introduced bitrate table (6 bits) for Rel-19 XR rate control.

Per QoS flow vs. Per DRB MAC CE

There are two options recorded in the chairman notes of the last RAN2 meeting. One is to introduce per-QoS flow granularity in the recommended bitrate MAC CE, and the other is to reuse the legacy per DRB granularity in the recommended bitrate MAC CE with introducing RRC signalling to show which QoS flows are subject to bitrate adaptation. The main concern for this issue is that whether the CU-DU split case can be supported. To solve this concern, one related LS[2] had been sent to SA4 and RAN3 in RAN2#128 meeting. During RAN2#129 meeting, the LS reply was received from RAN3[5], it stated that RAN3 discussed it and agreed both options have an impact on the F1 interface. RAN3 will further discuss whether gNB-DU can support per QoS flow data bit rate control when TU is available. During the RAN#107, the TU had been allocated, hence from work procedure point of view, the only issue is the time. Considering the current situation, both options have F1 interface impacts, that is to say, the concern raised to support the per-QoS flow granularity is not still valid. In fact, the current issue is in charge by RAN2, we would like that RAN2 directly decide that the per QoS flow granularity can be supported. For the RRC approach, it can be further discussed as need.

Proposal 2: RAN2 supports per-QoS flow Bitrate MAC CE in Rel-19 XR rate control.

Proposal 3: RAN2 sends one LS to RAN3 to synchronize the RAN2 agreements about XR UL rate control.

FFS on the bit rate type

During the MAC specification running CR discussion, the below Editor’s Note was added for the bit rate type:

The UL Rate Control procedure provides the MAC entity with information on UL physical-layer bit rate available to a logical channel or a QoS flow.

Editor’s Note:	FFS whether this bit rate is physical-layer bit rate  (as in the legacy recommended bit rate procedure) or some other type of bit rate.

During the post email discussion, our view was that the bit rate should be application-layer source encoding rate. The opponent company raised the below two reasons:

- In Rel-15, the bit rate is called physical layer bit rate, but more detailed information is needed.

- For function achievement, it is not reasonable that to ask gNB MAC layer to estimate application-layer bit rate, e.g. it may not be able to know how a particular application may encapsulate its date.

In the TS 26.114, there is one Note recording the related information. It shows that the bit rate is physical-layer bit rate. And more information about the calculation method is also recorded.

NOTE 3:	The recommended/queried bitrate as signalled over the LTE and NR access is defined to be in kbps at the physical layer. The uplink/downlink bitrate at the physical layer is rUL/DL=kLkT, where Lkis the bit-length of the k-th successfully transmitted/received TB by the UE within the window T. In TS 36.321 and 38.321, a window length of 2000 ms is applied.

Hence, it is reasonable that in the current Rel-19 MAC specification to capture that the UL Rate Control procedure provides the MAC entity with information on UL physical-layer bit rate available to a logical channel or a QoS flow.

Proposal 4: RAN2 confirms that the bit rate used in the new introduced Bitrate MAC CE in Rel-19 XR rate control is physical-layer bit rate (as in the legacy recommended bit rate procedure).

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: RAN2 adapts the exponential encoding for the new introduced bitrate table (6 bits) for Rel-19 XR rate control.

Proposal 2: RAN2 supports per-QoS flow Bitrate MAC CE in Rel-19 XR rate control.

Proposal 3: RAN2 sends one LS to RAN3 to synchronize the RAN2 agreements about XR UL rate control.

Proposal 4: RAN2 confirms that the bit rate used in the new introduced Bitrate MAC CE in Rel-19 XR rate control is physical-layer bit rate (as in the legacy recommended bit rate procedure).

Reference

- R2-2501335	Report from session on R18 MBS, R18 QoE and R19 XR	Session chair (Huawei)	report

- R2-2411218 LS on XR UL bit rate control

- R2-2409273	LS on appropriate range and granularity of bit rate adaptation for XR applications	RAN2	LS out	NR_XR_Ph3-Core	To:SA4

- R2-2500056 Reply to LS on appropriate range and granularity of bit rate adaptation for XR applications

- R2-2501508 Reply LS on XR UL Rate Control

- TS 26.114 IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction (Release 18)