3GPP TSG-RAN WG2 Meeting #129bis                                                   R2-2501876

Wuhan, China, 7th – 11th April 2025

Source:	CATT

Title:	Discussion on the Control Plane Procedures

Agenda Item:	8.13.3

Document for:	Discussion and Decision

Introduction

In the past three RAN2 meetings, the control plane of multi-hop Relay were discussed. In this contribution, we will further discuss the following issues based on approach 1:

- How to handle RLF?

- Whether assistant information is needed for QoS split？

- How to perform the system information forwarding?

- How to perform the paging forwarding?

Discussion

RLF handling

With the introduction of multi-hop, there are three kinds of links as below:

- Sidelink 1: The sidelink between Remote UE and first Relay UE;

- Sidelink 2: The sidelink between Relay UEs;

- Uu link: The Uu link between the last Relay UE and gNB.

Figure-1  Multi-hop relay

When analyzing the RLF handling, we should discuss based on the above three kinds of links:

Table-1 RLF handling for multi-hop Relay

| Failed link | Legacy Rel-17 behaviors | Rel-19 behaviors |
| --- | --- | --- |
| Sidelink 1 | Remote UE detected PC5 RLF, key behaviors: - Release the PC5 Relay RLC channels between Remote UE and Relay UE; - Consider the PC5-RRC connection between Remote UE and U2N Relay is released; - If Remote UE is in CONNECTED, initiate RRC connection re-establishment. Relay UE detected PC5 RLF: - Release the PC5 Relay RLC channels between Remote UE and Relay UE; - Consider the PC5-RRC connection between Remote UE and U2N Relay is released; If Relay UE is in CONNECTED, send SUI to gNB. | Remote UE detected PC5 RLF: same as Rel-17 U2N Remote UE. First Relay UE detected PC5 RLF: same as Rel-17 U2N Relay UE. |
| Sidelink 2 | N/A | First Relay UE detected PC5 RLF: enhancement is possible. Send NotificationMessageSidelink message to Remote UE to indicate the failure between first Relay UE and Intermediate Relay (in case of more than 2 Relays) or between first Relay UE and last Relay UE (in case of only 2 Relays). Last Relay UE detected PC5 RLF: same as Rel-17 U2N Relay UE. |
| Uu link | Relay UE detected Uu RLF: - Send NotificationMessageSidelink message to remote UE. | Relay UE detected Uu RLF: enhancement is possible. - Send NotificationMessageSidelink message to its child Relay UE and its child relay UE should forward it the Remote UE. |

Based on the analysis in the above table, the following are proposed:

Proposal 1: In approach 1, when Remote UE detects PC5 RLF, its behaviors are same as Rel-17 U2N Remote UE.

Proposal 2: In approach 1, when the first Relay UE detects PC5 RLF between the Remote UE and the first Relay UE, its behaviors are same as Rel-17 U2N Relay.

Proposal 3: In approach 1, when the first Relay UE detects PC5 RLF between the first Relay UE and Intermediate Relay/last Relay UE, it can send NotificationMessageSidelink to the Remote UE to indicate the failure between first Relay UE and Intermediate Relay/last Relay UE.

Proposal 4: In approach 1, when the last Relay UE detects PC5 RLF, its behaviors are same as Rel-17 U2N Relay UE.

Proposal 5: In approach 1, when the last Relay UE detects Uu RLF, it can send NotificationMessageSidelink to its child Relay UE, and its child Relay UE can forward it to the Remote UE.

QoS split for multi-hop relay

Regarding to the QoS split, in RAN2#128 meeting, the following agreement was reached for approach 1:

Agreements:

For the baseline procedure, the PDB for each hop is provided by the network according to gNB implementation.   This does not preclude the possibility of assistance information from the UE(s).

Based on the above agreement, it should further discuss whether the assistance information from the UE(s) is needed in order to assistant the NW to perform QoS split.

In Rel-17 U2N Relay, the NW performs the QoS split based on its implementation, no UE assistant information is reported. In case of multi-hop, although the number of hops is higher than Rel-17 U2N relay, it is still unnecessary to report assistance information. The main reason is that considering both the remote UE and the Relay UE may be moving, the PC5 link quality between two UEs is not stable and can be changed dynamically. However, the QoS split is semi-static. Hence, the PC5 link channel quality is not very trustable when performing QoS split.

Proposal 6:  In approach 1, UE assistance information is not needed for QoS split.

System information forwarding

In RAN2#128 meeting, regarding to the system information forwarding in case of multi-hop, the following agreements were reached:

Agreement:

In multi-hop, the U2N Remote UE acquires the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s).  FFS how to perform the forwarding and whether an intermediate relay UE can forward available SI directly (rather than retrieving it from the last relay UE).

In RAN2#129 meeting, according to the discussion, the below agreement related to the issue we discussed was listed as below:

Agreement:

An intermediate relay UE that is RRC_CONNECTED may use end-to-end RRC signaling to obtain its system information directly from its connected cell (i.e., behaving as a remote UE).

The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list.  FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.

For Rel-19 multi-hop U2N relay case, there is one case that for the RRC_IDLE/RRC_INACTIVE remote UE, its selecting path includes RRC_CONNECTED intermediate relay UE. For the RRC_CONNECTED intermediate relay UE, RAN2 already had agreement that it may use end-to-end RRC signaling to obtain its system information directly from its connected cell(i.e., behaving as a remote UE). The other case is that for the request system information, the intermediate relay UE has stored the valid SIB(s), then it can directly respond instead of forwarding the request to the parent node to further save signaling and time.

Proposal 7:  In approach 1, the intermediate Relay UE is allowed to forward the valid SIB(s) requested by its child UE(s) directly (rather than retrieving it from the last Relay UE) if it has stored the valid SIB(s).

Paging forwarding

In RAN2#129 meeting, some agreements had been reached for Rel-19 multi-hop U2N relay case. In the current case, we focus on the below FFS:

Agreements:

An intermediate relay UE in RRC_CONNECTED releases its paging-related information in the parent UE at least for itself.  FFS whether it also releases it for the child UEs and how the child UEs receive the paging when the intermediate relay UE is in RRC_CONNECTED.

This situation is similar as what we discussed in the system information part. For the RRC_IDLE/RRC_INACTIVE remote UE, there will be RRC_CONNECTED intermediate relay UE involved. Once the related intermediate relay UE in RRC_CONNECTED, it will release its paging-related information in the parent UE at least for itself. But it should not release the paging-related information for the child UEs. The basic procedure for require and forward paging can be reused as much as possible.

Proposal 8:  Once the intermediate relay UE in RRC_CONNECTED mode, it will not release the paging-related information for its child UEs, reuse the legacy PC5-RRC signaling for request and receive paging.

Conclusion

According to the analysis in section 2, for Approach 1, it is proposed:

RLF handling

Proposal 1: In approach 1, when Remote UE detects PC5 RLF, its behaviors are same as Rel-17 U2N Remote UE.

Proposal 2: In approach 1, when the first Relay UE detects PC5 RLF between the Remote UE and the first Relay UE, its behaviors are same as Rel-17 U2N Relay.

Proposal 3: In approach 1, when the first Relay UE detects PC5 RLF between the first Relay UE and Intermediate Relay/last Relay UE, it can send NotificationMessageSidelink to the Remote UE to indicate the failure between first Relay UE and Intermediate Relay/last Relay UE.

Proposal 4: In approach 1, when the last Relay UE detects PC5 RLF, its behaviors are same as Rel-17 U2N Relay UE.

Proposal 5: In approach 1, when the last Relay UE detects Uu RLF, it can send NotificationMessageSidelink to its child Relay UE, and its child Relay UE can forward it to the Remote UE.

QoS split

Proposal 6:  In approach 1, UE assistance information is not needed for QoS split.

System information forwarding

Proposal 7:  In approach 1, the intermediate Relay UE is allowed to forward the valid SIB(s) requested by its child UE(s) directly (rather than retrieving it from the last Relay UE) if it has stored the valid SIB(s).

Paging forwarding

Proposal 8:  Once the intermediate relay UE in RRC_CONNECTED mode, it will not release the paging-related information for its child UEs, reuse the legacy PC5-RRC signaling for request and receive paging.

Reference

- RP-242349  New WID on NR sidelink multi-hop relay

- TR23700-03   Study on system enhancement for Proximity based Service (ProSe) in the 5G System (5GS) Phase 3

- S2-2407207   LS on security considerations for 5G ProSe multi-hop relays

- R2-2411184  LS to SA2 on relay discovery announcement

- R2-2410006  Report of [POST127][402][Relay] Multi-hop relay control plane

- S2-2501334  Reply LS on relay discovery announcement (S2-2500045/R2-2411184)

- TS23.304  Proximity based Services (ProSe) in the 5G System (5GS)