3GPP TSG-RAN WG2 Meeting #129bis	R2-2501880

# Wuhan, China, 7 – 11 April 2025

Agenda item:		8.7.1

Source:	Xiaomi

Title:	UE capabilities for XR

Document for:		Discussion and Decision

Introduction

WID of Rel-19 XR RP-250107 [1] has the following RAN2 related scope:

- Support and specify multi-modality awareness for QoS flows in both DL and UL RAN [RAN3]: - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG. - Specify the following user plane enhancements [RAN2]: - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Specify uplink congestion signalling [RAN2, RAN3]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

In this contribution, we discuss the UE capabilities for XR, with the focus on RAN2-led items.

Discussion

General

In Rel-18 XR, separate UE capabilities are introduced for different XR features defined by RAN2 (i.e. additionalBS-Table-r18, delayStatusReport-r18, cg-RetransmissionMonitoringDisabling-r18, non-IntegerDRX-r18, supportOfPDU-SetDiscard-r18, psi-BasedDiscard-r18, ul-TrafficInfo-r18, supportOfSN-GapReport-r18).

In Rel-19 XR, considering the tradeoff between UE implementation and network implementation, some discussion might be needed on whether to follow Rel-18 approach (i.e. define separate UE capabilities for different XR features), or to define a generic Rel-19 XR capability which can include multiple features.

Proposal 1: RAN2 to discuss whether to define separate UE capabilities for different XR features (as in Rel-18 XR), or to define a generic Rel-19 XR capability which can include multiple features.

In the following sections of the contribution, it is assumed that separate UE capabilities are defined. This can be revisited if RAN2 agree to define a generic Rel-19 XR capability.

RRM measurement gaps/restrictions related enhancements

RAN1 has agreed to introduce DCI based measurement gap skipping solution, which is covered in the initial RAN1 UE features list for Rel-19 NR (R1-2501397 [2]) agreed in RAN1#120 meeting, as copied below:

In RAN2#129 meeting, RAN2 agreed that RRC based solution is not pursued: “RAN2 will not work on semi-static MG skipping solutions unless requested by RAN4/RAN1”.

It is assumed that RAN1 will continue working on UE capability for DCI based measurement gap skipping solution, and inform RAN2 via UE feature list. Eventually RAN2 will implement RAN1 feature list, as in previous releases.

Observation 1: UE capability for DCI based measurement gap skipping is handled in RAN1 feature list.

Multi-modality support

As discussed in RAN#107 meeting with the updated WID RP-250107 [1] (related part of WID is copied below), multi-modality only has RAN3 impact, without RAN2 impact. Therefore there is no need to introduced AS UE capability for multi-modality in Rel-19.

Proposal 2: RAN2 to confirm that AS UE capability for multi-modality is not needed in Rel-19.

Scheduling enhancements

LCP enhancements

RAN2 agreed on the following regarding LCP enhancements:

RAN2#127bis - As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. - We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold. RAN2#128 - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

It is proposed to introduce an optional UE capability for dynamic logical channel priority based on delay status of buffered data. Although the configuration of this feature is independent of DSR, some discussion might be needed on whether this feature should have Rel-18 DSR (i.e. delayStatusReport-r18) as its pre-requisite.

Regarding optional capability of fallback to default priority in the 2nd round of LCP, there was discussion on how UE capability is handled in R2-2500765 [3]. Three options are listed: no capability, capability without signalling, capability with signalling. The capability discussion is also related to whether to have RRC configuration to enable/disable the fallback to default priority in the 2nd stage of LCP, which is part of email discussion "[POST129][510][XR] RRC running CR (Huawei)". Discussion in RAN2 is needed on the capability aspect.

Proposal 3: An optional UE capability (e.g. lcp-PriorityAdjustment-r19) is introduced to indicate the support of dynamic logical channel priority based on delay status of buffered data. FFS whether a UE supporting this feature shall also indicate support of delayStatusReport-r18.

Proposal 4: RAN2 to discuss the capability handling regarding fallback to default priority in the 2nd round of LCP, e.g. whether it is a capability with or without signalling.

DSR enhancements

In Rel-18, optional UE capability for DSR is introduced, and corresponding part in TS 38.306 is copied below:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| delayStatusReport-r18 Indicates whether the UE supports the delay status report of the buffered data as specified in TS 38.321 [8], TS 38.331 [9], TS 38.323 [16] and TS 38.322 [36]. | UE | No | No | No |

RAN2 agreed on the following regarding Rel-19 DSR enhancements:

RAN2#127 - Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. RAN2#128 - The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

It is proposed to introduce an optional UE capability for enhanced DSR reporting with multiple thresholds. It is straightforward that the Rel-19 DSR capability has Rel-18 DSR capability delayStatusReport-r18 as pre-requisite, so that network can have the flexibility to choose to configure UE to use either Rel-18 DSR or Rel-19 DSR.

In addition, another capability is needed to indicate that UE can include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. This feature depends on previous feature of enhanced DSR reporting with multiple thresholds.

Proposal 5: An optional UE capability (e.g. enhancedDelayStatusReport-r19) is introduced to indicate the support of enhanced delay status report of the buffered data associated with multiple thresholds. A UE supporting this feature shall also indicate support of delayStatusReport-r18.

Proposal 6: An optional UE capability (e.g. delayStatusReportNonCriticalData-r19) is introduced to indicate the support of including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. A UE supporting this feature shall also indicate support of enhanced delay status report.

RLC enhancements

Timely RLC retransmissions

RAN2 agreed on the following regarding timely RLC retransmissions:

RAN2#128 - Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them. RAN2#129 - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC. - We have separate thresholds for autonomous reTx and for polling.

From RAN2 agreements, autonomous retransmission and polling enhancements are separate and independent features. Given that autonomous retransmission and polling enhancements have separate thresholds, it is straightforward to assume that those thresholds are different from the thresholds of DSR (as in RRC running CR under review, the thresholds are defined as autonomousReTxThreshold-r19 and enhancedPollingThreshold-r19). Therefore those two features do not have pre-requisites.

Proposal 7: An optional UE capability (e.g. autonomousRLC-Retx-r19) is introduced to indicate the support of autonomous RLC retransmission based on delay status. The capability does not have pre-requisites.

Proposal 8: An optional UE capability (e.g. enhancedPolling-r19) is introduced to indicate the support of enhanced polling based on delay status. The capability does not have pre-requisites.

Unnecessary retransmission avoidance

RAN2 agreed on the following regarding unnecessary retransmissions avoidance:

RAN2#127bis - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. ● TX side stops transmissions of an outdated SDU ● RX side abandons the SDU based on a local timer ● Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified RAN2#128 - A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. RAN2#129 - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - The new RLC timer at the RX is per RLC entity.

Main discussion point for UE capability is whether there is a joint or separate UE capabilities for UE Tx and Rx operation, which was briefly discussed in RAN2#129 meeting, with related meeting notes copied below:

DISCUSSION:

- QCM thinks whether it is possible to stop retransmission should be a UE capability.

- Sharp thinks what QCM wants is purely Rx based solution.

- Ericsson agrees we will need some capability and is OK to have separate ones for Rx and Tx side.

- Lenovo would prefer not to decouple the two parts.

It should be noted that UE Tx and Rx operation is for different directions even though a single RLC AM entity handles both DL and UL: UE Tx operation is for UL data, and UE Rx operation is for DL data. Currently in RLC running CR under review (email discussion “[POST129][512][XR] RLC running CR (vivo)”), there are separate configurations for Tx and Rx side operation (e.g. stopReTxObsoleteSDU for Tx side, and t-RxDiscard for Rx side). To avoid unnecessary transmission in DL, the joint operation of gNB Tx and UE Rx is needed. Similarly, to avoid unnecessary transmission in UL, the joint operation of UE Tx and gNB Rx is needed. This motivates separate UE capabilities for UE Tx and Rx operations.

Proposal 9: Separate UE capabilities are introduced for UE Tx and Rx operations for unnecessary retransmission avoidance.

Although one of the motivations to introduce unnecessary retransmission avoidance is to handle PDU set discard (UE capability supportOfPDU-SetDiscard-r18), enhanced UE Tx / Rx operation in Rel-19 is also beneficial for Rel-15 PDCP SDU discard, which is mandatory for UE. Therefore UE capabilities for UE Tx and Rx operations for unnecessary retransmission avoidance do not have pre-requisites.

Proposal 10: An optional UE capability (e.g. supportOfStopRetxObsoleteSDU-r19) is introduced to indicate the support of stopping retransmission of obsolete SDUs. The capability does not have pre-requisites.

Proposal 11: An optional UE capability (e.g. rlc-Rx-DiscardTimer-r19) is introduced to indicate the support of RLC receiver timer to abandon obsolete SDUs. The capability does not have pre-requisites.

Rate control

For legacy recommended bit rate MAC CE, following UE capabilities are introduced.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| recommendedBitRate Indicates whether the UE supports the bit rate recommendation message from the gNB to the UE as specified in TS 38.321 [8]. | UE | No | No | No |
| recommendedBitRateMultiplier-r16 Indicates whether the UE supports the bit rate multiplier for recommended bit rate MAC CE as specified in TS 38.321 [8], clause 6.1.3.20. This field is only applicable if the UE supports recommendedBitRate. | UE | No | No | No |
| recommendedBitRateQuery Indicates whether the UE supports the bit rate recommendation query message from the UE to the gNB as specified in TS 38.321 [8]. This field is only applicable if the UE supports recommendedBitRate. | UE | No | No | No |

RAN2 agreed on the following regarding rate control:

RAN2#128 - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. RAN2#129 - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed - Working assumption: ● Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. ● The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy).

Similar to legacy UE capability for recommendedBitRate, an optional UE capability can be introduced to indicate the support of UL rate control MAC CE. Given that Rel-19 feature is targeted for XR, which is different for the scenario of legacy recommended bit rate feature (MMTEL IMS voice and video enhancements, as from TS 38.300 clause 16.2.1), there is no need for Rel-19 rate control feature to have legacy recommended bit rate feature as dependency.

Proposal 12: An optional UE capability (e.g. ul-RateControl-r19) is introduced to indicate the support of UL rate control MAC CE from the gNB to the UE. The capability does not have pre-requisites.

If the working assumption for the rate query MAC CE is confirmed, then a corresponding UE capability can be introduced. In legacy, recommendedBitRateQuery-r16 has recommendedBitRate as pre-requisite. Similar principle can be applied for bit rate query message sent from UE to gNB.

Proposal 13: If working assumption is confirmed, an optional UE capability (e.g. ul-RateQuery-r19) is introduced to indicate the support of bit rate query message from the UE to the gNB. A UE supporting this feature shall also indicate support of UL rate control MAC CE.

Granularity of UE capability

All the above UE capabilities are L2 protocol related UE capabilities, similar to Rel-18 XR UE capabilities (e.g. supportOfPDU-SetDiscard-r18, non-IntegerDRX-r18, delayStatusReport-r18). XR UE capabilities defined by RAN2 in Rel-18 are per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF. Such granularity is applicable to Rel-19 XR UE capabilities defined by RAN2.

Proposal 14: Rel-19 XR UE capabilities defined by RAN2 are per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

Conclusion

In this contribution, we discuss the UE capabilities for XR, with the focus on RAN2-led items, and have the following observation:

Observation 1: UE capability for DCI based measurement gap skipping is handled in RAN1 feature list.

We proposal the following:

Proposal 1: RAN2 to discuss whether to define separate UE capabilities for different XR features (as in Rel-18 XR), or to define a generic Rel-19 XR capability which can include multiple features.

Proposal 2: RAN2 to confirm that AS UE capability for multi-modality is not needed in Rel-19.

Proposal 3: An optional UE capability (e.g. lcp-PriorityAdjustment-r19) is introduced to indicate the support of dynamic logical channel priority based on delay status of buffered data. FFS whether a UE supporting this feature shall also indicate support of delayStatusReport-r18.

Proposal 4: RAN2 to discuss the capability handling regarding fallback to default priority in the 2nd round of LCP, e.g. whether it is a capability with or without signalling.

Proposal 5: An optional UE capability (e.g. enhancedDelayStatusReport-r19) is introduced to indicate the support of enhanced delay status report of the buffered data associated with multiple thresholds. A UE supporting this feature shall also indicate support of delayStatusReport-r18.

Proposal 6: An optional UE capability (e.g. delayStatusReportNonCriticalData-r19) is introduced to indicate the support of including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. A UE supporting this feature shall also indicate support of enhanced delay status report.

Proposal 7: An optional UE capability (e.g. autonomousRLC-Retx-r19) is introduced to indicate the support of autonomous RLC retransmission based on delay status. The capability does not have pre-requisites.

Proposal 8: An optional UE capability (e.g. enhancedPolling-r19) is introduced to indicate the support of enhanced polling based on delay status. The capability does not have pre-requisites.

Proposal 9: Separate UE capabilities are introduced for UE Tx and Rx operations for unnecessary retransmission avoidance.

Proposal 10: An optional UE capability (e.g. supportOfStopRetxObsoleteSDU-r19) is introduced to indicate the support of stopping retransmission of obsolete SDUs. The capability does not have pre-requisites.

Proposal 11: An optional UE capability (e.g. rlc-Rx-DiscardTimer-r19) is introduced to indicate the support of RLC receiver timer to abandon obsolete SDUs. The capability does not have pre-requisites.

Proposal 12: An optional UE capability (e.g. ul-RateControl-r19) is introduced to indicate the support of UL rate control MAC CE from the gNB to the UE. The capability does not have pre-requisites.

Proposal 13: If working assumption is confirmed, an optional UE capability (e.g. ul-RateQuery-r19) is introduced to indicate the support of bit rate query message from the UE to the gNB. A UE supporting this feature shall also indicate support of UL rate control MAC CE.

Proposal 14: Rel-19 XR UE capabilities defined by RAN2 are per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

- References

[1] RP-250107, Nokia (Rapporteur), "Revised WID: XR (eXtended Reality) for NR Phase 3"

[2] R1-2501397, Moderators (AT&T, NTT DOCOMO, INC.), "Initial RAN1 UE features list for Rel-19 NR after RAN1 #120"

[3] R2-2500765, Nokia, Nokia Shanghai Bell, "LCP enhancements"