3GPP TSG-RAN WG2 Meeting #129bis							       R2-2501890

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.2.4

Source: 	NEC

Title:	Ambient-IoT Data Transmission

Document for:	Discussion/Decision

- Introduction

The Rel-19 WID for Ambient IoT (also referred to as A-IoT) includes following RAN2-led objectives [1].

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included. - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

This contribution discusses the data transmission aspects based on the RAN2-led objectives and the latest RAN1 and RAN2 agreements.

- Discussion

  - D2R Data transmission

    - Segmentation and message size

In RAN2#129 meeting [2], there are agreements on segmentation and message size:

1.	To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size.   If not possible, FFS if the 1 bit is sufficient.

2.	Segment retransmission is supported.

3.	For segment  retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

4.	R2D segmentation is not supported for R19 A-IoT.

Message size indication

In addition, in RAN2#127bis [3] meeting, there is an agreement on message size reporting shown as below:

- From RAN2 perspective, it is beneficial for the reader to know an estimate of expected D2R message size.  Two options can be captured: 1) from the CN and 2) from the device (simple message size indication).    Capture the advantages/disadvantages.

As we agreed it is beneficial for the reader to know an estimate of expected D2R message size, the main difference between two options from our perspective would be on different efficiency. Device based option is more dynamic reported than CN based option, therefore more real and accurate message size can be provided in each D2R transmission. With this real-time message size indication, reader can do the best scheduling to guarantee high efficiency of D2R message transmission. Apparently, the disadvantage of device based message size indication is more transmission overhead in PRDCH and obviously additional spec impact on indicator signalling design. However, for AIoT system it is worthy to have such real-time/frequent report since higher scheduling efficiency is of much importance for AIoT device. Lower efficiency D2R scheduling may result in additional D2R transmission with more power consumption. If device run out of power, energy harvesting and delay transmission would be envisioned which needs another round of random access procedure and large delay for this round of inventory/command task. From perspectives of both latency and energy consumption of AIoT devices, more efficient scheduling with relatively higher transmission overhead is preferred.

D2R message size reported by device can provide more efficient scheduling of D2R transmissions.

D2R message size reported by device can potentially provide D2R transmission with low latency and low power consumption.

D2R message size reported by device has larger spec impact and signaling overhead on extra design of report format.

In terms of device based D2R message size report, simple message size indication with less spec impact and simple signalling design as well as overhead is preferred. Targeting this, a ratio based message size reporting mechanism is proposed, where a ratio defined as remaining message size over previous scheduled size at device side is designed. By introducing this metric, efficient data scheduling for A-IoT system can be supported as shown in Fig. 1.

Fig. 1 Ratio based message size reporting mechanism for A-IoT communication

To be specific,

Step 1. A first PRDCH with TBS1 indication of a subsequent PDRCH is transmitted from reader to device.

Step 2. After device receives the PRDCH and obtains TBS1, a ratio r = remaining message size / TBS1 = (total D2R message size - TBS1) / TBS1 will be calculated by device. Next, device reports this ratio together with the scheduled D2R transmission in corresponding PDRCH.

Step 3. When reader receives PDRCH successfully, reader calculates the remaining D2R message size by multiply the reported r with TBS1. Based on the calculated remaining message size, reader could indicate TBS2 in another PRDCH transmission for another PDRCH scheduling. This procedure will repeat until the completion of all D2R message transmission.

Ratio based message size report is a simple and efficient mechanism for A-IoT message scheduling.

Ratio based message size reporting can be supported for efficient scheduling in A-IoT.

Transmission and Retransmission of a segment

As per the agreement of RAN2#129bis on retransmission, the start point of retransmission of segment can be determined by the offset, such as the offset plus one. However, how to perform the new transmission is not decided yet.

For transmission of each data segment, device only needs to know the starting and ending position for current transmission requested by reader. This can be implemented by following steps:

- Step 1: Upon D2R transmission of segment#0, the device stores a “start position” (starting position of segment#0) and a “next start position” (starting position of the next data segment to be transmitted);

- Step 2: In scheduling for D2R segment#1, reader provide a starting information and a size information to device. If segment#0 is successfully decoded, the starting information may indicate D2R segment#1 to start from the “next start position”; If segment#0 is not successfully decoded, the starting information may indicate D2R segment#1 to start from the “start position”;

- Step 3: Based on the starting information, the device considers the starting position of segment#1 to be the “start position” or the “next start position”. Then device construct segment#1 for transmission with the starting position and the size information.

Some examples can be seen in following figures:

Example of segment#0

Fig.2 Example of segment#1 (ACK for segment#0)

Fig.3 Example of segment#1: NACK for segment#0, size of segment#1 < size of segment#0)

Upon each segment transmission, the device stores a start position (starting position of the data segment for current transmission) and a next start position (starting position of the next data segment to be transmitted).

Upon scheduling for next segment, reader needs to provide a size information.

Upon receiving the D2R transmission scheduling information, device inform upper layer to construct the requested segment with the starting position and the size information.

Similar to the retransmission of one segment, when the upper layer data without segment is lost, the reader may send the offset and can set it as ‘0’ to indicate the retransmission of the unsegmented upper layer data.

RAN2 to confirm that the offset can be used to trigger the retransmission of the unsegmented upper layer data.

However, the content of MSG3 is mainly the device identifier, which may be stored in a memory other than that of the upper layer data for reading. Therefore, reader can resend the MSG2 for one device to trigger the retransmission of MSG3.

RAN2 to confirm that MSG2 is used to trigger the retransmission of MSG3.

  - R2D Data transmission

    - Repetition

Transmission via the A-IoT air interface between the reader and the A-IoT device may encounter signal fading or blockage, just like traditional Uu interface. This may lead to D2R data transmission failure. In RAN2#127bis [3] meeting, there is an agreement on repetition of R2D transmission (e.g., upper layer command) by the reader to trigger re-sending the same D2R response shown as below:

1 In case of D2R data transmission failure, device follows the reader instruction:

	it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and

	reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R).

That is, the reader may send a command several times. To control the number of repetitions, the following methods can be considered:

- The specific number of repetition is visible to the reader from CN to control the repetition.

- The survival time of a command is visible to the reader. And the specific repetition times depend on the reader implementation.

The control information for repetition (e.g. specific number of repetitions or survival time of a command) is useful to be visible to the reader from CN.

  - MAC PDU

In RAN2#129 meeting [2], there are agreements on MAC PDU format:

1.	Aim to design simple MAC PDU format design

2.	Support multiplexing of information for multiple devices in R2D message for msg2.  FFS others for multicast messages

3.	At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s).

4.	FFS whether for D2R we need message type field, any length and need for padding

5.	Specify message types and contents.  As starting point consider the following MAC message types.

	R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF))

	D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not)

	R2D MAC PDU (MSG2)

	D2R MAC PDU (MSG3 and data)

	R2D MAC PDU (R2D data)

	Other message types are FFS.  The message types may evolve based on functionality agreements.

    - Multiplexing

In RAN2#129 [2] meeting, there are agreements on paging identifier, e.g., device ID(s) or group ID:

1.	The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

2.	The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.   FFS if there is really a need for visibility in the MAC layer

It means that the device ID(s) or group ID will be processed by the upper layer of the device, such as the A-IoT NAS layer. Thus, the MAC PDU of the paging message includes at least the MAC control information on D2R scheduling and the SDU comprising device IDs.

Additionally, in RAN2#127bis [3] meeting, there is an agreement on energy bits shown as below:

Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1,MSG3 and Command Response message), if the device can.  1 bit indication can be captured in the TR.    We will capture the use case in the TR.   FFS whether it is reader controlled.

The energy status indication can be reported to the Reader via MAC layer. Also the message size can be reported to the Reader via MAC layer. These indicators, also referred to as AS layer control information, are highly relevant for data transmission. Upon receiving the D2R transmission scheduling info, the A-IoT device just multiplexes data stored in upper layer and D2R AS layer control information, if any, according to the TBS.

So we propose:

RAN2 to considerate the multiplexing of the upper layer data and the control information of MAC layer.

    - PDU format

Both the control information and the upper layer data can be transmitted in one MAC PDU, so the sub-MAC PDU can be introduced for the upper layer data and the MAC control element (CE) can be introduced for the control information. In addition, for control information such as the energy status indication, MAC CE of fixed length can be introduced to simplify the receiving operation of the Device. So we propose:

RAN2 to introduce sub-MAC PDU for higher layer data and MAC CE for the control information of MAC layer.

The MAC CE for the control information of MAC layer is of fixed length.

  - Assistance information on device energy status report

During the SI phase, the use of the assistance information on device energy status report was studied. Followings are captured in the TR:

-	The energy status report from the A-IoT device to the reader, which indicates that the A-IoT device, if the A-IoT device can determine this, does not have sufficient energy to perform the remaining/follow-up procedure.

-	The A-IoT device may report a 1-bit energy status indication to the reader in a D2R message. It needs to be further discussed if and how the reader controls this indication of such report.

-	The reader may take this indication into account for the remaining/follow-up procedure (up to the reader implementation), e.g., the reader may choose not to transmit the subsequent messages for some time, or the reader may choose to not do anything, etc.

In our understanding, energy status report is beneficial from the following perspectives:

-	improve communication availability: By indicating whether a device has sufficient energy to remain available for communication, energy status report helps maintain reliable communication links between the device and the reader. It ensures that devices can communicate successfully without being interrupted due to lack of energy.

-	Resource Optimization in A-IoT AS-layer procedures: energy status report allows for optimized use of communication resources by ensuring that the devices participating in CBRA and CFRA procedures are capable of completing necessary message exchanges, thereby avoiding resource wastage.

Support energy status report from A-IoT devices to reader.

- Conclusion

Based on the discussion in this document, we have the following observations and proposals:

Observation 1:	D2R message size reported by device can provide more efficient scheduling of D2R transmissions.

Observation 2:	D2R message size reported by device can potentially provide D2R transmission with low latency and low power consumption.

Observation 3:	D2R message size reported by device has larger spec impact and signaling overhead on extra design of report format.

Observation 4:	Ratio based message size report is a simple and efficient mechanism for A-IoT message scheduling.

Proposal 1:	Ratio based message size reporting can be supported for efficient scheduling in A-IoT.

Proposal 2:	Upon each segment transmission, the device stores a start position (starting position of the data segment for current transmission) and a next start position (starting position of the next data segment to be transmitted).

Proposal 3:	Upon scheduling for next segment, reader needs to provide a size information.

Proposal 4:	Upon receiving the D2R transmission scheduling information, device inform upper layer to construct the requested segment with the starting position and the size information.

Proposal 5:	RAN2 to confirm that the offset can be used to trigger the retransmission of the unsegmented upper layer data.

Proposal 6:	RAN2 to confirm that MSG2 is used to trigger the retransmission of MSG3.

Proposal 7:	The control information for repetition (e.g. specific number of repetitions or survival time of a command) is useful to be visible to the reader from CN.

Proposal 8:	RAN2 to considerate the multiplexing of the upper layer data and the control information of MAC layer.

Proposal 9:	RAN2 to introduce sub-MAC PDU for higher layer data and MAC CE for the control information of MAC layer.

Proposal 10:	The MAC CE for the control information of MAC layer is of fixed length.

Proposal 11:	Support energy status report from A-IoT devices to reader.

- Reference

- RP-250796, Rel-19 WID of Ambient IoT.

- Chairman Notes, RAN2 #129 meeting.

- Chairman Notes, RAN2 #127bis meeting.