3GPP TSG-RAN WG2 Meeting #129b	 R2-2501895

Wuhan, China, April 7-11, 2025

Agenda Item:	8.4.4

Source: 	NEC

Title:  	Discussion on LP-WUS in RRC_CONNECTED

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#129 meeting, the following agreements are made:

For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS whether short DRX cycle is used.

RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer.

RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer.

In this contribution, we further discuss remaining issue.

- 2 Discussion

- 2.1 Short DRX cycle for both opt 1-1 and opt 1-2

In last RAN2#129 meeting, we confirmed that DRX command MAC CE can be used with both opt 1-1 and opt 1-2 to stop active timer (including onD timer, IAT and new timer triggered by LP-WUS). One more issue is that the reception of DRX command MAC CE could trigger the use of short DRX, therefore, we need to further discuss whether/how to support short DRX. The reason why we introduce short DRX cycle is trying to make a balance between power saving and latency, if data is coming to the UE, it is possible that the UE will continue to receive data for a while, in this sense, short DRX cycle is used for to make the UE receive data a little bit earlier while sacrifice some power saving gain.

Option 1-1

For option 1-1, the usage of LP-WUS is similar to DCP mechanism, i.e., LP-WUS is used to trigger the follow-up drx-onDurationTimer which belongs to long DRX cycle. And when there is data burst coming, the UE still can consider to use short DRX cycle to balance power saving and latency. There seems no reason to restrict LP-WUS capable UE not to be configured with short DRX cycle.

Proposal-1: for option 1-1, LP-WUS can be configured with short DRX cycle simultaneously.

However, for option 1-1, there is still one more issue about whether LP-WUS is applied to short DRX cycle, i.e., whether the UE needs to monitor LP-WUS before every drx-onDurationTimer including both long and short DRX cycle. In our understanding, we think LP-WUS should not be applied to short DRX cycle. Because short DRX cycle is dynamic, e.g., whether to use short DRX depends on data traffic, which means the LP-WUS occasion/location is also dynamic, this increases the burden of RAN1 design. Also referring to DCP mechanism, DCP is only applied to long DRX cycle, since LP-WUS option 1-1 is just similar to DCP mechanism, maybe same principle should be applied.

Proposal-2: for option 1-1, LP-WUS is only applied to long DRX cycle, i.e., there is no LP-WUS monitoring occasion when short DRX cycle is used.

Option 1-2

In option 1-2, we already agreed that drx-onDurationTimer will not be started for PDCCH monitoring, instead, the UE monitor PDCCH during a new active timer triggered by LP-WUS or during other potential active timer (except for drx-onDurationTimer) is running. When it comes to short C-DRX cycle, thins are a little bit different, in legacy behavior, if drx-InactivityTimer is expired or DRX Command MAC CE is received, short C-DRX cycle will be used, and drx-onDurationTimer will be started more frequently for PDCCH monitoring based on short C-DRX cycle. However in option 1-2, drx-onDurationTimer will not be started for PDCCH monitoring and it is only for CSI reporting during the congifured window. In this sense it is questionable that whether we still need to consider short C-DRX mechanism in option 1-2. In our understanding, the only purpose of drx-onDurationTimer in option 1-2 is used to report CSI, in fact there is always different kind of active timer running where the UE could report CSI/SRS, in this sense, only drx-onDurationTimer belonging to long DRX cycle is enough, we do not need to configure short DRX cycle with LP-WUS option 1-2, it is more or less useless.

Proposal-3: for option 1-2, LP-WUS cannot be configured with short C-DRX simultaneously.

- 2.2 CSI reporting for option 1-2

In previous meeting, we agreed that For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time. This is referred to DCP mechanism, but it is not complete in our understanding.

Fig.1 UE report periodic CSI during the configured drx-onDurationTimer

Based on the agreement, it can be seen from Fig.1 that UE can report periodic CSI/L1-RSRP during the configured drx-onDurationTimer even though this timer will never be started. In legacy, why we need this control of CSI reporting is that when the drx-onDurationTimer is not running due to DCP is not detected, the UE still can report periodic CSI/L1-RSRP. In fact, the general principle should be as long as the UE is within the active time window but not started due to wake-up signalling, CSI reporting can be enabled. When it comes to LP-WUS, not only the legacy drx-onDurationTimer but also the new introduced active timer triggered by LP-WUS should be considered, if the new active timer is not started due to LP-WUS option 1-2, the network can configure (maybe reuse the same indication) whether the UE can report periodic CSI/L1-RSRP or not when this new timer is not running, this is exactly to refer DCP mechanism. The diagram can be seen from Fig.2.

Fig.2 network can configure whether the UE can report CSI or not during new active timer

Proposal-4: for option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the new timer introduced for LP-WUS opt 1-2 when this timer is not running.

- 2.3 LP-WUS monitoring occasion occurred in active time

Currently we agreed that for LP-WUS option 1-1, DCP baseline is taken as baseline, therefore, the following conditions highlighted below could trigger drx-onDurationTimer:

2>	else if the lpwus_PDCCHMonitoringTimer is not configured (i.e., LP-WUS Option 1-1):

3>	if LP-WUS monitoring is configured as specified in TS 38.213 [6], clause 10.X:

4>	if LP-WUS indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

4>	if all LP-WUS monitoring occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last LP-WUS occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4):

5>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

3>	else:

4>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

We can see from running CR that two conditions will trigger drx-onDurationTimer, first one is straightforward that the UE receives LP-WUS indication, while for the latter one, the condition means when the configured LP-WUS monitoring occasions occurred in active time considering some actions, the UE can not monitor LP-WUS so that is not aware of whether there is wake-up or not, in this regard, the UE could just blindly start drx-onDurationTimer for safety.

When it comes to LP-WUS option 1-2, the running CR is shown as below:

1>	if LP-WUS monitoring is configured and the lpwus_PDCCHMonitoringTimer for this DRX group is configured (i.e., LP-WUS Option 1-2):

2>	if LP-WUS indication is received from lower layer indicated to start lpwus_PDCCHMonitoringTimer, as specified in TS 38.213 [6]:

3> start lpwus_PDCCHMonitoringTimer from the beginning of the subframe indicated from lower layer.

For option 1-2, the LP-WUS cycle is configured independently from C-DRX cycle, and LP-WUS could trigger a new timer after its monitoring occasion with an offset. Here only one condition is specified, i.e., once the UE receives LP-WUS indication, the UE starts the new timer introduced for LP-WUS. In our understanding, same logic as option 1-1 should also be applied, when LP-WUS option 1-2 monitoring occasions occurred in active timer considering e.g., configured grant triggers retransmission timer so that the UE enters active time, the UE may not monitor LP-WUS, therefore the UE is not aware of whether there is wake-up indication or not, in this regard, the UE could just start this new timer as like DCP mechanism.

Fig.2 LP-WUS option 1-2 monitoring occasions occurred in active time

Proposal-5: for option 1-2, take option 1-1 as baseline, if all LP-WUS monitoring occasion(s) in time domain occurred in Active Time, the UE starts new timer from the beginning of the subframe indicated from lower layer.

- 2.4 others

It is recommended to give the new active timer triggered by LP-WUS a name, so that unnecessary terminology misunderstanding can be avoided and discussion can be facilitated. Currently the running CR give the named called lpwus_PDCCHMonitoringTimer. From our perspective, the definition of active time is broader than that of PDCCH timer (e.g., CSI reporting can be done in active time including this new timer). Therefore we prefer to name it as drx-onDurationTimer-LPWUS.

Proposal-6: name the new active timer triggered by LP-WUS as drx-onDurationTimerLP-WUS.

- 3 Conclusions

Proposal-1: for option 1-1, LP-WUS can be configured with short DRX cycle simultaneously.

Proposal-2: for option 1-1, LP-WUS is only applied to long DRX cycle, i.e., there is no LP-WUS monitoring occasion when short DRX cycle is used.

Proposal-3: for option 1-2, LP-WUS cannot be configured with short C-DRX simultaneously.

Proposal-4: for option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the new timer introduced for LP-WUS opt 1-2 when this timer is not running.

Proposal-5: for option 1-2, take option 1-1 as baseline, if all LP-WUS monitoring occasion(s) in time domain occurred in Active Time, the UE starts new timer from the beginning of the subframe indicated from lower layer.

Proposal-6: name the new active timer triggered by LP-WUS as drx-onDurationTimerLP-WUS.

- 4 References