3GPP TSG-RAN WG2 Meeting #129	bis                                                              R2-2501898

Wuhan, China, April. 7th – 11th, 2025

Source:                 CATT

Title:	                L1 event triggered measurement reporting

Agenda Item:	8.6.3

Document for:	Discussion and Decision

Introduction

In this paper, we further explore the open issues regarding L1 event-triggered measurement reporting and also the support of SP CSI-RS resource for LTM.

The organization of this contribution is as follows. In Section 2, we discuss the following aspects.

- MR MAC CE

- Signalling Configuration aspects

- Support of SP CSI-RS resource for LTM

Our proposals are summarized in Section 3.

Discussion

MR MAC CE

Indication in MR MAC CE to inform which beam(s) meets the event condition

It was agreed last meeting that there should be an explicit indication in MR MAC CE to inform which beam(s) meets the event condition.

- Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

For the detailed signalling design, two solutions are on the table,

a.	The UE reports in the measurement report MAC CE how many beams (among those included in the report) have fulfilled the event condition. The beams which have fulfilled the event conditions are included at the beginning of the MAC CE.

b.	The UE, for each reported beam. includes a flag which indicates whether this beam has fulfilled the event condition or not.

Both methods are viable, yet option b offers more flexibility.

Therefore the following is proposed:

Proposal 1: Introduce a per beam indication in MR MAC CE to inform whether the beam meets the event condition.

Indication in MR MAC CE to inform the leaving event condition is met

In the last meeting, there was a discussion regarding the necessity of an explicit indication to convey whether the reported beam has left the event condition. It was agreed that such an indication should be supported,

- Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

Before determine the way to indicate the beam met the leaving condition, another issue to be addressed is what information needed to be included in the MR for the beam met the leaving condition,

Option 1: Use the same format as that for the beam fulfilled the entering condition, i.e., beam id, and the L1 RSRP

Option 2: Beam id only,

In our view, it would be simpler to use a unified format for all the reported beams.

Therefore, it is proposed that,

Proposal 2a: In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event).

Regarding whether the indication should be per beam or per MAC CE, as for per MAC CE is one indication to indicate the measurement report is triggered due to the leaving condition is met. In our view, indicating it per MR MAC CE lacks practical significance. The purpose of this indication is to inform the network (NW) which beam has met the leaving condition. So one indication in the MAC CE to indicate the MR is reported due to the leaving condition is met shouldn’t be adopted.

Therefore, it is proposed that,

Proposal 2b: Introduce a per beam indication in MR MAC CE to inform the leaving event condition is met for the beam.

Truncated MR MAC CE

In the MAC running CR, MR_SENT_COUNTER(i.e., the number of L1 event triggered measurement report performed by UE) is incremented by 1 when a Truncated L1 measurement report MAC CE is sent,

2> else if the UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the Truncated L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the Truncated L1 measurement report MAC CE; 3> increment the MR_SENT_COUNTER by 1; 3> stop the periodical reporting timer, if running; 3> if reportAmount is configured in RRC, and the MR_SENT_COUNTER as defined for this ltm-CSI-ReportConfigId is less than reportAmount: 4> start the periodical reporting timer with the value of reportInterval for this ltm-CSI-ReportConfigId as defined within the corresponding LTM-CSI-reportConfig;

In our view, when counting the number of L1 event - triggered measurement reports, the intention is to count the original complete measurement report MAC CEs. Truncated L1 measurement report MAC CEs and subsequent MR MAC CEs of the same triggered event should not be double-counted. Therefore, it would be beneficial to clarify this.

Proposal 3: For event-triggered periodic measurement reporting, the reporting of the Truncated L1 measurement report MAC CE  are not counted as the number of L1 event-triggered measurement reports.

Signaling Configuration aspects

Whether to include candidate cell id into reporting configuration

In the last meeting, there was a discussion on whether to confirm the reuse of the R18 LTM approach for the association between measurement resource configuration and measurement reporting configuration. Some companies hold the view that such an association lacks flexibility. This is because within a single resource configuration, it is not permitted to provide different event configurations for reference signals (RSs) from different candidate cells.

Proposal5: Confirm on the previous agreement that "For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible". Comeback session: [ZTE]: Most of NW companies are ok with including candidate cell id into reporting configuration. However, we may need more time to check UE sides. [Samsung]: Including candidate cell id in reporting configuration would bring more complication to the UE since the UE needs to filter 2 steps (first based on LTM resource set and second based on candidate cell id among the ones that satisfied the first step). [LG]: Alternative is to enable NW to configure list of LTM resource set ids into reporting configuration, we can further think of until next meeting. Will be revisited next meeting.

In the last meeting, some companies proposed certain solutions. However, in our opinion, this is not an essential issue. The current design can still function properly. It can be up to the network implementation to configure RSs (e.g., RSs from the same candidate cell or for different candidate cells) which are intended to share the same event configuration in the same resource set.

Proposal 4: RAN2 confirms the previous agreement that "For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible". No additional enhancements are needed.

FFS whether it is possible to configure CSI-RS without SSB in the LTM-CSI-ResourceConfig

In the RRC Running CR, there is a FFS on whether it is possible to configure CSI-RS without SSB in the LTM-CSI-ResourceConfig and it is indicated that RAN2 waits for R1’s progress.

ltm-CSI-RS-ResourceSet This field indicates the resource set for LTM measuremenet based on CSI-RS. When the field is present, the UE shall ignore the field ltm-SSB-ResourceSet. Editor's NOTE: FFS whether it is possible to configure CSI-RS without SSB and wait for R1’s progress.

However, RAN1 has not held any discussions on this matter.

Observation 1: RAN1 has not engaged in discussions regarding the possibility of configuring CSI-RS in the absence of a valid ltm-SSB-ResourceSet in the LTM-CSI-ResourceConfig.

Considering the CSI-RS may has QCL with SSB, and it’s time domain configuration may be determined based on the timing of the SSB, whether it should be configured in the same LTM-CSI-ResourceConfig should be determined by RAN1, we suggest to send LS to RAN1 to ask them the question.

Proposal 5: RAN2 sends LS to RAN1 to confirm whether it is allow to configure CSI-RS in the LTM-CSI-ResourceConfig without a valid ltm-SSB-ResourceSet.

Coexistence of LTM event triggered reporting and mTRP operation at serving cells

In the scenario where there is the coexistence of LTM event-triggered reporting and mTRP operation at the serving cells, RAN1 has made the decision to defer the handling of this matter to RAN2.

Conclusion The following is up to RAN2: Coexistence of LTM event triggered reporting and mTRP operation at serving cells.

We understand it is a normal case that the serving cells are configured with mTRP when LTM event-triggered measurement is configured to UE at the same time.So it is proposed to support this case.

Proposal 6a:Support the coexistence of LTM event triggered reporting and mTRP operation at serving cells.

When the serving cells are configured with mTRP,there will be multiple indicated TCI states on a serving cell.

Since we have a agreement for current beam as follows,

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

So, this situation gives rise to a problem concerning which of the indicated TCI states of the serving cell should be utilized for the evaluation of LTM event-triggered measurements, especially when the current beam of the serving cell is part of the event evaluation (such as in events LTM2, LTM3, LTM5).

To tackle this issue, a seemingly straightforward solution is to allow the Network to determine and configure which specific indicated TCI state is to be employed for the event evaluation.

Consequently, the following proposal is put forward:

Proposal 6b: When LTM event-triggered reporting and mTRP operation coexist in the serving cells, the network configures which of the indicated TCI states of the serving cell is to be used for LTM event-triggered measurements evaluation.

Support of SP CSI-RS resource for LTM

RAN1 has reached an agreement to support semi-persistent CSI-RS, with the MAC CE being utilized to activate or deactivate the semi-persistent CSI-RS resource. RAN1 sent a LS [1] to RAN2, requesting the design of the corresponding MAC CE, as detailed below.

Agreement - Confirm the following working assumption made in RAN1#118bis Working Assumption(RAN1#118bis) In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2

Regarding the detailed design of the MAC CE, there are two viable options:

- Option 1: Extending the Existing MAC CE

This option involves extending the existing MAC CE, specifically the SP CSI-RS/CSI Resource Set Activation/Deactivation MAC CE, to be used for activating and deactivating the LTM SP CSI-RS. However, in the legacy SP CSI-RS activation/deactivation MAC CE, some fields like the Serving Cell ID and BWP ID are not relevant to the LTM SP CSI-RS. And considering until now, the SP CSI-RS can only be used to perform L1-RSRP, so the field IM and SP CSI-IM resource set ID are also not applicable for LTM. This would make the behavior of the UE quite complex, such as determining when to ignore certain fields.

- Option 2: Defining a New MAC CE

Under this alternative, a brand new MAC CE is defined solely for the activation and deactivation of the LTM SP CSI-RS. the following information can be included in the new MAC CE:

- An Activation/Deactivation (A/D) Indication that shows whether to activate or deactivate the indicated SP CSI-RS resource set.

- The Candidate Cell ID.

- The SP CSI-RS resource set ID.

- The TCI state for each CSI-RS resource within the SP NZP CSI-RS resource set indicated by SP CSI-RS resource set ID.

And considering the SP CSI-RS activation/deactivation should be informed to the candidate cell and the UE, which need interaction between the source cell and the candidate cell, the source cell and the UE, so it is more reasonable to include only one candidate cell in the MAC CE.

Clearly, when considering the UE behavior, Option 2 is simpler and more favorable.

Consequently, the following proposals are put forward:

Proposal 7a: A new MAC CE is defined for the activation and deactivation of the LTM SP CSI-RS resource of one candidate cell.

Proposal 7b: The new MAC CE include the following information:

- Indication of activating or deactivating of the indicated SP CSI-RS resource set

- Candidate cell ID

- SP CSI-RS resource set ID

- TCI state for each CSI-RS resource within the SP NZP CSI-RS resource set indicated by SP CSI-RS resource set ID

A reply LS to RAN2 is obviously needed.

Proposal 7c: RAN2 replies LS to RAN1 to inform the agreement above.

Conclusion

According to the previous analysis in section 2, our observations and proposals are summarized as follows:

Observation 1: RAN1 has not engaged in discussions regarding the possibility of configuring CSI-RS in the absence of a valid ltm-SSB-ResourceSet in the LTM-CSI-ResourceConfig.

Measurement report MAC CE

Proposal 1: Introduce a per beam indication in MR MAC CE to inform whether the beam meets the event condition.

Proposal 2a: In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event).

Proposal 2b: Introduce a per beam indication in MR MAC CE to inform the leaving event condition is met for the beam.

Proposal 3: For event-triggered periodic measurement reporting, the reporting of the Truncated L1 measurement report MAC CE  are not counted as the number of L1 event-triggered measurement reports.

Signaling Configuration aspects

Proposal 4: RAN2 confirms the previous agreement that "For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible". No additional enhancements are needed.

Proposal 5: RAN2 sends LS to RAN1 to confirm whether it is allow to configure CSI-RS in the LTM-CSI-ResourceConfig without a valid ltm-SSB-ResourceSet.

Proposal 6a:Support the coexistence of LTM event triggered reporting and mTRP operation at serving cells.

Proposal 6b: When LTM event-triggered reporting and mTRP operation coexist in the serving cells, the network configures which of the indicated TCI states of the serving cell is to be used for LTM event-triggered measurements evaluation.

Support of CSI-RS measurements for LTM

Proposal 7a: A new MAC CE is defined for the activation and deactivation of the LTM SP CSI-RS resource of one candidate cell.

Proposal 7b: The new MAC CE include the following information:

- Indication of activating or deactivating of the indicated SP CSI-RS resource set

- Candidate cell ID

- SP CSI-RS resource set ID

- TCI state for each CSI-RS resource within the SP NZP CSI-RS resource set indicated by SP CSI-RS resource set ID

Proposal 7c: RAN2 replies LS to RAN1 to inform the agreement above.

Reference

- LS on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells, R1- 2501500