3GPP TSG-RAN WG2 Meeting #129bis           	R2-2501899

Wuhan, China, Apr. 7th – 11th, 2025

Source:	CATT

Title:	Discussion on Conditional Intra-CU LTM

Agenda Item:    8.6.4

Document for:  Discussion and Decision

Introduction

This paper further discusses the remaining issues on the conditional intra-CU LTM [1]. In detail, the following issues are covered by our discussion,

- TAT handling for early TA

- Evaluation of  L1 based CLTM

- Beam selection for L3-based RACH-less CLTM

- TCI state determination upon RACH-less CLTM execution

- UE handling for mTRP 2TA on target case

The detailed discussion is provided in Section 2, and followed by Section 3 that summarizes the contribution of this paper.

Discussion

TAT handling for early TA

On the determination of TAT for early TA

As agreed by RAN2, the early TA is maintained by a new TAT associated with the corresponding candidate cell. Then the current RRC running CR provides the TAT configuration for early TA within the IE EarlyUL-SyncConfig-r18 in the field of ltm-EarlyUL-SyncConfig-r18.

EarlyUL-SyncConfig-r18 ::= SEQUENCE { … ..., [[ ltm-tdd-UL-DL-ConfigurationCommon-r18 TDD-UL-DL-ConfigCommon OPTIONAL, -- Cond TDD ltm-restrictedSetConfig-r18 ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB} OPTIONAL -- Need R ]], [[ ltm-TimeAlignmentTimer-r19 TimeAlignmentTimer OPTIONAL -- Need R ]] }

That means the TAT value for monitoring the valid of early TA is generated by the target cell.

However, as mentioned above, the TAT is used to monitor the valid of the value for early TA associated with a candidate cell. Thus the determination of the TAT value may consider the UE mobility status such as the UE speed. Note that this information is well known by the source cell but not the candidate cell. So we propose that the TAT value used for monitoring the early TA is determined by the source cell.

Proposal 1: The value of ltm-TimeAlignmentTimer-r19 is determined by the source cell.

TAT handling upon conditional LTM execution

Regarding how to handle the CLTM TAT for target cell, if not stopped upon CLTM cell switch execution to the target cell, it was discussed last meeting but not concluded yet

CLTM TAT for target cell is not stopped upon CLTM cell switch execution to the target cell. Rapporteur suggests to discuss how/whether to start the timeAlignmentTimer associated with PTAG upon CLTM cell switch execution to the target cell during CB session: - Option 1 (revert the offline agreement): start the timeAlignmentTimer associated with PTAG as in Rel-18; - Option 2 (keep the offline agreement): discuss the options below in the MAC running CR o Option 2.1: clarify the running TAT to act as the timeAlignmentTimer associated with PTAG by a note in the MAC running CR; o Option 2.2: define that UE can perform UL tx when the TAT for the target cell is running. Besides, we need to add a note that, if SCell and SpCell are in the different TAG, SCell may not work when this TAT for target cell is running. o Option 2.3: the UE stops the CLTM timer and starts the PTAG using the remaining time of the CLTM timer. Will be revisited next meeting.

Besides, it still remains open on how to handle the TAT associated with candidate cells upon conditional LTM execution.

CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell. FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

In Rel-18 LTM, the value of early TA is delivered to UE via LTM cell switch command. Upon receiving the TA in the LTM cell switch command, the UE applies the TA value and starts the TAT associated with the corresponding PTAG. Therefore, the maintenance of the TA can be aligned among the NW and UE.

When considering conditional LTM, UE will start the new timer upon receiving the early TA MAC CE. Therefore, upon the execution of conditional LTM, the new timer has been running for some time duration from the UE side, but the target cell is not aware of such time duration. Thus how the NW and UE align the maintenance of the TA should be further considered. One way is to stop the new timer for early TA, and then start the TAT of the PTAG configured by the target RRCReconfiguration associated with this TA value. By this way, the Rel-18 handling on the maintenance of TA in the current serving cell after LTM cell switch can be reused.

Besides the early TA associated with the target cell, the UE may maintain early TA for other candidate cells. If the UE keeps the ltm-TimeAlignmentTimer running for the other early TA, the new serving cell has no knowledge about when these TA value will be assumed to be invalid by the UE. Besides, the potential candidate cell for the current serving cell may not be the potential candidate cell for the target serving cell (i.e. new serving cell), so it may be not useful for the UE to maintain the early TA value after handover to this target cell. One simple way is to stop all running ltm-TimeAlignmentTimer for early TA upon cell switch.

Therefore, it is proposed that,

Proposal 2a: CLTM TAT for target cell is stopped upon CLTM cell switch execution to the target cell. Then UE starts the timeAlignmentTimer associated with PTAG as in Rel-18.

Proposal 2b: Upon the handover execution (e.g., conditional LTM, NW triggered LTM execution, legacy NW triggered handover, CHO), UE stops the ltm-TimeAlignmentTimer of other candidate cells.

Evaluation of  L1 based conditional LTM

In last RAN2 meeting, it was agreed to reuse the existing L1 events for conditional LTM.

Re-use existing L1 events for CLTM.

Regarding how to provide the execution condition for conditional LTM, it still captures the following editor note in the current RRC running CR,

-- Editor’s Note: How the execution conditions for CLTM are configured is pending the progresses on the event-triggered L1 measurements objective.

As per the current signaling framework for event triggered L1 measurement reporting, the execution conditions are configured by LTM-CSI-ReportConfig-r18, which is associated with the corresponding L1 measurement resource by ltm-ResourcesForChannelMeasurement-r18 defined by the IE LTM-CSI-ResourceConfigId-r18. The current signalling structure allows the NW to include the RS(s) from one or multiple candidate cells within an LTM-CSI-ResourceConfig-r18.

However, in the current RRC running CR for inter-CU LTM and conditional LTM, the L1 execution condition of one candidate cell is defined as LTM-CSI-ReportConfigId. That means the L1 execution condition of a candidate cell may be associated to the measurement RSs of other candidate cells.

ExecutionConditions-r19 ::= SEQUENCE { ltm-CandidateId-r19 LTM-CandidateId-r18, executionCondition-r19 CHOICE { l1-Conditions-r19 LTM-CSI-ReportConfigId-r18, l3-Conditions-r19 SEQUENCE (SIZE (1..2)) OF MeasId } OPTIONAL, -- Need R ... }

For example, for candidate cell 1, the configured l1-Conditions is associated with LTM-CSI-ResourceConfig-r18 which is configured with the RS from candidate cell 1, candidate cell 2, candidate cell 3. According to the LTM-CSI-ReportConfig, UE will perform event evaluation to all the RSs from candidate cell 1, 2 and 3, however, it is not reasonable to perform the event evaluation to the RS of the other candidate cells (cell 2, cell 3) when the L1 execution condition is configured for candidate cell 1.

Observation 1: In the CLTM evaluation, if the L1 execution condition of a candidate cell associated with an LTM-CSI-ResourceConfig-r18 which includes RSs from other candidate cells, then the RSs of those other candidate cells are evaluated. This seems unreasonable.

Note that in the current MAC running CR for conditional LTM,

> for each entry within the CLTM-ExecutionConditions: 2> if the eventLTM3 or eventLTM5 is configured in the corresponding ltm-CSI-ReportConfigId for l1-Conditions: Editor’s NOTE: The term for eventLTM3 or eventLTM5 is to be aligned with RRC later. 3> consider all beams of LTM candidate cell indicated by the ltm-CandidateId within this CLTM-ExecutionConditions, and the current beam of serving cell, i.e. the beam corresponds to the RS indicated by TCI State or the RS QCLed with the RS indicated by TCI State in the serving cell [as defined in clause 5.1.5 in TS 38.214], with the same RS type as the beam of candidate cell, to be applicable;

It says all beams of LTM candidate cell, but it doesn’t mention the relationship of the evaluated RS and the RSs configured within the execution condition.

To solve above ambiguity, we have the following candidate options for clarification.

- Option 1: Specify that, for the LTM-CSI-ReportConfigId configured as execution condition, the ltm-ResourcesForChannelMeasurement-r18 could only include the RS from the associated candidate cell.

- Option 2: Specify that, when performing conditional LTM evaluation, the UE only evaluates the RS within the ltm-ResourcesForChannelMeasurement-r18 associated with the l1 execution condition which is from the corresponding evaluated candidate cell.

- Option 3: UE evaluate all the RSs from the evaluated candidate cell by default.

Option3-1: LTM-CSI-ReportConfigId-r18 is configured as the execution condition, UE ignore the ltm-ResourcesForChannelMeasurement-r18 configured in the associated LTM-CSI-ReportConfig.

Option3-2: redesign the event configuration signalling structure to avoid the association between L1 execution condition and measurement resource configuration.. For example, a common event list is defined for both event triggered L1 measurement reporting and the conditional LTM. Then only the event index is configured as the L1 execution condition for each candidate configuration. By this way, the event configuration is decoupled from the L1 measurement resource configuration. UE measures all L1 RSs associated with a candidate cell for evaluation by default.

We are open to above options, so we suggested that,

Proposal 3a: In the CLTM evaluation, only the RSs of the same candidate cell can be evaluated if L1 execution condition is configured for the candidate cell.

Proposal 3b: If P3a is agreed, RAN2 down-selects between the following options,

Option 1: For the LTM-CSI-ReportConfigId configured as L1 execution condition of a candidate cell, NW ensures that the associated measurement resource config could only include the RSs of the corresponding candidate cell;

Option 2: If the L1 execution condition of a candidate cell associated with an LTM-CSI-ResourceConfig-r18 which includes RSs from other candidate cells, the UE only evaluates the RSs of the same candidate cell;

Option 3: UE evaluate all the RSs of candidate cell, not the RSs associated with the L1 execution condition:

Option 3-1: LTM-CSI-ReportConfigId-r18 is configured as the execution condition, UE ignore the ltm-ResourcesForChannelMeasurement-r18 configured in the associated LTM-CSI-ReportConfig

Option 3-2: redesign the event configuration signaling structure to avoid the association between L1 execution condition and measurement resource configuration.

Beam selection for L3-based RACH-less CLTM

For L1-based RACH-less conditional LTM, the target beam is the beam that satisfies the execution condition. If multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM. For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

In the current running CR, the following editor note should be confirmed.

5.3.5.18.6 LTM cell switch execution Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or upon the fulfilment of LTM cell switch conditions, the UE shall: 1> if this procedure is triggered due to fulfilment of CLTM cell switch conditions: 2> if more than one LTM candidate configuration has triggered this procedure: 3> select one of the LTM candidate configurations as the selected cell for the LTM cell switch execution; Editor’s Note: FFS how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution.

This FFS is only related to L3 based CLTM as in L1 based CLTM, it is clear that the candidate cell which the selected beam belongs to is the target cell after the beam is selected.

For L3 based CLTM, it is straightforward to follow the same principle of CHO. That is to say, if more than one candidate cells satisfy the L3 execution condition, it’s up to UE implementation to select one of the LTM candidate configurations as the target cell.

Proposal 4: For L3 based CLTM, it’s up to UE implementation to select one of the LTM candidate configurations as the target cell if more than one candidate cells satisfy the L3 execution condition.

Once the UE determines the target cell for L3-based conditional LTM, if the UE has a valid TA associated with the target cell, UE will perform RACH-less cell switch to the target cell. Then how to determine the target beam for L3-based RACH-less conditional LTM should be further discussed. From our view, the simplest way is just up to UE implementation to select a suitable beam to access to the target cell.

Proposal 5: For L3-based RACH-less conditional LTM, it’s up to UE implementation to select the target beam.

TCI state determination upon RACH-less CLTM execution

In R18 RACH-less LTM, the TCI state used in the target cell is indicated in the LTM cell switch command MAC CE. Upon receiving the MAC CE, the UE MAC layer will indicate the TCI state information to the physical layer so that the physical layer can apply the TCI state for transmitting/receiving in the target cell.

5.18.35 LTM Cell Switch Command The network may instruct the UE to perform LTM cell switch procedure by sending the LTM Cell Switch Command MAC CE described in clause 6.1.3.75. The MAC entity shall: ….. 3> indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

For RACH-less conditional LTM, only CG is supported for the first PUSCH transmission in the target cell. When UE determines the target beam no matter based on L1 execution condition or L3 execution condition, UE will use the CG resource associated with the target beam (target RS) to transmit the first PUSCH in the target cell. It seems UE does not need to determine a TCI state upon RACH-less conditional LTM execution. However, whether the physical layer needs the TCI state information is under RAN1 scope. So we suggest LS to RAN1 for further clarification.

Proposal 6: LS to RAN1 whether UE needs to determine the target TCI state upon the CG based RACH-less conditional LTM execution.

UE handling for mTRP 2TA on target case

In order to support the conditional LTM with two-TA, the TAG information is introduced in the LTM Candidate Timing Advance Command MAC CE as in the MAC running CR. This information can be used together with the tag-ptr in the TCI state configuration.

Thus, if UE determines the target beam  for RACH-less LTM, UE should firstly verify the early TA can be applied to the target beam, i.e., the target beam should be associated with the same TAG as indicated by the TAG information in the LTM Candidate Timing Advance Command MAC CE.

Proposal 7: For RACH-less conditional LTM, the early TA can be applied to the target beam only if the target beam is associated with the same TAG as indicated by the TAG information in the LTM Candidate Timing Advance Command MAC CE. The related UE behavior is to be captured in the MAC spec.

Conclusion

Based on the analysis in Section 2, our observations and proposals are summarized as follows:

Observation 1: In the CLTM evaluation, if the L1 execution condition of a candidate cell associated with an LTM-CSI-ResourceConfig-r18 which includes RSs from other candidate cells, then the RSs of those other candidate cells are evaluated. This seems unreasonable.

TAT handling for early TA

Proposal 1: The value of ltm-TimeAlignmentTimer-r19 is determined by the source cell.

Proposal 2a: CLTM TAT for target cell is stopped upon CLTM cell switch execution to the target cell. Then UE starts the timeAlignmentTimer associated with PTAG as in Rel-18.

Proposal 2b: Upon the handover execution (e.g., conditional LTM, NW triggered LTM execution, legacy NW triggered handover, CHO), UE stops the ltm-TimeAlignmentTimer of other candidate cells.

Evaluation of  L1 based CLTM

Proposal 3a: In the CLTM evaluation, only the RSs of the same candidate cell can be evaluated if L1 execution condition is configured for the candidate cell.

Proposal 3b: If P3a is agreed, RAN2 down-selects between the following options,

Option 1: For the LTM-CSI-ReportConfigId configured as L1 execution condition of a candidate cell, NW ensures that the associated measurement resource config could only include the RSs of the corresponding candidate cell;

Option 2: If the L1 execution condition of a candidate cell associated with an LTM-CSI-ResourceConfig-r18 which includes RSs from other candidate cells, the UE only evaluates the RSs of the same candidate cell;

Option 3: UE evaluate all the RSs of candidate cell, not the RSs associated with the L1 execution condition:

Option 3-1: LTM-CSI-ReportConfigId-r18 is configured as the execution condition, UE ignore the ltm-ResourcesForChannelMeasurement-r18 configured in the associated LTM-CSI-ReportConfig

Option 3-2: redesign the event configuration signaling structure to avoid the association between L1 execution condition and measurement resource configuration.

Beam selection for L3-based RACH-less CLTM

Proposal 4: For L3 based CLTM, it’s up to UE implementation to select one of the LTM candidate configurations as the target cell if more than one candidate cells satisfy the L3 execution condition.

Proposal 5: For L3-based RACH-less conditional LTM, it’s up to UE implementation to select the target beam.

TCI state determination upon RACH-less CLTM execution

Proposal 6: LS to RAN1 whether UE needs to determine the target TCI state upon the CG based RACH-less conditional LTM execution.

UE handling for mTRP 2TA on target case

Proposal 7: For RACH-less conditional LTM, the early TA can be applied to the target beam only if the target beam is associated with the same TAG as indicated by the TAG information in the LTM Candidate Timing Advance Command MAC CE. The related UE behavior is to be captured in the MAC spec.

Reference

[1] Draft_RAN2_129_Meeting_Report_v3