3GPP TSG RAN WG2 #129bis                                                                           R2-2501910

Wuhan, China, Apr. 7th – 11th, 2025

Source:	CATT

Title:	Discussion on the number of SDT UEs (LS S5-250827)

Agenda Item:	8.19

Document for:	Discussion and Decision

Introduction

Regarding the number of UEs in RRC_INACTIVE state with data transmission, “SA5 would like to ask RAN2 to evaluate whether there is a possibility to specify the number of UEs in RRC_INACTIVE state with data transmission” in [1]. And after some RAN2 discussion, RAN2 replied in [2] that “RAN2 confirms the network knows the number of UEs in RRC_INACTIVE state in a cell with an ongoing SDT session as the connection requests from such UEs are distinguished either based on dedicated RACH resources or based on a dedicated RRC resume cause.”

In this meeting, the latest SA5 reply LS has been received as below [3]:

SA5 thanks RAN2 for providing further information on SDT procedure and confirms the network knows the number of UEs in RRC_INACTIVE state in a cell with an ongoing SDT session. SA5 would like RAN2 to specify metrics that can measure the number of RRC_INACTIVE UEs in a cell with ongoing SDT. In the current 5G network, the number of UEs using SDT can be huge, especially in IoT scenarios, where the total amount of SDT data transmission can be very high, which may lead to potential network congestion or contribute to network congestion to some extent. There are benefits for operators to monitor the dynamics of the network state caused by these UEs with small data transmitting, to help analyze the network conditions. For instance, the percentage of the network workload contributed by SDT transmission, and the SDT traffic volume of UEs in a cell given that handover is not considered in most IoT scenarios. Besides, measurements can be implemented based on the statistics of a specific area consisting of some cells. Therefore, “occasional unsuccessfully completed SDT procedure” mentioned in R2-2409280 will not affect the use cases in SA5. In addition, SA5 notes that the specific definition in the tables of TS 38.314 clause 4.2.1.3 does not identify and restrict that the UE is in the RRC_CONNECTED state, but instead can be used generically for both the connected state and inactive state. The wording ‘in RRC_CONNECTED’ in the title of clause 4.2.1.3 may cause confusion.

Since SA5 asks RAN2 to provide further feedbacks and specify the needed metrics, in this contribution we further discuss how to respond to SA5.

Discussion

R19 Specification work for new feature

For the number of UEs in RRC_INACTIVE state with data transmission, SA5 confirms the use case is not affected when the UE may stop ongoing SDT due to some reason but network doesn’t know, since “measurements can be implemented based on the statistics of a specific area consisting of some cells”.

From RAN2 perspective, it is unnecessary to further distinguish whether an ongoing SDT procedure is successfully/unsuccessfully completed. As “SA5 would like RAN2 to specify metrics that can measure the number of RRC_INACTIVE UEs in a cell with ongoing SDT”, the corresponding measurements can be introduced.

Taking existing measurements for “Number of active UEs in RRC_CONNECTED” as an example, there are in total 8 metrics specified as below:

- Mean number of Active UEs in the DL per DRB per cell

- Max number of Active UEs in the DL per DRB per cell

- Mean number of Active UEs in the UL per DRB per cell

- Max number of Active UEs in the UL per DRB per cell

- Mean number of Active UEs per cell

- Max number of Active UEs per cell

- Mean number of Active UEs per DRB per cell

- Max number of Active UEs per DRB per cell

Since SA5 only mentioned “metrics that can measure the number of RRC_INACTIVE UEs in a cell with ongoing SDT” in general, but no details have been indicated in the reply LS, RAN2 needs to discuss the detailed metrics. In our view, the main idea is about the “the number of RRC_INACTIVE UEs in a cell with ongoing SDT”, so the metrics “Mean/max number of RRC_INACTIVE UEs with ongoing SDT per cell” should be included. And whether per DRB or per UL/DL DRB metrics are needed can be discussed in RAN2.

Besides the usage of DRB for SDT, “the usage of signalling radio bearer SRB2 for MO-SDT (over RA-SDT or CG-SDT) or MT-SDT (over RA or CG-SDT)” was also introduced in R17, which is mainly used for periodic reporting of UE location. Since the SA5 intention is to “monitor the dynamics of the network state caused by these UEs with small data transmitting”, it seems that per SRB or per UL/DL SRB metrics can be also be discussed.

Proposal 1: According to SA5 reply LS, the measurement metrics “Mean/max number of RRC_INACTIVE UEs with ongoing SDT per cell” are introduced in TS 38.314.

Proposal 2: RAN2 to discuss whether the following measurement metrics are also introduced:

- Mean/max number of RRC_INACTIVE UEs with ongoing SDT in the DL per DRB per cell

- Mean/max number of RRC_INACTIVE UEs with ongoing SDT in the UL per DRB per cell

- Mean/max number of RRC_INACTIVE UEs with ongoing SDT per DRB per cell

- Mean/max number of RRC_INACTIVE UEs with ongoing SDT in the DL per SRB per cell

- Mean/max number of RRC_INACTIVE UEs with ongoing SDT in the UL per SRB per cell

- Mean/max number of RRC_INACTIVE UEs with ongoing SDT per SRB per cell

Legacy specification change for clarification

As pointed out in the reply LS [3], “SA5 notes that the specific definition in the tables of TS 38.314 clause 4.2.1.3 does not identify and restrict that the UE is in the RRC_CONNECTED state, but instead can be used generically for both the connected state and inactive state. The wording ‘in RRC_CONNECTED’ in the title of clause 4.2.1.3 may cause confusion.”

In our understanding, the confusion indicated by SA5 is due to the usage of wording “active UE” in clause 4.2.1.3 Number of active UEs in RRC_CONNECTED and “inactive UE” in clause 4.2.1.4 Number of stored inactive UE contexts. Actually, in clause 4.2.1.3 active UEs refer to UEs in RRC_CONNECTED for which “applications are transmitting and receiving data”. And in clause 4.2.1.4, inactive UEs refer to UEs in RRC_INACTIVE. Therefore “active UEs” and “inactive UEs” are in different RRC states, which means “active UEs” and “inactive UEs” are defined in different levels. The corresponding clarification can be introduced in TS 38.314 to fix the confusion SA5 mentioned, and the TP is shown as below.

- 4.2.1.4 Number of stored inactive UE contexts - 4.2.1.4.1 General The objective of the measurement is to measure number of stored inactive UE (i.e., UEs in RRC_INACTIVE.) contexts for OAM performance observability. It is intended to be part of indication about the memory consumption in a RAN node. The measurement is obtained by sampling at a pre-defined interval, the number of inactive UE contexts for each gNB and then taking the arithmetic mean or maximum value over pre-defined time duration. <Unnecessary part omitted>

Proposal 3: As “active UEs” and “inactive UEs” are defined in different levels, R16-R18 spec change in TS 38.314 is needed to clarify in clause 4.2.1.4 inactive UEs refer to UEs in RRC_INACTIVE.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: According to SA5 reply LS, the measurement metrics “Mean/max number of RRC_INACTIVE UEs with ongoing SDT per cell” are introduced in TS 38.314.

Proposal 2: RAN2 to discuss whether the following measurement metrics are also introduced:

- Mean/max number of RRC_INACTIVE UEs with ongoing SDT in the DL per DRB per cell

- Mean/max number of RRC_INACTIVE UEs with ongoing SDT in the UL per DRB per cell

- Mean/max number of RRC_INACTIVE UEs with ongoing SDT per DRB per cell

- Mean/max number of RRC_INACTIVE UEs with ongoing SDT in the DL per SRB per cell

- Mean/max number of RRC_INACTIVE UEs with ongoing SDT in the UL per SRB per cell

- Mean/max number of RRC_INACTIVE UEs with ongoing SDT per SRB per cell

Proposal 3: As “active UEs” and “inactive UEs” are defined in different levels, R16-R18 spec change in TS 38.314 is needed to clarify in clause 4.2.1.4 inactive UEs refer to UEs in RRC_INACTIVE.

Reference

- S5-245138, LS on Number of UEs in RRC_INACTIVE state with data transmission, SA5

- R2-2409280, Reply to SA5 LS on Number of UEs in RRC_INACTIVE state with data transmission, RAN2

- S5-250827, Reply to RAN2 LS on Number of UEs in RRC_INACTIVE state with data transmission, SA5