3GPP TSG-RAN WG2 Meeting #129bis                                                             R2-2501912

Wuhan, China, Apr. 7th – 11th, 2025

Source:	CATT

Title:	Views on R19 SONMDT UE capabilities

Agenda Item:	8.10.1

Document for:	Discussion and Decision

Introduction

For R19 SONMDT [1], the following features have been discussed:

- MRO enhancements for LTM, CHO with candidate SCGs, and subsequent CPAC

- SON/MDT enhancements for Intra-NTN mobility

- SON/MDT enhancements for Network Slicing

- R18 leftovers for RACH optimization for SDT, MHI Enhancement for SCG Deactivation/Activation, MRO for MR-DC SCG failure

In this contribution, the initial views on UE capabilities of all features are provided based on current agreements.

Discussion

MRO enhancements for Rel-18 mobility features

LTM

According to the agreements made in RAN2#125bis meeting, RAN2 considers SHR, RA report and RLF for MCG LTM SON, and RAN2 would start work on MCG LTM.

RAN2 considers SHR, RA report and RLF for MCG LTM SON. RAN2 will start work on MCG LTM.

In later meetings, some agreements on RLF report and SHR for LTM have been achieved.

- RAN2#126 If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure. - RAN2#127 RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled. - RAN2#127bis Include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation.

Observation 1: some enhancements on RLF report and SHR for MCG LTM have been agreed.

From UE capability perspective, it should be optional for UE to log LTM related information in RLF report and SHR. Regarding whether a UE capability signalling is needed, we have noticed that rlfReportDAPS-r17 is introduced considering it may help network to retrieve necessary failure information for DAPS. Similarly, a UE capability signalling can be defined for RLF report for MCG LTM. And for SHR for LTM, no separate UE capability signalling is needed.

Proposal 1: an optional UE capability with signalling is defined for RLF report for MCG LTM.

Proposal 2: an optional UE capability without signalling is defined for SHR for MCG LTM.

CHO with candidate SCGs

At RAN2#128 meeting, it was agreed:

Agreements - RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements. - Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

Observation 2: some enhancements on RLF report, SHR and SCGFailureInformation for CHO with candidate SCG have been agreed

Regarding UE capability, similar to LTM, also considering rlfReportCHO-r17 has been defined, we have the following proposals:

Proposal 3: an optional UE capability with signalling is defined for RLF report for CHO with candidate SCG.

Proposal 4: an optional UE capability without signalling is defined for SHR/SCGFailureInformation for CHO with candidate SCG.

Subsequent CPAC

Since no concrete RAN2 progress has been achieved until RAN2#129, no UE capability issue needs to be discussed.

SON/MDT for Slicing

Since no concrete RAN2 progress has been achieved until RAN2#129, no UE capability issue needs to be discussed.

SON/MDT for NTN

At RAN2#129 meeting, it was agreed:

Existing IEs for CHO are used to report the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”.

In R17 RLF report, CHO Event A3 and A5 related information was introduced. And according to RAN2 analysis, the existing IEs can be reused for time/location based CHO. From UE capability perspective, when rlfReportCHO-r17 is indicated, UE can support the delivery of R16 CHO related information in RLF report. For R17 time/location based CHO, an optional UE capability without signalling can be introduced for the additional CHO information of time/location based execution conditions.

Proposal 5: an optional UE capability without signalling is defined for RLF report for time/location based CHO.

Leftovers from Rel-18

RACH optimization for SDT

The following agreements have been made for SDT:

RAN2#127bis For failed SDT case, UE includes the DL RSRP and UL data volume at the time of SDT evaluation in SON report. For successful SDT procedure, the UE does not log. RAN2 understands for SON/MDT R19 the SDT enhancements only relate to the RA-SDT procedure. This does not rule out the case when the UE falls back from RA-SDT. RAN2#129 Include cell re-selection as a failure cause for failed RA-SDT in RA-Report Introduce a new field in RA-Report to indicate the elapsed time since the execution of RA-SDT. Value in seconds. The maximum value is 172800 seconds.

Based on the agreements above, the RACH optimization for SDT is included in RA-report. Since a UE capability rach-Report-r16 has been defined for RA-report, a separate UE capability signaling for SDT related information is not necessary.

Proposal 6: an optional UE capability without signalling is defined for RA-report for SDT.

MHI Enhancement for SCG Deactivation/Activation

In RAN2#129meeting, it was agreed that:

We will introduce a UE based solution for SCG act/deact MHI, as an optional UE feature. UE reports the time the UE has spent in PSCell with SCG activated, TBD details, e.g. percentage of time or absolute time, etc.

Since pscell-MHI-Report-r17 has been defined to indicate whether the UE supports the storage of PSCell mobility history information and the reporting in UEInformationResponse message, there is no need to define another UE capability signaling for MHI for SCG deactivation.

Proposal 7: an optional UE capability without signalling is defined for MHI Enhancement for SCG Deactivation/Activation.

MRO for MR-DC SCG failure

The following agreements have been made for MRO for MR-DC SCG failure:

- RAN2#127 To support MRO for SCG failure in EN-DC, enhance SCGFailureInformationNR message to include previousPSCellId, failedPSCellId, timeSCGFailure. - RAN2#127bis Add reporting of the following parameters for SCG failure report in EN-DC scenario: - For failedPSCellId and previousPSCellId: frequency and the PCI of the PSCell; - For timeSCGFailure: value range 0-1023; - For failureType: Reuse the legacy field. - perRA-InfoList

The corresponding new UE capability is needed to add additional PSCell information in SCGFailureInformationNR message, but considering there is no RAN specific configuration or retrieval procedure in LTE specification, it’s unnecessary to define UE capability signalling.

Proposal 8: an optional UE capability without signalling is defined for MRO for EN-DC SCG failure in LTE specification.

The corresponding draft CRs to capture the proposals above are provided in [2][3][4].

Conclusion

According to the analysis in section 2, we have the observations and proposals as below:

LTM

Observation 1: some enhancements on RLF report and SHR for MCG LTM have been agreed.

Proposal 1: an optional UE capability with signalling is defined for RLF report for MCG LTM.

Proposal 2: an optional UE capability without signalling is defined for SHR for MCG LTM.

CHO with candidate SCGs

Observation 2: some enhancements on RLF report, SHR and SCGFailureInformation for CHO with candidate SCG have been agreed

Proposal 3: an optional UE capability with signalling is defined for RLF report for CHO with candidate SCG.

Proposal 4: an optional UE capability without signalling is defined for SHR/SCGFailureInformation for CHO with candidate SCG.

SON/MDT for NTN

Proposal 5: an optional UE capability without signalling is defined for RLF report for time/location based CHO.

RACH optimization for SDT

Proposal 6: an optional UE capability without signalling is defined for RA-report for SDT.

MHI Enhancement for SCG Deactivation/Activation

Proposal 7: an optional UE capability without signalling is defined for MHI Enhancement for SCG Deactivation/Activation.

MRO for MR-DC SCG failure

Proposal 8: an optional UE capability without signalling is defined for MRO for EN-DC SCG failure in LTE specification.

Reference

- RP-234038 New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4 CMCC

- R2-2501913, 38.331 running CR for Introduction of SONMDT UE Capabilities, CATT

- R2-2501914, 38.306 running CR for Introduction of SONMDT UE Capabilities, CATT

- R2-2501915, 36.331 running CR for Introduction of SONMDT UE Capabilities, CATT