3GPP TSG-RAN WG2 Meeting #129bis                                                             R2-2501918

Wuhan, China, Apr. 7th – 11th, 2025

Source:	CATT

Title:	Consideration on SONMDT enhancements for intra-NTN mobility

Agenda Item:	8.10.4

Document for:	Discussion and Decision

Introduction

For R19 SONMDT, RAN3 has sent an LS on NTN in previous meeting [1]:

- Regarding SON/MDT for NTN: On MRO for NTN, RAN3 agreed that UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not. On MDT for NTN, RAN3 assumes that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP. Further, RAN3 would like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, to help RAN3 decide whether to specify geographical area and/or mapped cell ID over NGAP.

Based on the RAN3 LS, RAN2 has made the agreement in last RAN2#129 meeting as below:

Existing IEs for CHO are used to report the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”.

In this contribution, we continue to discuss the remaining issue of intra-NTN mobility related SON/MDT enhancements.

Discussion

MDT for NTN

For geographical area, RAN3 assumes the geographical area defined for R19 MBS NTN can be reused for Area Scope of logged MDT over NGAP. According to the discussion progress in R19 NR NTN, in RAN2#125bis meeting MBS NTN agreed to consider 2 geographical area formats:

At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered): - - Circles (like for TN coverage) - - Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

In RAN2#127 meeting, it was further agreed the geographical area can be a set of regions:

The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

Therefore, the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, as the intention is the same for area scope of logged MDT and broadcast service area in NTN. And we could wait for the stage-3 detail until the MBS NTN running CR stabilizes, and reuse the RRC signalling IE for logged MDT area scope over Uu for NTN, as shown in [2]:

areaCoordinates-r19             CHOICE {

polygonArea                   OCTET STRING,

circleArea                    SEQUENCE {

referenceLocation-r19       ReferenceLocation-r17,

distanceRadius-r19          INTEGER(0..65535)

}

}

Proposal 1: RAN2 to confirm that the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN.

Since RAN3 asks RAN2 to provide feedback to the question on MDT for NTN, the corresponding reply LS can be sent back to RAN3 as shown in the Annex.

To RAN2 group. ACTION: RAN3 kindly asks RAN2 to take the above into consideration and update their specifications as needed. RAN3 also kindly requests RAN2 to provide feedback to the question on MDT for NTN.

Proposal 2: Send a Reply LS to RAN3 with RAN2 agreements on the MDT area scope of NTN.

For mapped cell ID, it is an ID transmitted in NGAP and it is not used in Uu. Considering after gNB receives the Area Scope for the logged MDT for NTN over NGAP, no matter it is described by the geographical area and/or the mapped cell ID, the gNB could use the single area scope format of logged MDT in Uu.

Proposal 3: The gNB uses the single format in Uu for configuring area scope of logged MDT, no matter it’s configured in the geographical area or mapped cell ID format in NGAP.

Multiple TA for NTN

In R17 NTN, gNB may broadcast a TA list in SIB1 in NTN scenario instead of single TAC: The trackingAreaList is broadcast in the PLMN-IdentityInfo in SIB1. After UE receives the SIB1, the UE AS will forward the trackingAreaList to upper layers (similar as the trackingAreaCode).

PLMN-IdentityInfo ::= SEQUENCE { plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity, trackingAreaCode TrackingAreaCode OPTIONAL, -- Need R ranac RAN-AreaCode OPTIONAL, -- Need R cellIdentity CellIdentity, cellReservedForOperatorUse ENUMERATED {reserved, notReserved}, ..., [[ iab-Support-r16 ENUMERATED {true} OPTIONAL -- Need S ]], [[ trackingAreaList-r17 SEQUENCE (SIZE (1..maxTAC-r17)) OF TrackingAreaCode OPTIONAL, -- Need R gNB-ID-Length-r17 INTEGER (22..32) OPTIONAL -- Cond eventID-TSS ]], [[ mobileIAB-Support-r18 ENUMERATED {true} OPTIONAL -- Need S ]] }

trackingAreaList List of Tracking Areas to which the cell indicated by cellIdentity field belongs. If this field is present, network does not configure trackingAreaCode. Total number of different TACs across different PLMN-IdentityInfos shall not exceed maxTAC. This field is only present in an NTN cell.

In SON or MDT related report, the cell is identified by the CGI information, i.e. CGI-Info-Logging-r16 or CGI-InfoEUTRALogging. There is only one TAC in the CGI information for logging in TN, which is not aligned with the TA list in NTN. Therefore how to set the TA information into the CGI for logging among the TAs in the NTN specific TA list should be considered. One option could be that it’s up to UE to select one TAC indicated in trackingAreaList and add it in CGI information. And it’s also possible to include the whole trackingAreaList in CGI info IE for NTN.

Proposal 4: RAN2 to consider how to set the TA information in the CGI report for NTN:

- Option 1: It’s up to UE to select one TAC indicated in trackingAreaList and add it in CGI info IE.

- Option 2: To include the whole trackingAreaList in CGI info IE for NTN.

Conclusion

According to the analysis in section 2, we have the proposals as below:

Proposal 1: RAN2 to confirm that the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN.

Proposal 2: Send a Reply LS to RAN3 with RAN2 agreements on the MDT area scope of NTN.

Proposal 3: The gNB uses the single format in Uu for configuring area scope of logged MDT, no matter it’s configured in the geographical area or mapped cell ID format in NGAP.

Proposal 4: RAN2 to consider how to set the TA information in the CGI report for NTN:

- Option 1: It’s up to UE to select one TAC indicated in trackingAreaList and add it in CGI info IE.

- Option 2: To include the whole trackingAreaList in CGI info IE for NTN.

Reference

- R3-247908, LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT, RAN3

- R2-2501304, Running RRC CR for NR NTN phase 3, Ericsson

Annex for draft reply LS

1. Overall Description:

RAN2 thanks RAN3 for the LS about RAN3 agreements with impact on UE. Regarding the RAN3 question on MDT for NTN, RAN2 has achieved the following consensus:

Area Scope for the logged MDT for NTN over Uu could reuse the geographical area defined for MBS NTN, and the single format can be used in Uu for configuring area scope of logged MDT, no matter it’s configured in the geographical area or mapped cell ID format in NGAP.

2. Actions:

To RAN3 group.

ACTION: 	RAN2 kindly asks RAN3 to take the above feedback into consideration.