3GPP TSG RAN WG2 #129bis                                                                             R2-2501921

Wuhan, China, Apr. 7th – 11th, 2025

Source:	CATT

Title:	Discussion on applicability reporting for BM use case

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

- Introduction

In RAN2#129 meeting, RAN2 discussed the LCM for UE-sided model for beam management use case and achieved the agreements as follows [1].

Agreements - Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1). - The full inference configuration is sent in CSI-ReportConfig. - Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. - FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.   FFS if we report explicit cause

If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

The provided periodic CSI configuration should be consistent with reported UE capabilities

FFS option B

Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

Semi-persistent reporting, activated by MAC CE/DCI

- Aperiodic CSI reporting, activated by DCI

In this contribution, we will further discuss the leftover issues for the applicability reporting for beam management use case.

- Discussion

In the beginning, RAN2 discussed applicable functionality reporting related issues, some agreements were made and sent to RAN1 in LS [2], some questions also were included in the LS for checking RAN1’s understanding. After RAN2#128 meeting, RAN2 receives the reply LS [3] from RAN1, and in this reply LS, RAN1 provides some replies to these issues of RAN2 in LS [2]. In RAN2#129 meeting, RAN2 discussed and achieved some agreements based on RAN1’s reply LS [3], but there are still some FFS issues which need to be discussed. In this section, we continue to discuss these FFS issues, and for the convenience of discussion, the signalling procedure figure of applicable functionality reporting is also placed here (see figure 1 below).

Figure1: Applicable functionality reporting signalling procedure

For Step 1/Step 2

- Granularity of functionality

In RAN2#127 meeting, RAN2 discussed and agreed the following functionality types which were included in the LS [2] sent to RAN1.

Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) Applicable functionalities refer to functionalities that the UE is ready to apply for inference Activated functionalities refer to functionalities already enabled for performing inference

In LS [2], RAN2 asked the issue of “In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others?”, and in reply LS [3], RAN1 gave the understanding of the three functionality types from RAN1’s perspective, as follow.

In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs - The concept/terminology “functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same.

Observation 1: In RAN1’s understanding, the meaning and the granularity of “functionality” for “supported functionalities”, “applicable functionalities” and “activated functionalities” may or may not be the same.

Furthermore, for the issue, RAN1 replies that “In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-enabled Features/FGs (including components and corresponding value ranges). These UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.”, thus, for the supported functionalities that UE needs to report in step 2, the reporting content and granularity can wait for RAN1’s further discussion.

Proposal 1: For supported functionalities reporting in step2, the reporting content and granularity can wait for RAN1’s further discussion.

For Step 3

- NW-side additional condition

In [2], RAN2 pointed out that these agreements of applicable functionality reporting agreed in RAN2 is based on the assumption “NW-side additional condition is assumed as associated ID.”, and further asked RAN1 whether the assumption is correct or not. Based on the issue, RAN1 gave the reply as follow in [3].

Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID? Answer to Q2: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Observation 2: RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID.

Another issues proposed by RAN2 in [2] e.g., “Is NW-side additional condition functionality specific?”, or “whether NW-side additional condition is mandatory or optional” etc., RAN1 did not give the clear answer and some details about NW-side additional condition are still under discussion in RAN1. RAN2 needs to wait for RAN1’s further discussion.

Proposal 2: RAN2 assumes NW-side additional condition as associated ID in RAN2’s discussion, more details can wait for RAN1.

- Configuration in Step 3

In RAN2#129 meeting, RAN2 agreed that inference configuration/parameters can be signalled in Step 3 and/or inference configuration can be signalled in Step 5 (i.e. option a and option b from RAN1). For option A, RAN2 agreed that the full inference configuration is sent in CSI-ReportConfig, however, for option B, it is still FFS. Considering that it is not the full inference configuration which can used for inference reporting, thus, it is more appropriate to be provided in otherConfig.

Proposal 3: For option B, one set or multiple sets of inference related parameters for applicability report only (not for inference) can be configured in otherConfig.

Another issue is what configuration information should be included in the set of inference related parameters for applicability reporting. According to RAN1’s agreement included in [3], the set of inference related parameters can be selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., the information listed below. The final inference related parameters are discussed and decided by RAN1.

- the associated ID;

- Set A related information;

- Set B related information;

- Report content related information;

- For BM-Case 2,

- Time instances related information for measurements;

- Time instances related information for prediction.

Proposal 4: The set of inference related parameters can be selected from the IEs in/referred by CSI-ReportConfig as a starting point, e.g., the information listed below. The final inference related parameters are discussed and decided by RAN1.

- the associated ID;

- Set A related information;

- Set B related information;

- Report content related information;

- For BM-Case 2,

- Time instances related information for measurements;

- Time instances related information for prediction.

For Step 4

- Applicability reporting in Step 4

In last meeting, RAN2 has agreed that for option A, upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete, UAI can be sent to update applicability. For option B, similar mechanism can be considered, i.e., UE reports the initial applicability via RRCReconfigurationComplete message when receive the inference related parameters in otherConfig, and UAI can be sent to update applicability.

Proposal 5: For option B, UE reports the initial applicability via RRCReconfigurationComplete when receive the inference related parameters in otherConfig, and UAI can be sent to update applicability.

Another issue is what contents should be reported in UAI when receives the inference related parameters in otherConfig, e.g., whether the UE needs to report all inference related parameters of applicable/non-applicable set of inference related parameters, or only to report the index of the applicable/non-applicable set of inference related parameters, from RAN2’s perspective, reporting index is more simple and less air interface signaling overhead.

Proposal 6: For option B, UE can report the index of the applicable/non-applicable set of inference related parameters.

- Non-applicable functionalities reporting

In last meeting, RAN2 agreed that “support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause”.

Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause

From our perspective, the intention of reporting the explicit cause is not clear and will introduce some extra discussion, e.g., what are the cause values that need to be reported, e.g., is it “NW-side additional condition is not matched”, or “UE-side additional condition is not matched, or “no available model”? And whether some other causes are needed e.g., “the Set A/Set B is not matched” etc. as these can also be considered in UE side for determining the functionality applicability.

In addition, whether it is really useful for network also needed to be discussed, e.g., if the cause value for an inapplicable functionality is “UE-side additional condition not matched”, the network cannot solve the issue, i.e., change the non-applicable functionality to applicable functionality as it is up to UE internal situation, or if the cause value is “no available model”, the network cannot transfer the model to UE based on the current discussion and agreements. Therefore, we prefer not introduce the explicit cause considering the complexity and the few benefits.

Proposal 7: No explicit cause is reported along with the non-applicable functionality reporting.

For Step 5

- Inference configuration contents in Step5

For option B, for performing inference, the network needs to send the full inference configuration (CSI-ReportConfig) to UE after receiving the applicability reporting. For the contents of inference configuration in Step5, RAN1 replied in [3] that RAN1 will discuss and design later. RAN2 can wait for RAN1’s discussion and progress.

Q6: What is the content of inference configuration in Step 5? Answer to Q6: Please refer to the agreements related to the Questions from RAN 2. The content of inference configuration as CSI-ReportConfig is to be designed later in RAN1.

Proposal 8: For option B, NW needs to configure CSI-ReportConfig for inference configuration in Step5. The contents of inference configuration in Step5 are up to RAN1.

- Applicable functionalities activation

In last meeting, for option A, RAN2 agreed the following agreements about the applicable functionalities activation.

Agreements - If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). - Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: Semi-persistent reporting, activated by MAC CE/DCI Aperiodic CSI reporting, activated by DCI

For option B, considering that the network needs to send the full inference configuration (CSI-ReportConfig) to UE after receiving the applicability reporting, therefore, for periodic CSI reporting, the UE activates the applicable functionalities when receiving the full inference configuration of the applicable functionality in Step5, for semi-persistent and aperiodic CSI report, legacy CSI framework is used, i.e., semi-persistent reporting is activated by MAC CE/DCI, aperiodic CSI reporting is activated by DCI.

Proposal 9: For option B, the UE activates the applicable functionalities as follow:

- Periodic CSI report is activated when receiving the full inference configuration of the applicable functionality in Step5;

- Semi-persistent and aperiodic CSI report are activated follow legacy CSI framework:

- Semi-persistent reporting, activated by MAC CE/DCI

- Aperiodic CSI reporting, activated by DCI

In [2], RAN2 proposed an issue that if more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated? In [3], RAN1 gave the following agreement, RAN1 agreed that for beam management use case, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

Proposal 10: If more than one functionality is configured in Step 3 or Step 5, multiple/all applicable functionalities can be activated.

- Conclusion

According to the discussion in section 2, we propose:

Observation 1: In RAN1’s understanding, the meaning and the granularity of “functionality” for “supported functionalities”, “applicable functionalities” and “activated functionalities” may or may not be the same.

Proposal 1: For supported functionalities reporting in step2, the reporting content and granularity can wait for RAN1’s further discussion.

Observation 2: RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID.

Proposal 2: RAN2 assumes NW-side additional condition as associated ID in RAN2’s discussion, more details can wait for RAN1.

Proposal 3: For option B, one set or multiple sets of inference related parameters for applicability report only (not for inference) can be configured in otherConfig.

Proposal 4: The set of inference related parameters can be selected from the IEs in/referred by CSI-ReportConfig as a starting point, e.g., the information listed below. The final inference related parameters are discussed and decided by RAN1.

- the associated ID;

- Set A related information;

- Set B related information;

- Report content related information;

- For BM-Case 2,

- Time instances related information for measurements;

- Time instances related information for prediction.

Proposal 5: For option B, UE reports the initial applicability via RRCReconfigurationComplete when receive the inference related parameters in otherConfig, and UAI can be sent to update applicability.

Proposal 6: For option B, UE can report the index of the applicable/non-applicable set of inference related parameters.

Proposal 7: No explicit cause is reported along with the non-applicable functionality reporting.

Proposal 8: For option B, NW needs to configure CSI-ReportConfig for inference configuration in Step5. The contents of inference configuration in Step5 are up to RAN1.

Proposal 9: For option B, the UE activates the applicable functionalities as follow:

- Periodic CSI report is activated when receiving the full inference configuration of the applicable functionality in Step5;

- Semi-persistent and aperiodic CSI report are activated follow legacy CSI framework:

- Semi-persistent reporting, activated by MAC CE/DCI

- Aperiodic CSI reporting, activated by DCI

Proposal 10: If more than one functionality is configured in Step 3 or Step 5, multiple/all applicable functionalities can be activated.

- References

- R2-2501702, RAN2#129 Meeting Report

- R2-2407848 LS on applicable functionality reporting for beam management UE-sided model

- R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model